| UNITED NATIONS |  | E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL TRANS/WP.11/2005/4 7 July 2005 ENGLISH Original: FRENCH |

### ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

Working Party on the Transport of
Perishable Foodstuffs
(Sixty-first session,
Geneva, 31 October-3 November 2005,
item 7 (b) of the provisional agenda)

# AGREEMENT ON THE INTERNATIONAL CARRIAGE OF PERISHABLE FOODSTUFFS AND ON THE SPECIAL EQUIPMENT TO BE USED FOR 
					SUCH CARRIAGE (ATP)

## Annex 1, appendix 2, paragraph 29 (c)

## Transmitted by the expert from Spain

Pursuant to paragraph 43 of the report of the Working Party on its sixtieth session, the representative of Spain proposes the addition of a new (iv) to annex 1, appendix 2, paragraph 29 (c), to read:

“(iii)	Vehicles over 12 years old as a maximum may not continue to carry perishable 	foodstuffs if they have not passed an insulating capacity inspection in accordance 	with annex 1, appendix 2, paragraphs 7 to 28, except in the following cases:

- if the vehicle is classed as heavily insulated (IR), the owner shall apply for a new certificate of compliance as normally insulated (IN) or the vehicle shall pass an insulation capacity inspection in accordance with annex 1, appendix 2, paragraphs 7 to 28, in a testing station approved for the purpose in order to keep its heavily insulated (IR) category;

GE.05-22047  (E)    180705    150805

- if the vehicle is classed as normally insulated (IN), it shall pass an insulation capacity inspection in accordance with annex 1, appendix 2, paragraphs 7 to 28, in a testing station approved for the purpose in order to be able to renew its certificate of compliance as normally insulated (IN);

If, however, the competent authority so requests, the vehicles referred to in the above two cases may, instead of undergoing an insulation capacity inspection in accordance with annex 1, appendix 2, paragraphs 7 to 28, in an approved testing station, undergo a special inspection performed by an expert, who may, with the criteria set out in paragraph 29 (a), accord a period of two years after which the vehicle must in all cases undergo an insulating capacity inspection in accordance with annex 1, appendix 2, paragraphs 7 to 28.”

-----