ECONOMIC COMMISSION FOR EUROPE				INF.24 E

INLAND TRANSPORT COMMITTEE

Working Party on the Transport of Dangerous Goods
(Seventy-ninth session, agenda item 4 (c),

Geneva, 7-11 November 2005)

# CHAPTER 9.2
Compliance of vehicles with regard to braking equipment

## Comments by the Government of Germany on TRANS/WP.15/2005/26

1.	In principle, Germany supports the effort of France to clarify the provisions on braking equipment. However, three important technical developments to improve safety need reflection in the amendments proposed by France:

- endurance braking system for motor vehicles, introduced in ECE-R 13, 06 series;

- automatic anti lock braking system for vehicles, introduced in ECE-R 13, 06 series;

- brake adjuster for vehicles, introduced in ECE-R 13, 09 series.

In addition, new(er) vehicles shall fulfil the ECE-R 13 series of amendments (or the adequate EC directive) applicable at the date of their first registration to avoid downgrading of braking equipment at a later stage, e. g. on repair or replacement of broken parts.

2.	Taking into account the French proposal and the explanations given above Germany proposes the following amendments:

In the table  in 9.2.1, under the heading “Braking equipment”,

- amend comment d to read:

“As from 1 January 2010, all vehicles must meet the requirements of ECE Regulation No. 13 or of  Directive 71/320/EEC as amended applicable at the date of their first registration, but at least ECE Regulation No. 13, 06 series of amendments or Directive 71/320/EEC as amended by Directive 91/422/EC.”

- amend comment g to read:

“As from 1 January 2010, all motor vehicles must meet the requirements of ECE Regulation No. 13 or of  Directive 71/320/EEC as amended applicable at the date of their first registration, but at least the Regulation No. 13, 09 series of amendments or Directive 71/320/EEC as amended by Directive 91/422/EC.”

__________