INF.25

OCTI/RID/CE/42/4e)

31 October 2005

Original: German

RID:		42nd Session of the Committee of Experts on the Transport of Dangerous Goods

(Madrid, 21 - 25 November 2005)

Subject:	UN 1267, 1268 and 3295, Packing Group I

Proposal transmitted by the Secretariat of OTIF

______________________________________________________________________________

SUMMARY

Explanatory summary:		Delete the Packing Group I entries for UN Nos. 1267, 1268 and 3295, for which special provision "640P" is given in column 6

Decision to be taken:		Amend Table A in Chapter 3.2.

Related documents:	 	OCTI/RID/GT-III/2002/39 (TRANS/WP.15/AC.1/2002/39)

OCTI/RID/GT-III/2003/29 (TRANS/WP.15/AC.1/2003/29)

INF.46 and INF.51 from the March 2003 Joint Meeting

OCTI/RID/GT-III/2004/11 (TRANS/WP.15/AC.1/2004/11)

In the 2005 edition of RID, a new line was included for Packing Group I of UN Nos. 1267, 1268 and 3295, which in column 2 contains the explanation "(vapour pressure at 50 °C not more than 110 kPa)" and in column 6 the special provisions 640P and 649 (see documents OCTI/RID/GT-III/2002/39 and …/2003/29 and informal documents INF.46 and INF.51 of the Joint Meeting in March 2003). The aim of this amendment was to make it easier to classify mixtures of flammable liquids containing small quantities of dissolved gases.

As a result of UIC's proposal OCTI/RID/GT-III/2004/11, which was adopted at the Joint Meeting in September 2004, among other things, the different lines for Packing Group I for UN Nos. 1267, 1268 and 3295 were combined, with the highest level tank (L4BN) being required in each case. However, in drafting the proposal, the special situation applicable as from 2005 with regard to these three substances was not taken into account.

After the 2007 amendments have been incorporated, this would lead to the following layout for the substances referred to (only the columns with different information are shown):

| (1) | (2) | (4) | (6) | (12) |
| --- | --- | --- | --- | --- |
| 1267 | PETROLEUM CRUDE OIL | I |  | L4BN |
| 1267 | PETROLEUM CRUDE OIL (vapour pressure at 50 °C not more than 110 kPa) | I | 640P | L1,5BN |
| 1268 | PETROLEUM DISTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. | I |  | L4BN |
| 1268 | PETROLEUM DISTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. (vapour pressure at 50 °C not more than 110 kPa) | I | 640P | L1,5BN |
| 3295 | HYDROCARBONS, LIQUID, N.O.S. | I |  | L4BN |
| 3295 | HYDROCARBONS, LIQUID, N.O.S. (vapour pressure at 50 °C not more than 110 kPa) | I | 640P | L1,5BN |

The explanation in each of the second entries for Packing Group I in column 2 and the indication of special provision "640P" in column 6 only makes sense if an equivalent entry is shown in each of the first entries.

As the aim of document OCTI/RID/GT-III/2004/11 was henceforth to allow only tanks with a test or calculation pressure of 4 bar for substances with a high vapour pressure, the following further 
amendments are proposed in addition to the amendments listed in document OCTI/RID/CE/42/4a).

Chapter 3.2

Table A

| UN Number | Column | Amendment |
| --- | --- | --- |
| 1267, 1268 and 3295 | Delete entries for which special provision "640P" is shown in column 6 |  |
| 1267, 1268 and 3295 | 6 | For those entries where special provision "640A" is currently indicated in column 6, add: "649" |

__________