| UNITED NATIONS |  | E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL TRANS/WP.15/AC.1/2005/23 14 December 2004 Original: ENGLISH |

ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

Working Party on the Transport of Dangerous Goods

Joint Meeting of the RID Safety Committee and the

Working Party on the Transport of Dangerous Goods

(Bern, 7-11 March 2005)

## NEW PROPOSALS OF AMENDMENTS TO RID/ADR/aDn

The 15 year interval between 2 periodic inspections

Transmitted by the European Liquefied Petroleum Gas Association ( (AEGPL)

Proposal

Summary : Proposal to change the wording of the special disposition v of the current P200 concerning the allowance for steel cylinders of an interval of 15 years between 2 periodic inspections.

Justification

1.  	The practice of the interval of 15 years between 2 periodic inspections for LPG cylinders is well established in many countries in Europe with a good safety record (see Annex);

- EN1440 is the European standard already referenced in ADR which lays down the requirements for the periodic inspection of LPG steel cylinders.

To get the benefit of the 15- year intervals, the following criteria should be addressed:

  - the LPG cylinders are designed according to EN1442 (already in ADR) or equivalent;

_________________

* / Circulated by the Central Office for International Carriage by Rail (OCTI) under the symbol OCTI/RID/GT-III/2005/23.

  - there is a system of external protection against corrosion which is being maintained;

  - the cylinders are filled according to EN1439 (already in ADR) or equivalent;

  - the cylinders are filled with LPG with a quality in accordance with a specification/standard such that internal corrosion is not caused;

  - the LPG cylinders are under the control of any LPG marketing organizations responsible for their distribution, filling and maintenance.

When all the above criteria are fulfilled, a 15-year interval may be applied.

When at least one of the criteria from c) to e) is not fulfilled, a 10-year interval must be applied.

- Consequently, the current special disposition v of P200 should be amended according to the existing practice.

Proposal

To amend (9) v as follows :

“the interval between inspections for welded steel cylinders may be extended to 15 years for cylinders dedicated to the carriage of substances of UN N°1011, 1965, 1075, 1978, where the requirements of EN1440: 1996 “Transportable refillable welded steel cylinders for Liquefied Petroleum gases (LPG) – Periodic requalification” are completely fulfilled for the 15 year periodic inspection period."

Safety implication : No

Annex  (ENGLISH ONLY)

| # LPG CYLINDERS REQUALIFICATION WITHIN EU/CEN COUNTRIES |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| COUNTRY |  | 15 years acceptance/date | # Comments |  | # Type of tests |  |  |  |  |  |  | # Authority requests |
|  |  |  |  |  | Ext. Insp. |  |  | Int. Insp. | Pres. test |  | Others |  |
| AUSTRIA |  | Yes |  |  | Yes |  |  |  | Yes |  |  | Quality management system + accordance to EN 1439 and 1440 |
| SWITZERLAND |  | Yes/1997 |  |  | Yes |  |  |  | Yes |  |  | Accordance to EN 1440. Yearly audit. |
| GERMANY |  | Yes | Cylinders manufactured from 1969. Under expertise for oldest cylinders. |  | Yes |  |  | Yes | Yes |  |  | Product and operation management system requested |
| BELGIUM |  | Yes |  |  | Yes |  |  | No | Yes |  | Pneumatic pressure test allowed | Accordance to EN 1439 and 1440. ST/SG/AC.10/C.3/2002/82 page 3 Periodical audit by qualified body. Date of the next requalification on the cylinder |
| France | Yes/1984 |  |  | Cylinders manufactured after about 1960-1965. 5 years interval for older cylinders |  | Yes | No |  | Yes |  | Periodical burst tests on samples. Pneumatic test on camping cylinders | Possibility of self assessment Quality management system + periodical audit |
| SPAIN | Waiting for a response of the authority |  |  |  |  |  |  |  |  |  |  |  |
| PORTUGAL | Yes |  |  | From date of manufacture for “e” cylinders. From last requalification date for others. |  | Yes | No |  | Yes |  |  |  |
| UNITED KINGDOM | Yes |  |  |  |  | Yes | Yes |  | Yes/No | Pressure test according to company |  | Accordance to EN 1440 practices |
| IRELAND | Yes |  |  |  |  | Yes | Yes |  | No |  |  | None. Professional code of practice in accordance with EN 1440. New or refurbished valve. |
| ITALY | Possible, but not applied |  |  | 10 years interval applied |  | Yes | No |  | Yes |  |  | ST/SG/AC.10/C.3/2002/82 page 3 Accordance to EN 1440 ? |
| DENMARK | Possible, but not applied |  |  | 10 years interval applied. 5 years interval for cylinders manufactured before 1956 |  | Yes | Yes |  | Yes |  |  | Accordance to EU directive 1999/36: Approved quality management system in accordance to 1999/36/EEC or EN 45004. Accordance to EN 1440. |
| FINLAND | Yes |  |  |  |  | Yes | Yes |  | Yes | Not more than 5% loss in weight |  | Accordance to EN 1440. New valve to be fitted. |
| SWEDEN | Yes |  |  |  |  | Yes | Yes |  | Yes | Not more than 5% loss in weight |  | Accordance to EN 1440. New valve to be fitted. |