| UNITED NATIONS |  | E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL TRANS/WP.15/AC.1/2005/26 17 December 2004 Original: ENGLISH |

ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

Working Party on the Transport of Dangerous Goods

Joint Meeting of the RID Safety Committee and the

Working Party on the Transport of Dangerous Goods

(Bern, 7-11 March 2005)

## NEW PROPOSALS OF AMENDMENTS TO RID/ADR/aDn

Initiative for a Multilateral Special Agreement under Section 1.5.1 ADR/RID

concerning the carriage of certain wastes concerning dangerous goods

Transmitted by the Government of Austria */

1.	During the last session of the Joint Meeting in Geneva from 13 to 17 september 2004, the Government of Austria had presented an informal document (INF.10) drawing the attention to a draft special multilateral agreement concerning the carriage of certain wastes containing dangerous goods it envisaged to send to ADR Contracting Parties/COTIF member States for signature.

2.	The reason for this initiative was that intensive communication with the representatives of enterprises entitled to waste disposal under the applicable law had made it quite clear that in considerable parts of their professional work carriage under full compliance with the provisions of ADR/RID is not possible.

3.	The need for specific provisions for waste management is already recognized in RID and ADR in some cases, e.g. for contaminated solids (special provisions 216, 217, 218), medical waste (special provision 565) batteries (special provisions 598, 636), disposal of IBCs (4.1.2.2 b) paint (special provision 650, ADR/RID 2005), waste of class 6.2 (7.3.2.6 ADR/RID 2005), etc.

_______

* /      Circulated by the Central Office for International Carriage by Rail (OCTI) under the symbol OCTI/RID/GT-III/2005/26.

4.	This is also the case for the application of ADR and RID in the European Union, since Commission Decisions 2003/635/EC and 2003/627/EC of 20 August 2003 authorize Member States, pursuant to Directives 94/55/EC and 96/49/EC respectively, to adopt certain derogations with regard to the transport of certain dangerous goods on their territories, some of which concern the carriage of waste, e.g.: Denmark RO-SQ 2.1, France-SQ 6.2, Belgium RO-LT 1.4, the Netherlands RO-LT 10.1-10.12, Sweden RO-LT 14.1 and RA-LT 14.1.

5.	The aim of informal document INF.10 was to provoke a discussion on this problem of carriage of waste and to get comments for the preparation of a multilateral agreement. Since this was not possible due to time constraints, the Government of Austria proposes to discuss this question on the basis of the draft elements for a multilateral agreement reproduced in the annex hereto.

Annex

Draft Elements for a possible multilateral agreement under section 1.5.1 of ADR / RID

on the carriage of certain wastes containing dangerous goods

# 1.	Introduction

## This Agreement shall apply only in connection with the collection and carriage of  wastes

## in line with the specific waste legislation framework.

  - By derogation from the provisions of ADR/RID, the carriage of wastes which are dangerous goods or which contain dangerous goods is allowed under the conditions of sections 2 to 7 below.

1.3	 This agreement shall not apply to the carriage of wastes of classes 1 and 7.

# 2.	Classification

## 2.1	Simplified assignment

2.1.1	Where

(a)	an authentic and significant documentation of the wastes is not available; and

(b)	the production process which the waste is descended from is unknown or in its results too variable; and

(c)	a testing on site is not possible or only possible with disproportionate large costs and effort; and

- no assignment can be made by experience (e.g. according to standards),

the assignment of wastes shall be made with simple test-methods, e.g. the smell of residues under consideration of the origin plausibility.

    - An assignment which takes the highest probable danger into account (“over-

classification”) is possible.

    - An approach in accordance with section 2.1.4 of ADR / RID is acceptable.

2.2	 Admixture of other material by mistake

### 2.2.1     Where, according to ADR / RID, wastes are assigned to a UN number or are not subject to the provisions of ADR / RID an admixture by mistake of items of a waste with a different classification can be left unconsidered if from the admixtured material no dangerous reaction and no essential  impact on the degree of danger of the total load can be expected.

2.3	Medicines

Special Provision 601 of Chapter 3.3 of ADR/RID shall apply to wastes of medicines (UN 1851, UN 3248, UN 3249) also if they are not packed any more in packagings of a type intended for retail sale or distribution.

# 3.	Packaging

3.1	The packaging, intermediate bulk container (IBC) or large packaging specified in Table   A of Chapter 3.2 for the relevant UN number shall be used.

3.2	For the following wastes, packagings (including IBCs and large packagings) which have expired or have not been     tested may also be used if their condition, contents and manner of carriage do not endanger the compliance with the protection provisions of section 4.1.1 of ADR / RID:

#### (a)	Dangerous wastes of Packing Group III.

#### (b)	Dangerous wastes of Packing Group II, which comply with the waste groups specified in the table in the Annex to this Agreement; and

# 4.	Carriage in bulk

# 4.1	Wastes of UN 1950 AEROSOLS of all classification codes with residue contents and without caps may be carried in bulk with adequate ventilation.

# 4.2 	Wastes of UN 3175 SOLIDS CONTAINING FLAMMABLE LIQUID, N.O.S. may be carried in bulk in closed or sheeted vehicles and closed or sheeted containers with adequate ventilation.

# 5.	Marking of packages

The provisions of Chapter 5.2 of ADR/RID shall apply with the following derogations:

5.1	The labels may be attached to the package in the way provided for in 5.2.2.1.6 of ADR/RID, last sentence, also in such cases where the requirements specified in the mentioned provision are not met.

5.2	The mark indicating the UN number may appear also in the lower half of the label.

# 6.	Information in the dangerous goods transport document

The provisions of section 5.4.1 of ADR/RID shall apply with the following derogations:

6.1	Where the proper shipping name has to be supplemented with the technical name, the supplementation of the technical name shall not be obligatory, if the proper waste name is evident from the consignment note according to waste legislation.

6.2	The total quantity of dangerous goods according to 5.4.1.1.1 (f) may be estimated.

6.3	For empty means of containment according to 5.4.1.1.6, a sufficiently distinctive general description of the dangerous load or of the part of it concerned may be indicated instead of specifications according to 5.4.1.1.1 (e), without indicating the number of items.

# 7.	Other provisions

All other relevant provisions of ADR / RID shall apply.

| EWC-group | Description of waste | UN-No. | Name and description according to ADR / RID |
| --- | --- | --- | --- |
| 060704 | solutions and acids - acids and acid-mixture with custom-designed constituents | 3264 | CORROSIVE, LIQUID, ACIDIC, INORGANIC, N.O.S. |
| 070203 | organic halogenated solvents, washing liquids and mother liquors | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 070204 | petroleum | 1268 | PETROLEUM DESTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070207 | solvent based sludges, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 070208 | solvent based sludges, non halogenated | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070304 | petroleum | 1268 | PETROLEUM DESTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070307 | solvent based sludges, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070310 | solvent based operating facilities, non halogenated | 3175 | SOLIDS or mixtures of solids (such as preparations and wastes) CONTAINING FLAMMABLE LIQUID, N.O.S. having a flash-point up to 61° |
| 070403 | solvent based mixtures, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 070404 | petroleum | 1268 | PETROLEUM DESTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070404 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S.(vapour pressure at 50 °C more than 175 kPa) |
| 070407 | solvent based sludges, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 070410 | solvent based operating facilities without halogenated compounds | 3175 | SOLIDS or mixtures of solids (such as preparations and wastes) CONTAINING FLAMMABLE LIQUID, N.O.S. having a flash-point up to 61° |
| 070503 | solvent based mixtures, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 070504 | petroleum | 1268 | PETROLEUM DESTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070504 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S.(vapour pressure at 50 °C more than 175 kPa) |
| 070507 | solvent based sludges, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 070510 | solvent based operating facilities without halogenated compounds | 3175 | SOLIDS or mixtures of solids (such as preparations and wastes) CONTAINING FLAMMABLE LIQUID, N.O.S. having a flash-point up to 61° |
| 070603 | solvent based mixtures, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 070604 | Petroleum | 1268 | PETROLEUM DESTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070604 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070610 | solvent based operating facilities without halogenated compounds | 3175 | SOLIDS or mixtures of solids (such as preparations and wastes) CONTAINING FLAMMABLE LIQUID, N.O.S. having a flash-point up to 61° |
| 070703 | solvent based mixtures, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 070704 | Petroleum | 1268 | PETROLEUM DESTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070704 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °CC more than 175 kPa) |
| 080114 | solvent based lacquer- and paint sludge | 1263 | PAINT (including paint, lacquer, enamel, stain, shellac, varnish, polish, liquid filler and liquid lacquer bas) or PAINT RELATED MATERIAL (including paint thinning and reducing compound) (vapour pressure at 50°C more than 175 kPa) |
| 080316 | acids and acid-mixture with custom-designed constituents | 3264 | CORROSIVE, LIQUID, ACIDIC, INORGANIC, N.O.S. |
| 090121 | liquors and liquors-mixture with custom-designed constituents | 3266 | CORROSIVE, LIQUID, BASIC, INORGANIC, N.O.S |
| 100913 | resin residues, non cured | 1325 | FLAMMABLE SOLID, ORGANIC, N.O.S. |
| 110105 | acids and acid-mixture with custom-designed constituents | 3264 | CORROSIVE, LIQUID, ACIDIC, INORGANIC, N.O.S. |
| 110107 | liquors and liquors-mixture with custom-designed constituents | 3266 | CORROSIVE, LIQUID, BASIC, INORGANIC, N.O.S |
| 110115 | acids and acid-mixture with custom-designed constituents | 3264 | CORROSIVE, LIQUID, ACIDIC, INORGANIC, N.O.S. |
| 140603 | solvent based mixtures, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 140604 | solvent based sludges, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 140605 | solvent based sludges, non halogenated | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 140605 | solvent based operating facilities without halogenated compounds | 3175 | SOLIDS or mixtures of solids (such as preparations and wastes) CONTAINING FLAMMABLE LIQUID, N.O.S. having a flash-point up to 61° |
| 150202 | solvent based operating facilities without halogenated compounds | 3175 | SOLIDS or mixtures of solids (such as preparations and wastes) CONTAINING FLAMMABLE LIQUID, N.O.S. having a flash-point up to 61° |
| 160114 | solvent based mixtures, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 190209 | solvent based sludges, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 190807 | acids and acid-mixture with custom-designed constituents | 3266 | CORROSIVE, LIQUID, BASIC, INORGANIC, N.O.S |
| 200113 | solvent based mixtures, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 200113 | cleaner solvent | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 200114 | acids and acids-mixture, anorganic, n.o.s. | 3264 | CORROSIVE, LIQUID, ACIDIC, INORGANIC, N.O.S. |
| 200114 | acids and acid-mixture with custom-designed constituents | 3264 | CORROSIVE, LIQUID, ACIDIC, INORGANIC, N.O.S. |
| 200127 | resin residues, non cured | 1325 | FLAMMABLE SOLID, ORGANIC, N.O.S. |
| 040103 | solvent based sludges, non halogenated | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 040103 | solvent based sludges, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 060106 | acids and acids-mixture, anorganic, n.o.s. | 3264 | CORROSIVE, LIQUID, ACIDIC, INORGANIC, N.O.S. |
| 060106 | acids and acid-mixture with custom-designed constituents | 3264 | CORROSIVE, LIQUID, ACIDIC, INORGANIC, N.O.S. |
| 060205 | liquors and liquors-mixture, n.o.s. | 3266 | CORROSIVE, LIQUID, BASIC, INORGANIC, N.O.S |
| 070103 | solvent based mixtures, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 070104 | Petroleum | 1268 | PETROLEUM DESTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070104 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070107 | solvent based sludges, non halogenated | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070107 | solvent based sludges, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| EWC-group | Description of waste | UN-No. | Name and description according to ADR / RID |
| 070110 | solvent based operating facilities without halogenated compounds | 3175 | SOLIDS or mixtures of solids (such as preparations and wastes) CONTAINING FLAMMABLE LIQUID, N.O.S. having a flash-point up to 61° |
| 070204 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070210 | solvent based operating facilities without halogenated compounds | 3175 | SOLIDS or mixtures of solids (such as preparations and wastes) CONTAINING FLAMMABLE LIQUID, N.O.S. having a flash-point up to 61° |
| 070304 | Petroleum | 1268 | PETROLEUM DESTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070304 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °CC more than 175 kPa) |
| 070308 | solvent based sludges, non halogenated | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070508 | solvent based sludges, halogenated | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070710 | solvent based operating facilities without halogenated compounds | 3175 | SOLIDS or mixtures of solids (such as preparations and wastes) CONTAINING FLAMMABLE LIQUID, N.O.S. having a flash-point up to 61° |
| 080111 | waste lacquers and waste paint if solvent based and/or containing heavy-metals and not completely cured residues in packagings | 1325 | FLAMMABLE SOLID, ORGANIC, N.O.S. |
| 080113 | lacquer- and paint sludge, halogenated | 1263 | PAINT (including paint, lacquer, enamel, stain, shellac, varnish, polish, liquid filler and liquid lacquer bas) or PAINT RELATED MATERIAL (including paint thinning and reducing compound) (vapour pressure at 50°C more than 175 kPa) |
| 080118 | lacquer- and paint sludge, halogenated | 1263 | PAINT (including paint, lacquer, enamel, stain, shellac, varnish, polish, liquid filler and liquid lacquer bas) or PAINT RELATED MATERIAL (including paint thinning and reducing compound) (vapour pressure at 50°C more than 175 kPa) |
| 080119 | lacquer- and paint sludge, halogenated | 1263 | PAINT (including paint, lacquer, enamel, stain, shellac, varnish, polish, liquid filler and liquid lacquer bas) or PAINT RELATED MATERIAL (including paint thinning and reducing compound) (vapour pressure at 50°C more than 175 kPa) |
| 080119 | waste of lacquers and paint if solvent based and/or containing heavy-metals and not completely cured residues in packagings | 1325 | FLAMMABLE SOLID, ORGANIC, N.O.S. |
| 080121 | liquors and liquors mixture with custom-designed constituents | 3266 | CORROSIVE, LIQUID, BASIC, INORGANIC, N.O.S |
| 080409 | resin residues, non cured | 1325 | FLAMMABLE SOLID, ORGANIC, N.O.S. |
| 080411 | resin residues, non cured | 1325 | FLAMMABLE SOLID, ORGANIC, N.O.S. |
| 101013 | resin residues, non cured | 1325 | FLAMMABLE SOLID, ORGANIC, N.O.S. |
| 110106 | acids and acids-mixture, anorganic, n.o.s. | 3264 | CORROSIVE, LIQUID, ACIDIC, INORGANIC, N.O.S. |
| 110113 | cleaner solvent | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 110115 | liquors and liquors mixture with custom-designed constituents | 3266 | CORROSIVE, LIQUID, BASIC, INORGANIC, N.O.S |
| 130703 | Petroleum | 1268 | PETROLEUM DESTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 140603 | Petroleum | 1268 | PETROLEUM DESTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 140603 | cleaner solvent | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 140603 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 150110 | packaging materials with hazardous pollution, predominantly organic | 3175 | SOLIDS or mixtures of solids (such as preparations and wastes) CONTAINING FLAMMABLE LIQUID, N.O.S. having a flash-point up to 61° |
| 150110 | packaging material with acidly contents | 3244 | SOLIDS CONTAINING CORROSIVE LIQUID, N.O.S. |
| 150110 | pakaging material with toxic contents | 3243 | SOLIDS CONTAINING TOXIC LIQUID, N.O.S. |
| 150202 | solid operating facilites contaminated with grease and oil | 3175 | SOLIDS or mixtures of solids (such as preparations and wastes) CONTAINING FLAMMABLE LIQUID, N.O.S. having a flash-point up to 61° |
| 150202 | filter materials, filters bags with custum-designed hazardous constituents, predominantly inorganic | 3088 | SELF-HEATING SOLID, ORGANIC, N.O.S. |
| 160114 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 160115 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 160305 | resin residues, non cured | 1325 | FLAMMABLE SOLID, ORGANIC, N.O.S. |
| 190208 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 190208 | solvent based mixtures, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 190209 | solvent based sludges, non halogenated | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 190808 | acids and acid-mixture with custom-designed constituents | 3264 | CORROSIVE, LIQUID, ACIDIC, INORGANIC, N.O.S. |
| 190808 | liquors and liquors mixtures with custom-designed constituents | 3266 | CORROSIVE, LIQUID, BASIC, INORGANIC, N.O.S |
| 200113 | liquors and liquors mixtures with custom-designed constituents | 3266 | CORROSIVE, LIQUID, BASIC, INORGANIC, N.O.S |
| 200113 | Petroleum | 1268 | PETROLEUM DESTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 200113 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 200115 | liquors and liquors mixtures, n.o.s. | 3266 | CORROSIVE, LIQUID, BASIC, INORGANIC, N.O.S |
| 200119 | pesticides and insecticides | 3021 | PESTICIDE, LIQUID, FLAMMABLE, ORGANIC, TOXIC, N.O.S., flash-point less than 23 ° C |
| 200127 | wastes of lacquers and paint if solvent based and/or containing heavy-metals and not completely cured residues in packagings | 1325 | FLAMMABLE SOLID, ORGANIC, N.O.S. |
| 200129 | Household cleaning agent, marked als flammable, acidly and less toxic | 3286 | FLAMMABLE LIQUID, TOXIC, CORROSIVE, N.O.S. |

_________