| NATIONS UNIES | # E |
| --- | --- |

|  | Conseil économique et social | Distr. GÉNÉRALE TRANS/WP.15/AC.1/2005/26 17 décembre 2004 FRANÇAIS Original: ANGLAIS |
| --- | --- | --- |

COMMISSION ÉCONOMIQUE POUR L’EUROPE

COMITÉ DES TRANSPORTS INTÉRIEURS

Groupe de travail des transports de marchandises dangereuses

Réunion commune de la Commission de sécurité du RID et
du Groupe de travail des transports de marchandises dangereuses 
(Berne, 7-11 mars 2005)

NOUVELLES PROPOSITIONS D’AMENDEMENTS
AUX RÈGLEMENTS RID/ADR/ADN

Initiative en faveur d’un accord particulier multilatéral au sens de la sous-section 1.5.1
du RID et de l’ADR concernant le transport de certains déchets apparentés aux marchandises dangereuses

Communication du Gouvernement autrichien*

1.	Lors de la dernière session de la Réunion commune, tenue du 13 au 17 septembre 2004 à Genève, le Gouvernement autrichien a présenté un document informel (INF.10), dans lequel il attirait l’attention sur un projet d’accord particulier multilatéral concernant le transport de certains déchets apparentés aux marchandises dangereuses qu’il comptait envoyer aux Parties contractantes de l’ADR et aux États membres de la COTIF pour signature.

2.	Cette initiative a été motivée par le fait qu’il était apparu, au terme de nombreux échanges avec les représentants des entreprises autorisées par la législation applicable à procéder à l’élimination des déchets, que bon nombre des opérations de transport qu’ils réalisaient dans le cadre de leur profession ne pouvaient pas être effectuées en totale conformité avec les dispositions de l’ADR et du RID.

3.	Le RID et l’ADR ont dans certains cas déjà tenu compte de la nécessité de disposer de dispositions spécifiques pour la gestion des déchets, par exemple pour les matières solides contaminées (dispositions spéciales 216, 217 et 218), les déchets médicaux (disposition spéciale 565), les piles (dispositions spéciales 598 et 636), l’élimination des GRV (4.1.2.2 b)) et des peintures (disposition spéciale 650, ADR/RID 2005), les déchets de la classe 6.2 (7.3.2.6, ADR/RID 2005), etc.

4.	De même, aux fins de l’application de l’ADR et du RID dans l’Union européenne, les décisions de la Commission 2003/635/CE et 2003/627/CE du 20 août 2003 autorisent les États membres en vertu des directives 94/55/CE et 96/49/CE à adopter certaines dérogations concernant le transport de certaines marchandises dangereuses sur leur territoire, dont certaines portent sur le transport des déchets, par exemple, Danemark RO-SQ 2.1, France-SQ 6.2, Belgique RO-LT 1.4, Pays-Bas RO-LT 10.1-10.12, Suède RO-LT 14.1 et RA-LT 14.1.

5.	Le document informel INF.10 visait à susciter le débat sur le problème posé par le transport des déchets et à rassembler des observations en vue de l’élaboration d’un accord multilatéral. Cela n’ayant pas été possible faute de temps, le Gouvernement autrichien propose d’examiner la question sur la base de l’avant-projet d’accord multilatéral figurant en annexe.

Annexe

Premiers éléments en vue d’un éventuel accord multilatéral au sens de la section 1.5.1
de l’ADR et du RID sur le transport de certains déchets contenant
des marchandises dangereuses

1.	Introduction

1.1	Le présent accord ne s’applique qu’à la collecte et au transport des déchets dans le cadre de la législation spécifique sur les déchets.

1.2	Par dérogation aux dispositions de l’ADR et du RID, le transport des déchets qui sont ou contiennent des marchandises dangereuses est autorisé dans les conditions visées aux sections 2 à 7 ci-après.

1.3	Le présent accord ne s’applique pas au transport des déchets des classes 1 et 7.

2.	Classement

2.1	Affectation simplifiée

2.1.1	Lorsque

a)	aucune documentation authentique et substantielle sur les déchets n’est disponible;

b)	le processus de production dont est issu le déchet est inconnu ou donne des résultats trop variables;

c)	les essais sur site ne sont pas possibles ou impliquent des coûts et des efforts démesurés;

d)	aucune affectation ne peut être faite sur la base de données d’expérience (par exemple, selon les normes);

on utilise, pour l’affectation des déchets, des méthodes d’essai simples, par exemple l’odeur des résidus peut permettre de déterminer leur origine probable.

2.1.2	Une affectation qui tient compte de la probabilité de risque la plus élevée («surclassement») est possible.

2.1.3	Il est acceptable de procéder conformément à la section 2.1.4 de l’ADR et du RID.

2.2	Mélange involontaire avec une autre matière

2.2.1	Lorsque, en vertu des dispositions de l’ADR ou du RID, des déchets sont affectés d’un numéro ONU ou ne sont pas soumis aux dispositions de l’ADR ou du RID, on peut ne pas tenir compte d’un éventuel mélange involontaire avec des déchets répondant à un classement différent si le mélange obtenu ne donne lieu à aucune réaction dangereuse et n’est pas supposé avoir d’incidence majeure sur le degré de dangerosité de l’ensemble du chargement.

2.3	Médicaments

La disposition spéciale 601 du chapitre 3.3 de l’ADR et du RID s’applique aux déchets de médicaments (UN 1851, UN 3248, UN 3249), même s’ils ne sont plus emballés dans des emballages conçus pour la vente ou la distribution au détail.

3.	Emballage

3.1	Il convient d’utiliser l’emballage, le grand récipient pour vrac (GRV) ou les grands emballages indiqués dans le Tableau A du Chapitre 3.2 pour le numéro ONU concerné.

3.2	Pour les autres déchets, on peut également utiliser des emballages (y compris des GRV et des grands emballages) périmés ou qui n’ont pas subi d’épreuves si leur état, leur contenu ou leur mode de transport ne compromet pas le respect des dispositions de protection de la section 4.1.1 de l’ADR ou du RID:

a)	Déchets dangereux du Groupe d’emballage III;

b)	Déchets dangereux du Groupe d’emballage II, qui satisfont aux critères des groupes de déchets indiqués dans le tableau figurant dans l’annexe au présent accord;

4.	Transport en vrac

4.1	Les déchets des GÉNÉRATEURS D’AÉROSOLS qui portent le numéro ONU 1950 (tous codes de classement confondus), qui contiennent un résidu et qui n’ont pas de couvercle peuvent être transportés en vrac, dans des conditions d’aération adéquates.

4.2 	Les déchets de SOLIDES CONTENANT DU LIQUIDE INFLAMMABLE, N.S.A. portant le numéro ONU 3175 peuvent être transportés en vrac dans des véhicules ou conteneurs fermés ou bâchés pourvus d’une aération adéquate.

5.	Marquage des emballages

Les dispositions du chapitre 5.2 de l’ADR et du RID s’appliquent, moyennant les dérogations suivantes;

5.1	Les étiquettes peuvent être fixées sur le colis de la manière prévue dans la dernière phrase du 5.2.2.1.6 de l’ADR et du RID, également dans les cas où les prescriptions indiquées dans la disposition mentionnée ne sont pas satisfaites.

5.2	La marque indiquant le numéro ONU peut également figurer dans la moitié inférieure de l’étiquette.

6.	Renseignements à faire figurer dans le document de transport pour les marchandises dangereuses

Les dispositions de la section 5.4.1 de l’ADR et du RID s’appliquent, moyennant les dérogations suivantes:

6.1	Lorsque la désignation officielle de transport doit être complétée du nom technique, ce complément d’informations n’est pas obligatoire si le nom exact du déchet est indiqué clairement dans la lettre de voiture conformément à la législation relative aux déchets.

6.2	Il est possible d’estimer la quantité totale de marchandises dangereuses conformément au 5.4.1.1.1 f).

6.3	Pour les moyens de confinement vides mentionnés au 5.4.1.1.6, une description générale suffisamment précise du chargement dangereux ou de la partie concernée suffit (en remplacement des données techniques du 5.4.1.1.1 e)), sans indication du nombre d’objets.

7.	Autres dispositions

Toutes les autres dispositions pertinentes de l’ADR et du RID s’appliquent.

| EWC-group | Description of waste | UN-No. | Name and description according to ADR / RID |
| --- | --- | --- | --- |
| 060704 | solutions and acids - acids and acid-mixture with custom-designed constituents | 3264 | CORROSIVE, LIQUID, ACIDIC, INORGANIC, N.O.S. |
| 070203 | organic halogenated solvents, washing liquids and mother liquors | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 070204 | petroleum | 1268 | PETROLEUM DESTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070207 | solvent based sludges, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 070208 | solvent based sludges, non halogenated | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070304 | petroleum | 1268 | PETROLEUM DESTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070307 | solvent based sludges, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070310 | solvent based operating facilities, non halogenated | 3175 | SOLIDS or mixtures of solids (such as preparations and wastes) CONTAINING FLAMMABLE LIQUID, N.O.S. having a flash-point up to 61° |
| 070403 | solvent based mixtures, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 070404 | petroleum | 1268 | PETROLEUM DESTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070404 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S.(vapour pressure at 50 °C more than 175 kPa) |
| 070407 | solvent based sludges, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 070410 | solvent based operating facilities without halogenated compounds | 3175 | SOLIDS or mixtures of solids (such as preparations and wastes) CONTAINING FLAMMABLE LIQUID, N.O.S. having a flash-point up to 61° |
| 070503 | solvent based mixtures, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 070504 | petroleum | 1268 | PETROLEUM DESTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070504 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S.(vapour pressure at 50 °C more than 175 kPa) |
| 070507 | solvent based sludges, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 070510 | solvent based operating facilities without halogenated compounds | 3175 | SOLIDS or mixtures of solids (such as preparations and wastes) CONTAINING FLAMMABLE LIQUID, N.O.S. having a flash-point up to 61° |
| 070603 | solvent based mixtures, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 070604 | Petroleum | 1268 | PETROLEUM DESTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070604 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070610 | solvent based operating facilities without halogenated compounds | 3175 | SOLIDS or mixtures of solids (such as preparations and wastes) CONTAINING FLAMMABLE LIQUID, N.O.S. having a flash-point up to 61° |
| 070703 | solvent based mixtures, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 070704 | Petroleum | 1268 | PETROLEUM DESTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070704 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °CC more than 175 kPa) |
| 080114 | solvent based lacquer- and paint sludge | 1263 | PAINT (including paint, lacquer, enamel, stain, shellac, varnish, polish, liquid filler and liquid lacquer bas) or PAINT RELATED MATERIAL (including paint thinning and reducing compound) (vapour pressure at 50°C more than 175 kPa) |
| 080316 | acids and acid-mixture with custom-designed constituents | 3264 | CORROSIVE, LIQUID, ACIDIC, INORGANIC, N.O.S. |
| 090121 | liquors and liquors-mixture with custom-designed constituents | 3266 | CORROSIVE, LIQUID, BASIC, INORGANIC, N.O.S |
| 100913 | resin residues, non cured | 1325 | FLAMMABLE SOLID, ORGANIC, N.O.S. |
| 110105 | acids and acid-mixture with custom-designed constituents | 3264 | CORROSIVE, LIQUID, ACIDIC, INORGANIC, N.O.S. |
| 110107 | liquors and liquors-mixture with custom-designed constituents | 3266 | CORROSIVE, LIQUID, BASIC, INORGANIC, N.O.S |
| 110115 | acids and acid-mixture with custom-designed constituents | 3264 | CORROSIVE, LIQUID, ACIDIC, INORGANIC, N.O.S. |
| 140603 | solvent based mixtures, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 140604 | solvent based sludges, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 140605 | solvent based sludges, non halogenated | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 140605 | solvent based operating facilities without halogenated compounds | 3175 | SOLIDS or mixtures of solids (such as preparations and wastes) CONTAINING FLAMMABLE LIQUID, N.O.S. having a flash-point up to 61° |
| 150202 | solvent based operating facilities without halogenated compounds | 3175 | SOLIDS or mixtures of solids (such as preparations and wastes) CONTAINING FLAMMABLE LIQUID, N.O.S. having a flash-point up to 61° |
| 160114 | solvent based mixtures, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 190209 | solvent based sludges, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 190807 | acids and acid-mixture with custom-designed constituents | 3266 | CORROSIVE, LIQUID, BASIC, INORGANIC, N.O.S |
| 200113 | solvent based mixtures, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 200113 | cleaner solvent | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 200114 | acids and acids-mixture, anorganic, n.o.s. | 3264 | CORROSIVE, LIQUID, ACIDIC, INORGANIC, N.O.S. |
| 200114 | acids and acid-mixture with custom-designed constituents | 3264 | CORROSIVE, LIQUID, ACIDIC, INORGANIC, N.O.S. |
| 200127 | resin residues, non cured | 1325 | FLAMMABLE SOLID, ORGANIC, N.O.S. |
| 040103 | solvent based sludges, non halogenated | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 040103 | solvent based sludges, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 060106 | acids and acids-mixture, anorganic, n.o.s. | 3264 | CORROSIVE, LIQUID, ACIDIC, INORGANIC, N.O.S. |
| 060106 | acids and acid-mixture with custom-designed constituents | 3264 | CORROSIVE, LIQUID, ACIDIC, INORGANIC, N.O.S. |
| 060205 | liquors and liquors-mixture, n.o.s. | 3266 | CORROSIVE, LIQUID, BASIC, INORGANIC, N.O.S |
| 070103 | solvent based mixtures, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 070104 | Petroleum | 1268 | PETROLEUM DESTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070104 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070107 | solvent based sludges, non halogenated | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070107 | solvent based sludges, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 070110 | solvent based operating facilities without halogenated compounds | 3175 | SOLIDS or mixtures of solids (such as preparations and wastes) CONTAINING FLAMMABLE LIQUID, N.O.S. having a flash-point up to 61° |
| 070204 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070210 | solvent based operating facilities without halogenated compounds | 3175 | SOLIDS or mixtures of solids (such as preparations and wastes) CONTAINING FLAMMABLE LIQUID, N.O.S. having a flash-point up to 61° |
| 070304 | Petroleum | 1268 | PETROLEUM DESTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070304 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °CC more than 175 kPa) |
| 070308 | solvent based sludges, non halogenated | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070508 | solvent based sludges, halogenated | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070710 | solvent based operating facilities without halogenated compounds | 3175 | SOLIDS or mixtures of solids (such as preparations and wastes) CONTAINING FLAMMABLE LIQUID, N.O.S. having a flash-point up to 61° |
| 080111 | waste lacquers and waste paint if solvent based and/or containing heavy-metals and not completely cured residues in packagings | 1325 | FLAMMABLE SOLID, ORGANIC, N.O.S. |
| 080113 | lacquer- and paint sludge, halogenated | 1263 | PAINT (including paint, lacquer, enamel, stain, shellac, varnish, polish, liquid filler and liquid lacquer bas) or PAINT RELATED MATERIAL (including paint thinning and reducing compound) (vapour pressure at 50°C more than 175 kPa) |
| 080118 | lacquer- and paint sludge, halogenated | 1263 | PAINT (including paint, lacquer, enamel, stain, shellac, varnish, polish, liquid filler and liquid lacquer bas) or PAINT RELATED MATERIAL (including paint thinning and reducing compound) (vapour pressure at 50°C more than 175 kPa) |
| 080119 | lacquer- and paint sludge, halogenated | 1263 | PAINT (including paint, lacquer, enamel, stain, shellac, varnish, polish, liquid filler and liquid lacquer bas) or PAINT RELATED MATERIAL (including paint thinning and reducing compound) (vapour pressure at 50°C more than 175 kPa) |
| 080119 | waste of lacquers and paint if solvent based and/or containing heavy-metals and not completely cured residues in packagings | 1325 | FLAMMABLE SOLID, ORGANIC, N.O.S. |
| 080121 | liquors and liquors mixture with custom-designed constituents | 3266 | CORROSIVE, LIQUID, BASIC, INORGANIC, N.O.S |
| 080409 | resin residues, non cured | 1325 | FLAMMABLE SOLID, ORGANIC, N.O.S. |
| 080411 | resin residues, non cured | 1325 | FLAMMABLE SOLID, ORGANIC, N.O.S. |
| 101013 | resin residues, non cured | 1325 | FLAMMABLE SOLID, ORGANIC, N.O.S. |
| 110106 | acids and acids-mixture, anorganic, n.o.s. | 3264 | CORROSIVE, LIQUID, ACIDIC, INORGANIC, N.O.S. |
| 110113 | cleaner solvent | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 110115 | liquors and liquors mixture with custom-designed constituents | 3266 | CORROSIVE, LIQUID, BASIC, INORGANIC, N.O.S |
| 130703 | Petroleum | 1268 | PETROLEUM DESTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 140603 | Petroleum | 1268 | PETROLEUM DESTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 140603 | cleaner solvent | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 140603 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 150110 | packaging materials with hazardous pollution, predominantly organic | 3175 | SOLIDS or mixtures of solids (such as preparations and wastes) CONTAINING FLAMMABLE LIQUID, N.O.S. having a flash-point up to 61° |
| 150110 | packaging material with acidly contents | 3244 | SOLIDS CONTAINING CORROSIVE LIQUID, N.O.S. |
| 150110 | pakaging material with toxic contents | 3243 | SOLIDS CONTAINING TOXIC LIQUID, N.O.S. |
| 150202 | solid operating facilites contaminated with grease and oil | 3175 | SOLIDS or mixtures of solids (such as preparations and wastes) CONTAINING FLAMMABLE LIQUID, N.O.S. having a flash-point up to 61° |
| 150202 | filter materials, filters bags with custum-designed hazardous constituents, predominantly inorganic | 3088 | SELF-HEATING SOLID, ORGANIC, N.O.S. |
| 160114 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 160115 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 160305 | resin residues, non cured | 1325 | FLAMMABLE SOLID, ORGANIC, N.O.S. |
| 190208 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 190208 | solvent based mixtures, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 190209 | solvent based sludges, non halogenated | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 190808 | acids and acid-mixture with custom-designed constituents | 3264 | CORROSIVE, LIQUID, ACIDIC, INORGANIC, N.O.S. |
| 190808 | liquors and liquors mixtures with custom-designed constituents | 3266 | CORROSIVE, LIQUID, BASIC, INORGANIC, N.O.S |
| 200113 | liquors and liquors mixtures with custom-designed constituents | 3266 | CORROSIVE, LIQUID, BASIC, INORGANIC, N.O.S |
| 200113 | Petroleum | 1268 | PETROLEUM DESTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 200113 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 200115 | liquors and liquors mixtures, n.o.s. | 3266 | CORROSIVE, LIQUID, BASIC, INORGANIC, N.O.S |
| 200119 | pesticides and insecticides | 3021 | PESTICIDE, LIQUID, FLAMMABLE, ORGANIC, TOXIC, N.O.S., flash-point less than 23 ° C |
| 200127 | wastes of lacquers and paint if solvent based and/or containing heavy-metals and not completely cured residues in packagings | 1325 | FLAMMABLE SOLID, ORGANIC, N.O.S. |
| 200129 | Household cleaning agent, marked als flammable, acidly and less toxic | 3286 | FLAMMABLE LIQUID, TOXIC, CORROSIVE, N.O.S. |

-----

* Document diffusé par l’Office central des transports internationaux ferroviaires (OCTI) sous la cote OCTI/RID/GT-III/2005/26.