GE.04-24716   (R)   160205   160205

| ОРГАНИЗАЦИЯ ОБЪЕДИНЕННЫХ НАЦИЙ | #### E |
| --- | --- |

| 1 | ЭКОНОМИЧЕСКИЙ И СОЦИАЛЬНЫЙ СОВЕТ | Distr. TRANS/WP.15/AC.1/2005/26 17 December 2004 RUSSIAN Original: |
| --- | --- | --- |

ЕВРОПЕЙСКАЯ ЭКОНОМИЧЕСКАЯ КОМИССИЯ

КОМИТЕТ ПО ВНУТРЕННЕМУ ТРАНСПОРТУ

Рабочая группа по перевозкам опасных грузов

Совместное совещание Комиссии МПОГ по вопросам

безопасности и Рабочей группы по перевозкам опасных грузов

(Берн, 7-11 марта 2005 года)

НОВЫЕ ПРЕДЛОЖЕНИЯ О ВНЕСЕНИИ ПОПРАВОК В МПОГ/ДОПОГ/ВОПОГ

Инициатива в отношении многостороннего специального соглашения, касающегося перевозки некоторых отходов, содержащих опасные грузы, в соответствии с разделом 1.5.1 ДОПОГ/МПОГ

Передано правительством Австрии

1.	На последней сессии Совместного совещания, состоявшейся в Женеве 13-17 сентября 2004 года, правительство Австрии представило неофициальный документ (INF.10), в котором обращается внимание на проект специального многостороннего соглашения, касающегося перевозки некоторых отходов, содержащих опасные грузы, который оно намеревалось направить Договаривающимся сторонам ДОПОГ/государствам - участникам КОТИФ для подписания.

2.	Причина этой инициативы заключалась в том, что интенсивные консультации с представителями предприятий, имеющих право на удаление отходов на основании действующего законодательства, достаточно ясно показали, что во многих областях их профессиональной деятельности невозможно осуществлять перевозки, полностью соблюдая при этом положения ДОПОГ/МПОГ.

3.	Необходимость наличия конкретных положений по обработке и удалению отходов уже признана в МПОГ и ДОПОГ в ряде случаев, например:  загрязненные твердые вещества (специальные положения 216, 217, 218), медицинские отходы (специальное положение 565), батареи (специальные положения 598, 636), удаление КСГМГ (пункт 4.1.2.2 b)), краска (специальное положение 650 ДОПОГ/МПОГ 2005 года), отходы класса 6.2 (подраздел 7.3.2.6 ДОПОГ/МПОГ 2005 года) и т.д.

4.	То же самое касается и применения ДОПОГ и МПОГ в Европейском союзе, поскольку решения 2003/635/ЕС и 2003/627/ЕС Комиссии от 20 августа 2003 года разрешают государствам-членам принять, на основании директив 94/55/ЕС и 96/49/ЕС соответственно, определенные отступления в связи с перевозкой ряда опасных грузов по их территориям, некоторые из которых касаются перевозки отходов, например Дания RO-SQ 2.1, Франция SQ 6.2, Бельгия RO-LT 1.4, Нидерланды RO-LT 10.1-10.12, Швеция RO-LT 14.1 и RA-LT 14.1.

5.	Цель неофициального документа INF.10 состояла в том, чтобы положить начало обсуждению проблемы перевозки отходов и получить замечания по поводу подготовки многостороннего соглашения.  Поскольку из-за недостатка времени сделать это оказалось невозможным, правительство Австрии предлагает обсудить данный вопрос на основе проекта элементов многостороннего соглашения, которые приводятся в приложении к настоящему документу.

Приложение

Проекты элементов возможного многостороннего соглашения о перевозке некоторых отходов, содержащих опасные грузы, в соответствии с разделом 1.5.1 ДОПОГ/МПОГ

1.	Введение

1.1	Настоящее Соглашение применяется только в связи со сбором и перевозкой отходов в соответствии с конкретной системой законодательства по вопросам отходов.

1.2	В отступление от положений ДОПОГ/МПОГ перевозка отходов, которые являются опасными грузами или которые содержат опасные грузы, разрешается при соблюдении условий, установленных в разделах 2-7 ниже.

1.3	Настоящее Соглашение не применяется к перевозке отходов классов 1 и 7.

2.	Классификация

2.1	Упрощенная классификация

2.1.1	В тех случаях, когда

а)	не имеется подлинной и существенной документации на отходы;  и

b)	производственный процесс, в ходе которого образовались отходы, неизвестен, или его результаты слишком изменчивы;  и

с)	проведение испытаний на месте невозможно или возможно лишь при несоразмерно больших издержках и усилиях;  и

d)	классификацию невозможно осуществить на основании опыта (например, в соответствии со стандартами),

классификация отходов осуществляется с использованием простых методов анализа, например путем определения вероятного происхождения отходов по их запаху.

2.1.2	Возможна классификация, при которой учитывается наибольшая вероятная опасность ("завышенная классификация").

2.1.3	Приемлемым является подход на основе положений раздела 2.1.4 ДОПОГ/МПОГ.

2.2	Добавление другого материала по ошибке

2.2.1	В тех случаях, когда в соответствии с ДОПОГ/МПОГ отходы относятся к тому или иному номеру ООН и не подпадают под действие положений ДОПОГ/МПОГ, добавление по ошибке элементов отходов с иной классификацией можно не учитывать, если от добавленного материала нельзя ожидать опасной реакции и существенного влияния на степень опасности всего груза.

2.3	Лекарственные препараты

Специальное положение 601 главы 3.3 ДОПОГ/МПОГ применяется также к отходам медицинских препаратов (№ ООН 1851, 3248, 3249), если они уже не находятся в таре, предназначенной для розничной продажи или распределения.

3.	Тара

3.1	Используется тара, контейнеры средней грузоподъемности для массовых грузов (КСГМГ) или крупногабаритная тара, указанные в таблице А главы 3.2 для соответствующего номера ООН.

3.2	Тара (включая КСГМГ и крупногабаритную тару), срок испытания которой истек или которая не была испытана, может также использоваться для нижеследующих отходов, если ее состояние, содержимое и способ перевозки не ставят под угрозу соблюдение положений раздела 4.1.1 ДОПОГ/МПОГ, касающихся защиты:

а)	опасных отходов, группы упаковки III;

b)	опасных отходов, группы упаковки II, которые соответствуют группам отходов, указанным в таблице в приложении к настоящему Соглашению.

4.	Перевозка навалом/насыпью

4.1	Отходы № ООН 1950 АЭРОЗОЛЕЙ всех классификационных кодов с остатками содержимого и без колпачков могут перевозиться навалом при наличии надлежащей вентиляции.

4.2	Отходы № ООН 3175 ВЕЩЕСТВ ТВЕРДЫХ, СОДЕРЖАЩИХ ЛЕГКОВОСПЛАМЕНЯЮЩУЮСЯ ЖИДКОСТЬ, Н.У.К., могут перевозиться навалом/насыпью в закрытых или крытых брезентом транспортных средствах и в закрытых или крытых брезентом контейнерах при наличии надлежащей вентиляции.

5.	Маркировка упаковок

Применяются положения главы 5.2 ДОПОГ/МПОГ со следующими отступлениями:

5.1	Знаки могут размещаться на упаковке так, как это предусмотрено в последнем предложении пункта 5.2.2.1.6 ДОПОГ/МПОГ, также и в тех случаях, когда не выполняются требования, предусмотренные в вышеуказанном положении.

5.2	Номер ООН может быть указан также в нижней половине знака.

6.	Информация, указываемая в транспортном документе на опасные грузы

Применяются положения раздела 5.4.1 ДОПОГ/МПОГ со следующими отступлениями:

6.1	В тех случаях, когда надлежащее отгрузочное наименование должно дополняться техническим названием, добавление технического названия не является обязательным, если в накладной указано правильное наименование отходов в соответствии с законодательством по вопросам отходов.

6.2	Общее количество опасных грузов в соответствии с пунктом 5.4.1.1.1 f) может быть указано приблизительно.

6.3	В случае порожних средств удержания продукта, предусмотренных в пункте 5.4.1.1.6, вместо данных, требуемых в соответствии с пунктом 5.4.1.1.1 е), может быть приведено достаточно характерное общее описание соответствующего опасного груза или его части без указания количества элементов.

7.	Прочие положения

Применяются все прочие соответствующие положения ДОПОГ/МПОГ.

| EWC-group | Description of waste | UN-No. | Name and description according to ADR / RID |
| --- | --- | --- | --- |
| 060704 | solutions and acids - acids and acid-mixture with custom-designed constituents | 3264 | CORROSIVE, LIQUID, ACIDIC, INORGANIC, N.O.S. |
| 070203 | organic halogenated solvents, washing liquids and mother liquors | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 070204 | petroleum | 1268 | PETROLEUM DESTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070207 | solvent based sludges, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 070208 | solvent based sludges, non halogenated | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070304 | petroleum | 1268 | PETROLEUM DESTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070307 | solvent based sludges, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070310 | solvent based operating facilities, non halogenated | 3175 | SOLIDS or mixtures of solids (such as preparations and wastes) CONTAINING FLAMMABLE LIQUID, N.O.S. having a flash-point up to 61° |
| 070403 | solvent based mixtures, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 070404 | petroleum | 1268 | PETROLEUM DESTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070404 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S.(vapour pressure at 50 °C more than 175 kPa) |
| 070407 | solvent based sludges, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 070410 | solvent based operating facilities without halogenated compounds | 3175 | SOLIDS or mixtures of solids (such as preparations and wastes) CONTAINING FLAMMABLE LIQUID, N.O.S. having a flash-point up to 61° |
| 070503 | solvent based mixtures, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 070504 | petroleum | 1268 | PETROLEUM DESTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070504 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S.(vapour pressure at 50 °C more than 175 kPa) |
| 070507 | solvent based sludges, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 070510 | solvent based operating facilities without halogenated compounds | 3175 | SOLIDS or mixtures of solids (such as preparations and wastes) CONTAINING FLAMMABLE LIQUID, N.O.S. having a flash-point up to 61° |
| 070603 | solvent based mixtures, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 070604 | Petroleum | 1268 | PETROLEUM DESTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070604 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070610 | solvent based operating facilities without halogenated compounds | 3175 | SOLIDS or mixtures of solids (such as preparations and wastes) CONTAINING FLAMMABLE LIQUID, N.O.S. having a flash-point up to 61° |
| 070703 | solvent based mixtures, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 070704 | Petroleum | 1268 | PETROLEUM DESTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070704 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °CC more than 175 kPa) |
| 080114 | solvent based lacquer- and paint sludge | 1263 | PAINT (including paint, lacquer, enamel, stain, shellac, varnish, polish, liquid filler and liquid lacquer bas) or PAINT RELATED MATERIAL (including paint thinning and reducing compound) (vapour pressure at 50°C more than 175 kPa) |
| 080316 | acids and acid-mixture with custom-designed constituents | 3264 | CORROSIVE, LIQUID, ACIDIC, INORGANIC, N.O.S. |
| 090121 | liquors and liquors-mixture with custom-designed constituents | 3266 | CORROSIVE, LIQUID, BASIC, INORGANIC, N.O.S |
| 100913 | resin residues, non cured | 1325 | FLAMMABLE SOLID, ORGANIC, N.O.S. |
| 110105 | acids and acid-mixture with custom-designed constituents | 3264 | CORROSIVE, LIQUID, ACIDIC, INORGANIC, N.O.S. |
| 110107 | liquors and liquors-mixture with custom-designed constituents | 3266 | CORROSIVE, LIQUID, BASIC, INORGANIC, N.O.S |
| 110115 | acids and acid-mixture with custom-designed constituents | 3264 | CORROSIVE, LIQUID, ACIDIC, INORGANIC, N.O.S. |
| 140603 | solvent based mixtures, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 140604 | solvent based sludges, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 140605 | solvent based sludges, non halogenated | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 140605 | solvent based operating facilities without halogenated compounds | 3175 | SOLIDS or mixtures of solids (such as preparations and wastes) CONTAINING FLAMMABLE LIQUID, N.O.S. having a flash-point up to 61° |
| 150202 | solvent based operating facilities without halogenated compounds | 3175 | SOLIDS or mixtures of solids (such as preparations and wastes) CONTAINING FLAMMABLE LIQUID, N.O.S. having a flash-point up to 61° |
| 160114 | solvent based mixtures, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 190209 | solvent based sludges, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 190807 | acids and acid-mixture with custom-designed constituents | 3266 | CORROSIVE, LIQUID, BASIC, INORGANIC, N.O.S |
| 200113 | solvent based mixtures, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 200113 | cleaner solvent | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 200114 | acids and acids-mixture, anorganic, n.o.s. | 3264 | CORROSIVE, LIQUID, ACIDIC, INORGANIC, N.O.S. |
| 200114 | acids and acid-mixture with custom-designed constituents | 3264 | CORROSIVE, LIQUID, ACIDIC, INORGANIC, N.O.S. |
| 200127 | resin residues, non cured | 1325 | FLAMMABLE SOLID, ORGANIC, N.O.S. |
| 040103 | solvent based sludges, non halogenated | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 040103 | solvent based sludges, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 060106 | acids and acids-mixture, anorganic, n.o.s. | 3264 | CORROSIVE, LIQUID, ACIDIC, INORGANIC, N.O.S. |
| 060106 | acids and acid-mixture with custom-designed constituents | 3264 | CORROSIVE, LIQUID, ACIDIC, INORGANIC, N.O.S. |
| 060205 | liquors and liquors-mixture, n.o.s. | 3266 | CORROSIVE, LIQUID, BASIC, INORGANIC, N.O.S |
| 070103 | solvent based mixtures, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 070104 | Petroleum | 1268 | PETROLEUM DESTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070104 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070107 | solvent based sludges, non halogenated | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070107 | solvent based sludges, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| EWC-group | Description of waste | UN-No. | Name and description according to ADR / RID |
| 070110 | solvent based operating facilities without halogenated compounds | 3175 | SOLIDS or mixtures of solids (such as preparations and wastes) CONTAINING FLAMMABLE LIQUID, N.O.S. having a flash-point up to 61° |
| 070204 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070210 | solvent based operating facilities without halogenated compounds | 3175 | SOLIDS or mixtures of solids (such as preparations and wastes) CONTAINING FLAMMABLE LIQUID, N.O.S. having a flash-point up to 61° |
| 070304 | Petroleum | 1268 | PETROLEUM DESTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070304 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °CC more than 175 kPa) |
| 070308 | solvent based sludges, non halogenated | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070508 | solvent based sludges, halogenated | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 070710 | solvent based operating facilities without halogenated compounds | 3175 | SOLIDS or mixtures of solids (such as preparations and wastes) CONTAINING FLAMMABLE LIQUID, N.O.S. having a flash-point up to 61° |
| 080111 | waste lacquers and waste paint if solvent based and/or containing heavy-metals and not completely cured residues in packagings | 1325 | FLAMMABLE SOLID, ORGANIC, N.O.S. |
| 080113 | lacquer- and paint sludge, halogenated | 1263 | PAINT (including paint, lacquer, enamel, stain, shellac, varnish, polish, liquid filler and liquid lacquer bas) or PAINT RELATED MATERIAL (including paint thinning and reducing compound) (vapour pressure at 50°C more than 175 kPa) |
| 080118 | lacquer- and paint sludge, halogenated | 1263 | PAINT (including paint, lacquer, enamel, stain, shellac, varnish, polish, liquid filler and liquid lacquer bas) or PAINT RELATED MATERIAL (including paint thinning and reducing compound) (vapour pressure at 50°C more than 175 kPa) |
| 080119 | lacquer- and paint sludge, halogenated | 1263 | PAINT (including paint, lacquer, enamel, stain, shellac, varnish, polish, liquid filler and liquid lacquer bas) or PAINT RELATED MATERIAL (including paint thinning and reducing compound) (vapour pressure at 50°C more than 175 kPa) |
| 080119 | waste of lacquers and paint if solvent based and/or containing heavy-metals and not completely cured residues in packagings | 1325 | FLAMMABLE SOLID, ORGANIC, N.O.S. |
| 080121 | liquors and liquors mixture with custom-designed constituents | 3266 | CORROSIVE, LIQUID, BASIC, INORGANIC, N.O.S |
| 080409 | resin residues, non cured | 1325 | FLAMMABLE SOLID, ORGANIC, N.O.S. |
| 080411 | resin residues, non cured | 1325 | FLAMMABLE SOLID, ORGANIC, N.O.S. |
| 101013 | resin residues, non cured | 1325 | FLAMMABLE SOLID, ORGANIC, N.O.S. |
| 110106 | acids and acids-mixture, anorganic, n.o.s. | 3264 | CORROSIVE, LIQUID, ACIDIC, INORGANIC, N.O.S. |
| 110113 | cleaner solvent | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 110115 | liquors and liquors mixture with custom-designed constituents | 3266 | CORROSIVE, LIQUID, BASIC, INORGANIC, N.O.S |
| 130703 | Petroleum | 1268 | PETROLEUM DESTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 140603 | Petroleum | 1268 | PETROLEUM DESTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 140603 | cleaner solvent | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 140603 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 150110 | packaging materials with hazardous pollution, predominantly organic | 3175 | SOLIDS or mixtures of solids (such as preparations and wastes) CONTAINING FLAMMABLE LIQUID, N.O.S. having a flash-point up to 61° |
| 150110 | packaging material with acidly contents | 3244 | SOLIDS CONTAINING CORROSIVE LIQUID, N.O.S. |
| 150110 | pakaging material with toxic contents | 3243 | SOLIDS CONTAINING TOXIC LIQUID, N.O.S. |
| 150202 | solid operating facilites contaminated with grease and oil | 3175 | SOLIDS or mixtures of solids (such as preparations and wastes) CONTAINING FLAMMABLE LIQUID, N.O.S. having a flash-point up to 61° |
| 150202 | filter materials, filters bags with custum-designed hazardous constituents, predominantly inorganic | 3088 | SELF-HEATING SOLID, ORGANIC, N.O.S. |
| 160114 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 160115 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 160305 | resin residues, non cured | 1325 | FLAMMABLE SOLID, ORGANIC, N.O.S. |
| 190208 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 190208 | solvent based mixtures, halogenated | 1992 | FLAMMABLE, LIQUID, TOXIC, N.O.S. |
| 190209 | solvent based sludges, non halogenated | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 190808 | acids and acid-mixture with custom-designed constituents | 3264 | CORROSIVE, LIQUID, ACIDIC, INORGANIC, N.O.S. |
| 190808 | liquors and liquors mixtures with custom-designed constituents | 3266 | CORROSIVE, LIQUID, BASIC, INORGANIC, N.O.S |
| 200113 | liquors and liquors mixtures with custom-designed constituents | 3266 | CORROSIVE, LIQUID, BASIC, INORGANIC, N.O.S |
| 200113 | Petroleum | 1268 | PETROLEUM DESTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 200113 | solvent based mixtures without halogenated compounds, paint- and lacquer dilution | 1993 | FLAMMABLE, LIQUID, N.O.S. (vapour pressure at 50 °C more than 175 kPa) |
| 200115 | liquors and liquors mixtures, n.o.s. | 3266 | CORROSIVE, LIQUID, BASIC, INORGANIC, N.O.S |
| 200119 | pesticides and insecticides | 3021 | PESTICIDE, LIQUID, FLAMMABLE, ORGANIC, TOXIC, N.O.S., flash-point less than 23 ° C |
| 200127 | wastes of lacquers and paint if solvent based and/or containing heavy-metals and not completely cured residues in packagings | 1325 | FLAMMABLE SOLID, ORGANIC, N.O.S. |
| 200129 | Household cleaning agent, marked als flammable, acidly and less toxic | 3286 | FLAMMABLE LIQUID, TOXIC, CORROSIVE, N.O.S. |

----------

 	Распространено Центральным бюро международных железнодорожных перевозок (ЦБМЖП) в качестве документа OCTI/RID/GT-III/2005/26.