| ## UNITED NATIONS | # E |
| --- | --- |

|  | Economic and Social Council | Distr. GENERAL TRANS/WP.29/2005/17 22 December 2004 ENGLISH Original: ENGLISH and FRENCH |
| --- | --- | --- |

##### ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations (WP.29)

(One-hundred-and-thirty-fifth session, 8-11 March 2005,

agenda item 4.2.13.)

PROPOSAL FOR SUPPLEMENT 8 TO THE 01 SERIES OF AMENDMENTS

TO REGULATION No. 52

(Small capacity passenger vehicles, M2 and M3 category)

Transmitted by the Working Party on General Safety Provisions (GRSG)

Note :	The text reproduced below was adopted by GRSG at its eighty-seventh session and is transmitted for consideration to WP.29 and AC.1 (TRANS/WP.29/GRSG/66, para. 4).  It is based on the text of document TRANS/WP.29/GRSG/2004/13, not amended.

__________

This document is a working document circulated for discussion and comments. The use of this document for other purposes is the entire responsibility of the user. Documents are also available via the INTERNET: http://www.unece.org/trans/main/welcwp29.htm

Paragraph 5.7.8.1., amend to read:

"5.7.8.1.	Over each seating position and, except in the case of the seat(s) alongside the driver, its associated foot space, there shall be measured a free space with a height of not less than 90 cm, measured from the highest point of the uncompressed seat cushion and at least 135 cm from the mean level of the floor in the foot space.  In the case of vehicles to which paragraph 5.7.1.9. applies, and also for seat(s) alongside the driver, these dimensions may be reduced to 120 cm, measured from the floor and 80 cm measured from the highest point of the uncompressed seat cushion."

______________