| ## UNITED NATIONS | # E |
| --- | --- |

|  | Economic and Social Council | Distr. GENERAL TRANS/WP.29/2005/24 22 December 2004 Original: ENGLISH |
| --- | --- | --- |

##### ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations (WP.29)

(One-hundred-and-thirty-fifth session, 8-11 March 2005,

agenda item 7.1.)

PROPOSAL FOR AMENDMENTS TO ANNEX 7 TO THE CONSOLIDATED RESOLUTION

ON THE CONSTRUCTION OF VEHICLES (R.E.3)

(TRANS/WP.29/78/Rev.1/Amend.2)

Transmitted by the Working Party on General Safety Provisions (GRSG)

Note:	The text reproduced below was adopted by GRSG at its eighty-seventh session and is transmitted for consideration to WP.29 and AC.1 (TRANS/WP.29/GRSG/66, para. 44).  It is based on document TRANS/WP.29/GRSG/2004/17, as amended.

__________

This document is a working document circulated for discussion and comments. The use of this document for other purposes is the entire responsibility of the user. Documents are also available via the INTERNET: http://www.unece.org/trans/main/welcwp29.htm

Annex 7, insert the following new paragraphs 1.6. and 1.7.

"1.6.	Category L6

A vehicle with four wheels whose unladen mass is not more than 350 kg, not including the mass of the batteries in case of electric vehicles, whose maximum design speed is not more than 45 km/h, and whose engine cylinder capacity does not exceed 50 cm3 for spark (positive) ignition engines, or whose maximum net power output does not exceed 4kW in the case of other internal combustion engines, or whose maximum continuous rated power does not exceed 4 kW in the case of electric engines.

  - Category L7

A vehicle with four wheels, other than that classified for the category L6, whose unladen mass is not more than 400 kg (550 kg for vehicles intended for carrying goods), not including the mass of batteries in the case of electric vehicles and whose maximum continuous rated power does not exceed 15 kW."

____________