| ## UNITED NATIONS | # E |
| --- | --- |

|  | Economic and Social Council | Distr. GENERAL TRANS/WP.29/2005/66 21 July 2005 ENGLISH Original: ENGLISH and FRENCH |
| --- | --- | --- |

###### ECONOMIC COMMISSION FOR EUROPE

- INLAND TRANSPORT COMMITTEE

- World Forum for Harmonization of Vehicle Regulations (WP.29)

(One-hundred-and-thirty-seventh session, 15-18 November 2005,

agenda item 5.2.11.)

PROPOSAL FOR SUPPLEMENT 26 TO THE 03 SERIES OF AMENDMENTS

TO REGULATION No. 37

(Filament lamps)

Transmitted by the Working Party on Lighting and Light-Signalling (GRE)

Note:  The text reproduced below was adopted by GRE at its fifty-fourth session, and is transmitted for consideration to WP.29 and to AC.1 (TRANS/WP.29/GRE/54, paras. 51 and 72).  It is based on documents TRANS/WP.29/GRE/2005/4 and TRANS/WP.29/GRE/2005/6, both not amended.

This document is a working document circulated for discussion and comments. The use of this document for other purposes is the entire responsibility of the user. Documents are also available via the INTERNET: http://www.unece.org/trans/main/welcwp29.htm

Paragraph 3.6.2., amend to read:

"3.6.2.	The colorimetric characteristics of .....

…..

finished filament lamps emitting selective-yellow light:

limit towards red		y ≥  0.138 + 0.580 x

limit towards green		y ≤  1.290 x - 0.100

limit towards white:	y ≥  0.940 - x and

y ≥  0.440;

finished filament lamps emitting amber light:

.…… "

Annex 1,

Sheet H1/1, footnote 4/, amend to read:

"4/	The colour of the light emitted shall be white or selective-yellow."

Sheet H3/1, footnote 1/, amend to read:

"1/	The colour of the light emitted shall be white or selective-yellow."

Sheet H4/1, footnote 3/, amend to read:

"3/	The colour of the light emitted shall be white or selective-yellow."

Sheet H7/1, footnote 3/, amend to read:

"3/	The colour of the light emitted shall be white or selective-yellow."

Sheet H8/1, footnote 4/, amend to read:

"4/	The colour of the light emitted shall be white or selective-yellow."

Sheet H8/3, the table, the IEC sheet number, category H8B, amend to read "sheet 7004-146-1".

Sheet H9/3, the table, the IEC sheet number, category H9B, amend to read "sheet 7004-146-1".

Sheet H11/1, footnote 4/, amend to read:

"4/	The colour of the light emitted shall be white or selective-yellow."

Sheet H11/3, the table, the IEC sheet number, category H11B, amend to read "sheet 7004-146-1".

Sheet HB3/2, footnote 6/, amend to read:

"6/	The colour of the light emitted shall be white or selective-yellow."

Sheet HB4/2, footnote 6/, amend to read:

"6/	The colour of the light emitted shall be white or selective-yellow."

Sheet HS1/1, footnote 3/, amend to read:

"3/	The colour of the light emitted shall be white or selective-yellow."

Sheet HS5/3, the table, the IEC sheet number, amend to read "sheet 7004-138-1" (remove the square brackets).

Sheet R2/1, footnote 2/, amend to read:

"2/	The colour of the light emitted shall be white or selective-yellow."

Sheet S1/S2/1, footnote 1/, amend to read:

"1/	The colour of the light emitted shall be white or selective-yellow."

Sheet S3/1, footnote 1/,	 amend to read:

"1/	The colour of the light emitted shall be white or selective-yellow."

Sheet WR21/5W/1, the table, the IEC sheet number, amend to read "sheet 7004-106-3".

- - - - -