10-R41WG-05

05/04/29

Report of 3rd session of Ad-hoc R41 GRB Group, Roma, 05/04/26-27

The list of participants :

Italy		: Messrs Erario, Alburno

Germany	: Messrs Theis, Stevens, Miss Schroeder

USA		: Mr Feith

NL		: Messrs Stoffels, Kortbeek

EU		: Mr Schneider

CH		: Mr Schguanin

JASIC,NTSEL: Messrs Yamamoto, Tanaka, Morita

IMMA		: Messrs Chesnel, Rogers, Tsujimura

ISO		: Mr Segers, Moore

OICA		: Mr Gerhard

ETRTO	: Mr Dimitri

1.	Minutes of 2/R41WG session

Agreed	: The minutes of 2/R41WG session (07-R41WG-05 of 05/03/07)

2.	Developments in ISO

Documents: The presentation from the WG16 Chairman					(Annex1.ppt)

: The IMMA presentation about "Maximum engine speed during acceleration” (Annex2.pdf)

Noted	: The following action items following the meeting held on the 05/04/08 between IMMA, ISO

And RWTUV:

      - Consolidation of the different databases; WMTC data (30'000km),  additional ISO data and German data; by TUV and IMMA

      - Re-analysis of the consolidated database using the ISO method

: The reminder that:

The best possible correlation between noise and acceleration was achieved via

a PMR relation.

Engine speed was an indirect result of the demand for an imposed minimum

acceleration.

: IMMA had asked the people inside JAMA to see the differences (and the significance

of these differences) between the WG16 and the non-consolidated WMTC databases

: The evaluation had confirmed that the new ISO362 method was satisfactory for reproducing

the real urban world conditions: 95% of the driving conditions had been covered.

: The differences in results were due to differences if traffic pattern for each region.

: The confirmation that urban, rural & motorway data were included in the WMTC database.

: The differences between the WMTC analysis and the TUV analysis had been due to the use of

just the German data by TUV

: The slide 2 of Annex 2

Each test result point (X) was a specific machine.

Any abnormal behaviour was due to the specific gear ratios.

All the machines tested had reached the prescribed acceleration rate at different engines

speeds.

A standardized driver weight of 75kg was included.

: The N/S result obtained with WMTC/Part2 was higher than the one obtained with the only

WMTC/Part1 urban module.

: The slide 4 of Annex 2:

The blue curve showed the accelerations in the German data at the transition between urban

& rural areas, as a result of individual driving behaviour

The general acceptance that the noise situation depended on the type of Region

Analysis of the rural part of the WMTC GTR database had covered different regions whereas

the TUV analysis was only based on one single Region (ie. European).

: NL wanted to know if the new method could detect noisy, sporty exhaust systems

: The EU wanted a clear explanation of how the analysis had been carried out, ie how the data

had generated the curves.

Agreed	: The N/S curve would be re-aligned based on the outcome of the re-analysis of the consolidated

database

: The new test protocol was able to cover 95% of the defined conditions from the WMTC

database.

: JASIC supported the new test protocol.

: NL, CH and EU would require non urban considerations to be added.

: USA suggested adjusting the new test protocol to include a large motorcycle sub-category .

: Germany would keep its reserve until the database was reanalysed but was open to accepting

the new test protocol and the ASEP approach.

3.	General philosophy for enforcing noise control

Noted	: Germany stated that new measurements were under way.

: Germany asked for the comparative test results obtained by using the existing and new test

procedures.

: The reminder that on average, a PTW was not noisier than a car but the spread of accelerations

was much higher.

: IMMA suggested that if the nuisance was too high from a particular place, specific measures

such as a lower speed limit at that location would be appropriate.

: A R2 value of 46% showed that PMR explained almost half of the variance in N/S, which was a

high figure. The remaining 50% were caused by other variables such as driving conditions.

: A multiple regression with all variables would explain 99% of the N/S.

: To assess later benefits, the actual situation with regards to noise complaints caused by PTWs

had to be quantified.

: The concept of adding ASEP would address the rural operating conditions.

: The reminder that comparing measurement methods would depend on the context.

: NL was concerned that including all the WMTC data would compromise the analysis of the

European situation.

: The reminder that ECE regulations had been taken up widely around the world.

: In spite of the fact that the USA respected the scientific validity of the new test protocol, they

did not think the test procedure took into account the different real accelerations.

: IMMA reminded the group that nobody had shown that complaints came from legal

motorcycles.

: The sources of noise and complaints were analysed in the “Curious Silence” publication.

: It was a fact that machines generally were used at partial throttle, which is what the test protocol

simulated by combining full acceleration and cruise-by test runs. As such it gave a more

balanced assessment of the different noise sources at partial throttle than the existing test.

The new test protocol could not control the fitment of illegal exhausts

: The benefit to the citizen would come from the limit values which would go with the new test

protocol.

: Mr Stevens stated that:

There was no requirement to accelerate at 50km/h in gear 2 or 3 with the existing test

procedure.

The old method was related to vehicle speed and then by the gearshift behaviour.

It favoured larger motorcycles and penalised smaller vehicles

: The graph on noise nuisance in The Netherlands				(Annex3.ppt)

: The type approval of RESS was covered by ECE R92

: The State of California had made public an extensive noise DB based on their stationary noise

test procedure used for discriminating illegal RESS, and using a limit value.

: The European system was based on a reference value for each vehicle type.

: The table of issues to be addressed by the amendment to R41: 		(Annex4.ppt)

    - Correct simulation of noise source,

    - Coverage of higher engine speeds

    - Supplement for enforcement

: The cruise part of the procedure was necessary because the end purpose of a test protocol was to

define limits for noise sources in all operating conditions.

: The real distribution of noise sources would not be addressed under the acceleration mode.

: Italy restated that urban conditions were the 1st target.

: USA requested that the comparative tests should include the US test procedure.

: Back to back testing with respect to US & ISO362 procedures would help assess the differences

before addressing the limit values discussion.

: USA suggested asking ISO to define a new number instead of keeping the ISO362.

Agreed	: IMMA would provide the group with the data for the individual test points in their presentation.

: Germany was working on an in use control test to provide a tool for comparing with TA

measured values.

: Germany would provide the background information on the "rural" problem

: The need to quantify in terms of end results the new test protocol.

: The table of issues to be addressed by the amendment to R41 would be developed to ensure that

all aspects of noise control were adequately considered

: The need to demonstrate that the new test procedure could distinguish between different  types

of exhaust for the same motorcycle

: Additional requirements for engine speed under urban & rural with different vehicle speeds per

each of the 3 size categories would improve the new test protocol and would be developed by

ISO.

: Germany would provide the group with the list of PTWs exhibiting oscillations at the

measurement point for the stationary test before the next meeting.

: IMAGINE would be presented to the group by the Netherlands, when available.

: The question of the ISO number would be checked by the WG42 chairman

4.	General philosophy for changing the Reg41 amendment project into a GTR project

Noted	      : The USA's suggestion that the project should be converted into one for a GTR, because AC3

had agreed that it was possible to lock a test method into a GTR while the harmonised limit

values were being developed (see the WMTC project).

The original two-step approach of amending R41 and then developing a GTR implied, at least

in theory, that the test procedure was open to further discussion.

: The EU reminder that the Commission would run a cost benefit analysis even for an

amendment of Reg41.

: The Cost benefit analysis for the test procedure alone would not be in terms of $ but in terms of

a general assessment.

: NL stated that manufacturers spent unnecessary money on raising the noise output to just below

the type approval limit value

: IMMA asked NL to table the justification for this statement.

: IMMA would ask its Members to consider the USA suggestion for the GTR.

: The Chairman thought that until the group had agreed a recommendation for GRB to consider,

it should remain with the mandate defined by the Terms of Reference.

: The notes of the meeting held on the 05/04/08 would be attached to the minutes    (Annex5.doc)

Agreed	      : The group agreed to discuss the matter and agree a possible recommendation to GRB and then

to WP29 at the next R41 group at the end of August.

5.	The stationary noise test

Noted	: Germany stated that the need for a stationary test had diminished for the time being because of

the push for a new dynamic enforcement test

: The review of the data on aging in relation to the stationary test from Germany had been

postponed.

: For technical inspections in Germany, the stationary test would be used as a filter.

: The group had discussed the possibility of either improving the stationary test (and leave it for enforcement purpose) and introducing a new simplified drive-by test for those who wanted it

: IMMA thought the group should retain both options in the regulation

: The feasibility for having a data connection point on the motorcycle, so that the stationary test

could be administered easily, was under discussion by IMMA Members.

: IMMA's observation that a "drive by" test for enforcement would require more space and equipment and it would not be easy to have it accepted by the police, who already refused to use the stationary test

: This discussion would be a good introduction to a later discussions of Reg 59 & 92

Agreed	: Each member of the group would send to the Secretariat a statement of known problems with

the stationary test.

: The group would study the "drive by test" proposed by Germany when it had been circulated.

6.	The consolidated draft text

Document	: 02-R41WG-05-ann1,2, the first draft produced by Italy

Agreed	: This item was deferred until the test procedure had been agreed

- 7.	Next steps : process & timetable

Agreed	: The following next steps:

    - By 05/05/31, the hand over of the WG16/IMMA database behind the curves, in an excel format, including current Reg41 test results for those points (or TA data points)

    - By 05/05/31, the upgraded WG16 test protocol would be completed and approved formally by WG16.

    - By 05/06/30, the delivery from Germany of the "drive by" proposal.

    - By 05/07/30, opinions on the proposal to change the project into a GTR.

8.	Future meetings

Agreed	: The next meeting (4/R41WG): 05/08/23-24 in Milwaukee/USA

: Further details would follow when available from the Secretariat.

Philippe C. CHESNEL