DISCUSSION PAPER ON ‘PRESENCE’ AND ‘COLOUR’ IN THE GTR

Transmitted by the Expert from the Netherlands

It is probably well known, by the numerous remarks made by the Netherlands before and during the last meeting, that we do not consider options as the best solution for global harmony. A gtr with options is not a real gtr, according to us. It undermines the philosophy of harmonization, in our view.

However, after the last two meetings in particular, it has become apparent that agreement upon global common windows can not be reached. Or at least can not be reached for the time being. Therefore, one of the possibilities to gain progress could be, unfortunately, by means of introducing "options" in the gtr.

Other solutions, of course, also exist, such as:

- Leave those lamps, on which agreement can not be found, out of the gtr; or

- Stop the discussions on the gtr all together.

Being optimistic, these kinds of solutions would not be our primary choice. So, that leaves us to the solution by means of "options".

Could options be accepted then? The answer is actually “yes” and “no”.

Yes, it is acceptable, but only as “an intermediate station”.

No, it is not acceptable as “a terminal station”.

As “an intermediate station” it is important to distinguish between, let's say, a 'Global option' and an 'Alternative (Regional) option'. The options cannot be referred to as fully equal in value, i.e. simply (a) or (b). There has to be, somehow, a clear distinction between the primary option and secondary option. The end goal will be then, during further development, to eliminate each and every one of the secondary options, hence eradicate all regional derogations.

So, in short:

A modular solution for the time being, but ever striving for that one and only global solution.

An example of how this may be introduced in the text of the draft gtr is shown on the following pages of this discussion paper.

Explanation of text marking used in example

Text  marked green	: Agreement

Text  marked yellow	: Common window

Text  marked red		: Disagreement

4.21.	Based on a determination by each Contracting Party, tThe requirements for presence of the lighting and light signalling devices and the colours of the light emitted or reflected by the lighting and light signalling devices are the following:

DEVICE			PRESENCE							COLOUR

Option a				Option b				Option a				Option b

GLOBAL				ALTERNATIVE (Regional)		GLOBAL				ALTERNATIVE (Regional)

Driving beam headlamp		Mandatory on motor vehicles.					White

Prohibited on trailers.

Passing beam headlamp		Mandatory on motor vehicles.					White

Prohibited on trailers.

Front fog lamp			Optional on motor vehicles.					White				or Selective yellow

Prohibited on trailers.										(in CP’s applying ECE-R.48…)

Reversing lamp			Mandatory on motor vehicles.					White

Optional on trailers.

Direction indicator lamp		front								Amber

and Hazard warning signal	Mandatory on motor vehicles.

Optional on trailers

side								Amber

Mandatory on motor vehicles	Optional on all vehicles

Optional on trailers 		(in US and Canada)

rear								Amber				or Red

Mandatory on all vehicles.									(in US and Canada)

Additional side direction 	Mandatory on category 2		Optional 			Amber

indicator			motor vehicles and trailers	(outside Japan)

exceeding 8,000 kg in gross

vehicle mass, except

truck-tractors.

Stop lamp			Mandatory on all vehicles.					Red

DEVICE			PRESENCE							COLOUR

Option a				Option b				Option a				Option b

GLOBAL				ALTERNATIVE (Regional)		GLOBAL				ALTERNATIVE (Regional)

Center stop lamp		Mandatory on all category 1-1					Red

vehicles and on category 2

vehicles with gross vehicle mass

of less than 4550 kg and

structural width of less than

2030 mm.

Optional on other vehicles.

Rear registration plate		Mandatory on all vehicles.					White

illuminating device

Front position lamp		Mandatory on motor vehicles					White				or Amber

and on trailers over 1,500 mm									(in US and Canada)

in structural width

Optional on other trailers.		Optional on all trailers

(in US and Canada)

Rear position lamp		Mandatory on all vehicles.					Red

Rear fog lamp			Mandatory on all vehicles.	Optional on all vehicles		Red

(in US and Canada)

Parking lamp			Optional on category 1						(towards the) front:

vehicles with structural length					White

less than 6,000 mm and						(towards the) rear:

structural width less than						Red or amber

2030 mm.

Prohibited on all other vehicles.

DEVICE			PRESENCE							COLOUR

Option a				Option b				Option a				Option b

GLOBAL				ALTERNATIVE (Regional)		GLOBAL				ALTERNATIVE (Regional)

End-outline marker lamp	Mandatory on vehicles over							front:

2030 mm in structural width,							White

except in the case of rear								(elsewhere)

end-outline marker lamps								Amber

on incomplete vehicles.								(in US and Canada)

Optional on vehicles between					rear:

1800 mm and 2030 mm in 					Red

structural width and in the

case of rear end–outline marker

lamps on incomplete vehicles

Prohibited on other vehicles.

Rear retro-reflector 		Mandatory on motor vehicles. (1)	Mandatory on all vehicles	Red

non-triangular							(in US and Canada)

# Rear retro-reflector 		Mandatory on trailers; (2)		Optional on trailers		Red

triangular (per Annex 5)		If triangular rear retro-		(in US and Canada)

reflectors are mandatory,

additional non-triangular rear

retro-reflectors are allowed,

provided that they are grouped

together with the other rear

light-signalling devices.

Prohibited on other vehicles.

Front retro-reflector		Mandatory on trailers and	Optional on all vehicles		White

on motor vehicles having all	(in US and Canada)		(or same as incident light)

forward facing lamps with

reflectors concealable.

Optional on other motor vehicles.

(1)	: To be regarderd in conjunction with “Rear retro-reflector triangular”

(2)	: To be regarderd in conjunction with “Rear retro-reflector non-triangular”

DEVICE			PRESENCE							COLOUR

Option a				Option b				Option a				Option b

GLOBAL				ALTERNATIVE (Regional)		GLOBAL				ALTERNATIVE (Regional)

Side retro-reflector		Mandatory on all vehicles	Mandatory on all vehicles	front:

with structural length of more	Amber

than 6000 mm.			intermediate:

Optional on all vehicles with	Amber

structural length of 6000 mm	rearmost:

or less.				Red				or Amber

(outside US and Canada)						(outside US and Canada)

Side-marker lamp		Mandatory on all vehicles	Mandatory on all vehicles	front:

with structural length of more	Amber

than 6000 mm.			intermediate:

Optional on all vehicles with	Amber

structural length of 6000 mm	rearmost:

or less.				Red				or Amber

(outside US and Canada)						(outside US and Canada)

Daytime running lamp				Optional on motor vehicles			White				or Amber to White

(in ……)									(in US and Canada)

Mandatory on motor vehicles

(in Canada)

Prohibited on motor vehicles

(in Japan)

Prohibited on trailers.

Identification lamps				Optional on ……						front:

(in ……)							Amber

Mandatory on vehicles over			rear:

2030 mm in structural width,			Red

except Optional on truck

tractors,

(in US and Canada);

Prohibited

(in ……)

Cornering lamp				Optional on motor vehicles.					White

Prohibited on trailers.

_____________________