|  | EUROPEAN COMMISSION ENTERPRISE AND INDUSTRY DIRECTORATE-GENERAL Consumer goods Automotive industry |
| --- | --- |

Brussels, 12 May 2005

F5 D(2005)

COMMISSION WORKING DOCUMENT TO GRPE

Subject: Scope of ECE Regulations No. 49 and 83.

1.  Background

In the last meeting of GRPE the European Commission presented a document intended to clarify the scope of Regulations under the 1958 Agreement covered by that group. In the meeting, it was stated that Regulations No. 49 and 83 represented a special complexity due to two facts:

- Changes in the scope had been introduced several times in successive amendments.

- Relationship between the two Regulations.

- The chairman of GRPE made a recommendation to set up a small working group, involving interested people from Member States and organisations, to deal with this matter.

- On 14 March 2005 a working group with representatives from the European Commission, U.K., Sweden, Spain, Italy and the Netherlands met in Brussels to discuss the issue. The representative of France sent some comments before the meeting, and the representative of IMMA indicated his agreement with the document.

- The following proposals are now made to GRPE.

- 2.- REGULATION ECE No. 49

Proposal

Paragraph 1., amend to read:

1.     SCOPE

1.1. This regulation applies to vehicles of categories M and N and their engines, as shown by table A, with regard to the tests foreseen for these vehicles in table B.

Table A. APPLICABILITY

|  |  | Positive-Ignition Engines |  |  | Compression-Ignition Engines |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Max. Weight | Petrol | NG (2) | LPG (3) | Diesel | NG (2) | Ethanol |
| M1 | ≤ 3.5 t | - | - | - | - | - | - |
|  | > 3.5 t | - | 49 | 49 | 49 | 49 | 49 |
| M2 | - | - | 49 | 49 | 49//83 (1) | 49 | 49 |
| M3 | - | - | 49 | 49 | 49 | 49 | 49 |
| N1 | - | - | 49//83 | 49//83 | 49//83 | 49//83 | 49 |
| N2 | - | - | 49 | 49 | 49//83 (1) | 49 | 49 |
| N3 | - | - | 49 | 49 | 49 | 49 | 49 |

49//83 = Manufacturers can obtain type approval according to this Regulation or Reg. 83, see under “Equivalent approvals”.

(1) Regulation No. 83 applies only for vehicles with a reference mass ≤ 2840 Kg as an extension of an approval given for an engine used in vehicles of categories M1 or N1.

(2) Natural Gas.

(3) Liquefied Petroleum Gas.

Table B. REQUIREMENTS

|  | Positive-Ignition Engines |  |  | Compression-Ignition Engines |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Petrol | NG (2) | LPG (3) | Diesel | NG (2) | Ethanol |
| Gaseous pollutants | - | Yes | Yes | Yes | Yes | Yes |
| Particulates | - | - | - | Yes | - | - |

1.2.  EQUIVALENT APPROVALS

The following do not need to be approved according to this Regulation if they are part of a vehicle approved according to Regulation No. 83:

-	compression-ignition engines to be mounted in vehicles of sub-categories N1, N2 and M2 fuelled with diesel and 	compression-ignition engines fuelled with Natural Gas to be mounted in vehicles of sub-category N1

-	positive-ignition engines fuelled with Natural Gas or LPG to be mounted in vehicles of sub-category N1

3.- REGULATION ECE No. 83

Proposal

Paragraph 1., amend to read:

1.     SCOPE

This regulation applies to vehicles of categories M and N as shown by table A, with regard to the tests foreseen for these vehicles in table B.

Table A. APPLICABILITY

|  | Max. Weight | Petrol | Diesel | NG (2) | LPG (3) |
| --- | --- | --- | --- | --- | --- |
| M1 | ≤ 3.5 t | 83 | 83 | 83 | 83 |
|  | > 3.5 t | 83 | - | - | - |
| M2 | - | 83 | 49//83 (1) | - | - |
| M3 | - | 83 | - | - | - |
| N1 | - | 83 | 49//83 | 49//83 | 49//83 |
| N2 | - | 83 | 49//83 (1) | - | - |
| N3 | - | 83 | - | - | - |

49//83 = Manufacturers can obtain type approval according to this Regulation or Reg. 49.

(1) Regulation No. 83 applies only for vehicles with a reference mass ≤ 2840 Kg as an extension of an approval given for an engine used in vehicles of categories M1 or N1.

(2) Natural Gas.

(3) Liquefied Petroleum Gas.

Table B. REQUIREMENTS

| Requirements | Positive-ignition engined vehicles of categories M and N including hybrid vehicles |  |  | Compression-ignition engined vehicles of categories M1 and N1 including hybrid vehicles |
| --- | --- | --- | --- | --- |
|  | petrol fuelled vehicle | bi-fuel vehicle | mono-fuel vehicle |  |
| Gaseous pollutants | Yes (max. mass  3.5 t) | Yes (test with both fuel types) (max. mass  3.5 t) | Yes (max. mass  3.5 t) | Yes (max. mass  3.5 t) |
| Particulates | - | - | - | Yes (max. mass  3.5 t) |
| Idle emissions | Yes | Yes (test with both fuel types) | Yes | - |
| Crankcase emissions | Yes | Yes (test only with petrol) | Yes | - |
| Evaporative emissions | Yes (max. mass  3.5 t) | Yes (test only with petrol) (max. mass  3.5 t) | - | - |
| Durability | Yes (max. mass  3.5 t) | Yes (test only with petrol) (max. mass  3.5 t) | Yes (max. mass  3.5 t) | Yes (max. mass  3.5 t) |
| Low temperature emissions | Yes (max. mass  3.5 t) | Yes (max. mass  3.5 t) (test only with petrol) | - | - |
| In-use conformity | Yes (max. mass  3.5 t) | Yes (max. mass  3.5 t) | Yes. (max. mass  3.5 t) | Yes (max. mass  3.5 t) |
| On-board diagnostics | Yes (max. mass  3.5 t) | Yes (max. mass  3.5 t) | Yes (max. mass  3.5 t) | Yes (max. mass  3.5 t) |

4.- REMAINING QUESTIONS

The group considered that the issue of “dual-fuel” (diesel + gas) has to be taken into account in the near future, as engines with this technology are already in the market.

The necessity for the introduction of definitions, e.g. for mono-fuel, LPG, NG, dual-fuel engines, etc was discussed and the Commission will be in contact with car manufacturers and fuel associations in order to get common definitions.

- - - - -