Combination Compatibility with Respect to Stability			GRRF ad-hoc Document No. EVSC05-08

| Towing Vehicle |  | Semi-Trailers |  |  |  | Centre Axle Trailers |  |  |  | Full (Drawbar) Trailers |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Without ISO7638 | With ABS | With RSF | With YC + RSF | Without ISO7638 | With ABS | With RSF | With YC + RSF | Without ISO7638 | With ABS | With RSF | With YC + RSF |
| Tractor | Without ISO7638 | NC | NC | NC 1/ |  |  |  |  |  |  |  |  |  |
|  | With ISO7638 | NC | NC | IMP |  |  |  |  |  |  |  |  |  |
|  | With DC | IMP | IMP | IMP |  |  |  |  |  |  |  |  |  |
|  | With DC + ROC | IMP | IMP | IMP |  |  |  |  |  |  |  |  |  |
| Truck | Without ISO7638 |  |  |  |  | NC | NC | NC 1/ | NC 1/ | NC | NC | NC 1/ | NC 1/ |
|  | With ABS |  |  |  |  | NC | NC | IMP | IMP | NC | NC | IMP | IMP |
|  | With DC |  |  |  |  | IMP 2/ | IMP 2/ | IMP | IMP 3/ | IMP 2/ | IMP 2/ | IMP 3/ | IMP 3/ |
|  | With DC + ROC |  |  |  |  | IMP 2/ | IMP 2/ | IMP | IMP 3/ | IMP 2/ | IMP 2/ | IMP 3/ | IMP 3/ |

YC:	Means a yaw control system installed on the trailer

NC:	Means that the stability of the combination remains unchanged

IMP:	Means that the stability of the combination would or could be improved

1/:	Stability function(s) not functional as the tractor has no ISO7638 power supply is available

2/:	Identification problem between centre axle and full trailers as the trailer has no electric control line to identify the trailer type

3/:	It is not clear how the respective truck and trailer systems will interact and how this will impact on combination stability therefore it may be necessary to define priorities to inhibit either the truck or trailer stability system.