EVSC05-19

Comparison of old and new ECE-Regulation No. 13 - Annex 13

Mandatory minimum requirement for category 1 (motor vehicles) and category B (trailers)

|  | Today - 2005 | 1979 |
| --- | --- | --- |
| Title Annex 13 | Test requirements for vehicles fitted with anti-lock systems | Requirements applicable to tests for braking systems equipped with anti-lock devices (wheel-lock preventers) |
| Cat. 1 | A vehicle equipped with a category 1 anti-lock system shall meet all the relevant requirements of annex 13. ECE-R13/5.2.1.22 [M2, M3, N2. N3] [Mandatory ABS on certain M3 and N3 motor vehicles since 1991 - 88/194/EEC - Annex I/ 2.2.1.22] | no mandatory ABS category defined |
| Split-µ (performance & stability) |  | - |
| Cat. B | A trailer equipped with a category B anti lock system shall meet all the relevant requirements of annex 13, except paragraph 6.3.2. 6.3.2 Split requirements ZRALHεH ≥ 0.5 and ZRALHZRALL ≥ 2 ECE-R13/5.2.213 [O3 an O4] [Mandatory ABS on O4 trailers since 1991 - 88/194/EEC - Annex I/ 2.2.2.13] | no mandatory ABS category defined |
| Split-µ (stability) |  | - |
| Axle sensoring [in the case of trailers] | sensors on all wheels of a controlled axle is required | at an axle only one sensor may be fitted |
| Warning signal sequence and “memory function” requirement |  | - |
| Utilisation of adhesion [] - motor vehicles | on high and low adhesion surfaces unladen & laden | only on high adhesion surfaces only unladen |
| Utilisation of adhesion [] - trailers | only on high adhesion surfaces unladen | dto. |
|  determination | front and rear axles considered App.2: The coefficient of adhesion kM [or kR] shall be determined by weighting with the dynamic axle loads. k = kf.Ffdyn + kr.FrdynP . g ε = zALk Summary: Whole anti-lock system is considered! | only one axle is considered ε = zmaxk motor vehicle Zm  see below Z0 ε = z1z0 trailer Z0 is the maximum braking ratio obtained by braking one axle without locking the wheels, the anti-lock device being disconnected. Z1 is the braking ratio obtained by braking the same axle with the anti-lock device in operation. Summary: Divided anti-lock system is considered! |
| Energy consumption test surface for trailers | still artificial old test conditions *- test track with a surface having a high coefficient of adhesion * Test with unladen vehicle with proportioning /load-sensing valve held in the "laden" position This test condition does reflect the real word conditions! | dto. |
| High to Low transition  Trailer (annexes XIV or 19): |  | - |
| Low to High transition Motor vehicles (annexes X or 13 and trailers (annexes XIV or 19) |  | - |

Personal comment

-	In order to make the introduction of the 1979 ABS regulations for the homologation of trailer anti-lock systems at that time acceptable only a high-µ-test surface was required due to lack of suitable low-µ-surfaces.

-	With respect of introducing new stability systems: It is better to start with requirements which are not 100% perfect than not to start at all!

-	In getting more experience by assessing and testing new stability systems we have a better chance to further develop the initial regulations than never to have the chance to gain this experience at all!

Winfried Gaupp

3rd EVSC meeting Brussels (15th April 2005)