Amended GRRF Ad-hoc Electronic Vehicle Stability Control Working Group Document as a result of 15th April 2005 meeting.

Based upon Document EVSC 04-01 Rev 3 – changes highlighted in yellow (old paragraphs 2.28. and 2.29. combined, old paragraph 5.2.1.31.4. included in paragraph 5.2.1.31.2., annex 2 simplified, annex 19 brought in line).

#### WABCO remarks in blue

Proposed amendments to ECE Reg.13

.

.

.

F Towing vehicle – trailer stability compatibility

| Towing vehicle |  | Semi-trailer |  |  | Centre axle trailer |  |  | Full (drawbar) trailer |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Without ABS | With ABS | With RSF | Without ABS | With ABS | With RSF | Without ABS | With ABS | With RSF |
| Tractor | Without ABS | OK | OK | -(1) |  |  |  |  |  |  |
|  | With ABS | OK | OK | OK |  |  |  |  |  |  |
|  | With VSF | OK | OK | OK |  |  |  |  |  |  |
| Truck | Without ABS |  |  |  | OK | OK | -(1) | OK | OK | -(1) |
|  | With ABS |  |  |  | OK | OK | OK | OK | OK | OK |
|  | With VSF |  |  |  | ?(2) | ?(2) | ?(2) | ?(3) | ?(3) | ?(3) |

OK = no problem regarding the braking interaction of the combination vehicle

Notes:

(1) Roll stability function (RSF) not functional due to no power supply – no ISO 7638 connector on towing vehicle. Stability of combination vehicle not effected – therefore, OK.

(2) Identification problem between centre axle trailer and full trailer. Centre axle trailers behave as semi-trailers – therefore, OK; but full trailer behaviour is a potential problem due to rotational freedom between front axle(s) and rear axle(s) (see note 3).

(3) Due to the rotational freedom between the front axle(s) and the rear axle(s) it may be advisable is permitted to automatically switch-off the truck vehicle stability function (VSF) when a full trailer is coupled so that possibility of trailer swing is minimised when the vehicle stability function comes into operation. Not clear if both directional control and roll-over control would be switched off, or only directional control.

In case of towing vehicles it is permitted to switch-off (not automatically) the truck vehicle stability function (VSF) if a full trailer is coupled so that the possibility of trailer swing is prevented in case of VSF is working/controlling.