EVSC05-34

Remarks EVSC MMI

Remarks concerning the Man Machine Interface for the indication of Intervention and Malfunction of the vehicle stability function.

Current text:

5.2.1.31.4.        Interventions of the vehicle stability function shall be

indicated to the driver by a specific optical warning signal. The

indication  shall  be  present  as  long as the vehicle stability

function  is  active.  The warning signals specified in paragraph

5.2.1.29. of this Regulation shall not be used for this purpose.

Interventions  of  the  vehicle  stability  function  used in any

learning   process   to   determine   the   vehicle   operational

characteristics shall not generate the above signal.

5.2.1.31.5.        A  vehicle stability function failure or defect shall be

detected  and  indicated  to  the  driver by the specific optical

warning signal referred to in paragraph 5.2.1.31.4 5. The warning

signal  shall  be  constant  and  remain displayed as long as the

failure  or defect persists and the ignition (start) switch is in

the "on" (run) position.

Remarks:

How  can  the  driver  differentiate  whether  the  stability  function  is intervening or is defect? Should there be a difference? In  any  case,  there  should  not  be  any design restriction, e.g. matrix displays on the dashboard which may cycle between different warnings should also be allowed.

Dr. Kai Frederik Zastrow