INFORMAL GROUP WORKING RESULT			PVGTR 2005-3d

Ranking of Importance (Imp.):  	1 – 5  (5 = most important)

Ranking of Difficulty (Diff.): 	1 – 5  (5 = most difficult)

Reason for Difficulty (Diff.): 	C   Self-Certification/Type-Approval issue

(C-1: requiring full details of test procedure)

Reason of  Option                            (C-2: Requirements placed in a separate TA Annex)

Co; Cost impact                                      T   Timing issue

Ph; Different Philosophy		       P   Different philosophy

Ce; Different Certification System       Te Technical issue

TBD More discussion necessary

[OPTIONAL PART of the PVGTR means a package of requirements which can be mandated as a whole at the discretion of an Administrative Authority.]

| # ISSUES STILL IN DISCUSSION | # TIMING | # Imp. | # Diff. | # Opt’ (Candidate) |
| --- | --- | --- | --- | --- |
| 1) RBS as part of the Service Braking system. Definition of categories A and B Tests in Neutral - Compensation needed Disconnection on RGB assisted braking Vehicle speed for Heating proc. and hot test | Japan. Proposal was explained in Aug. 04, Should have been discussed in December 2004 but this has really not started yet! Decision by Dec 2005? | 5 | 5 P,Te | TBD |
| 2) Full Power System – Pressure fall. 50% or Number of applications remaining or delete? | discussed in December 2004 & June 2005 No. of applications left preferred June Decision not possible, to be decided Sept. 2005 | 3 | 3 P | TBD |
| # 3) Terminology, Definitions Combined list in PVGTRbraking-wip.e | Discussion to be continued where necessary. | 2 | 2 Te |  |
| # 4) GTR Draft Document These are updated after each meeting to reflect the discussion decisions with issue letter updated | PVGTR2005-1c/2005-2c to be discussed in detail at Sept 2005 meeting, Some items discussed and amendments made. Decision December ? |  |  |  |

| # ISSUES DECIDED by PVGTR informal group |  | # Imp. | # Diff. | # Opt’ (candidate) |
| --- | --- | --- | --- | --- |
| 5) Test Procedures , Integrated or associated? | integrated, see doc. PVGTRAx1-wip | 5 | 3 C-1 |  |
| 6) Parking brake – friction type ? | Not required, see doc. PVGTRswg2-1a | 5 | 2 Te |  |
| 7) Wheel Locking Order | Defined on two different surfaces | 3 | 3 Te |  |
| 8) Parking Brake holding test | Initial brake temperature: 65 – 100oC | 2 | 1 Te |  |
| 9) Definition of vmax | use FMVSS 135 definition | 1 | 1 Te |  |
| 10) Tests – Steering correction | Allowed on each stop as in FMVSS 135 | 1 | 1 Te |  |
| 11) Hot Performance: No of Stops | perform 2 tests (only one may count) | 1 | 2 Te |  |
| 12) EMC requirement, | no, but to be set out in the introduction | 5 | 5 P |  |
| 13) Braking distribution (for non-ABS vehicles) | Calculation where this can be accepted by certification process. Technical Service can therefore omit the wheel locking order test | 5 | 5 C-2 | Ce |
| 14) Annex CEL | In the text as an option. | 5 | 5 | Ce |
| 15) Warning signal function test, switch as alternative to automatic lamp check allowed? | ##### No! | 2 | 2 |  |
| 16) Special requirements for single circuit needed ? | #### No! | 1 | 4 P |  |
| 17) Burnish/Bedding of linings Option is now incorporated in PVGTR 2005-2 Section 5.2.1. | #### Agreed Sept 04 Wheel locking removed from the PVGTR 2005-2b text |  |  | Ce |
| # 18) Parking brake - dynamic requirement? | Proposal by SWG that this is not required. see doc. PVGTRswg2-1a Agreed Jan. 05: delete Annex 3, 2.3.6, but keep 5.2.2.4 | 5 | 2 P |  |
| 19) Foot control only for service brake (or alternatives possible yes with restrictions.) | Agreed Jan. 05: R13-H text used (limited alternatives are possible) |  |  |  |
| 20) Type 0 test practice Pedal force depends on whether ABS is in control. ABS-Max possible. Non ABS-Max level avoiding skidding. ABS vehicles will make the Hot performance test with 500N pedal force even though ABS cycling may be produced. | Agreed Jan. 05 but recinded in Apr. New conditions developed and new cold ref. test added. GTR.text updated and Hot test included Agreed April 2005 Rediscussed June 2005: Cold performance reference test deleted. New proposal for hot performance requirements expected for Sep. 2005 |  |  |  |
| 22) Display of warning signals | Proposal by SWG see “PVGTR2004-17eRev2” PVGTR 2005-1b update GRSG-GTR Cross-referred 2nd warning signal (yellow) accepted by US Decided April 2005 | 2 | P | or Ph (if not agreed in GRSG) |
| 23) Reservoir Fluid – design and volume? | Compromise proposal by EW discussed in April 05 Agreed April 2005 | 2 | 5 Te |  |
| 24) Reservoir Labelling | Compromise proposal by EW discussed in April 05 Decided April 2005 | 2 | 5 P |  |
| 25) Stopping distance / MFDD (both could be used as in PVGTRtest-wipf) | Compliance with both but only if marginal, should both be measured Decided April 05 | 5 | 5 P |  |
| 26) PFC of test surface - requirement 0,9 max? Allowance made for lower levels of adhesion. | Briefly discussed in Dec. 2004 See condition in 5.1.3.4. Agreed at April 2005 | 5 | 4 C-1 |  |
| # 33) EBS requirement for service brake Final acceptance still needed from USA. | Updated proposal of SWG: Keep R13-H text with the following changes: - Delete par. 5.2.8.1 and 5.2.8.2 (compensation) Agreed June 2005 - Change 5.2.20.1 to secondary performance Agreed Jan. 05 , confirmed June 2005 - Delete 5.2.20.2 Agreed Jan. 05 - calculation alternative for 5.2.20.6 now an option in the text 4.3.18.5. Agreed June 2005 - Delete 5.2.20.7 suggested Agreed June 2005 S6.3.12 of FMVSS 135 (Elect. actuated braking) clause added in 5.2.7.3.2. Agreed June 2005. | 5 | 5 T |  |
| # ISSUES STILL TO BE DECIDED / CONFIRMED by PVGTR informal group | # TIMING | # Imp. | # Diff. | # Opt’ # (candidate) |
| 27) Static parking braking performance with trailer yes/no | Majority wants this safety requirement, US position is no. Discussed in Jan/Apr/June 2005, Compromise allows additional use of in-gear parking or automatic transmission set in park. Decision by September 2005 | 3 | 5 P | or Co (if not agreed) |
| 28) EPB requirement for parking brake | Agreement looked possible in Jan. 05 R13-H paragraph 5.2.19.2 not to be deleted Decision by June 2005 not possible, to be decided Sept. 2005 | 5 | 5 T | TBD |
| ##### 29) ABS - Performance definition | “Adhesion utilisation” requirements [unladen] on low µ surface as in R13-H will now be required as an “optional”Appendix Requirements on stability assessment as in PVGTR2005-2b remain for high, low and split µ and transitions. Design requirements (4S/3M) removed. Almost decided June 2005, after OPTION accepted. Only “unladen” in adhesion utilisation test still open, to be decided Sept. 2005 | 5 | 5 Te | Ce adhesion utilisation test ([unladen] on low adhesion only) accepted as an option |
| 30) Unbraked trailer – service braking performance specified: yes/no | Annex 3, par. 2.1.2.calculation will be needed in GTR US to consider further if VM,s calculation will be an acceptable alternative to test. Calculation put into 5.2.7.4.4. of PVGTR 2005-2c. Decision by September 2005 |  |  | or Co (if not agreed) |
| 31) Update of provisions for PTI yes/no? | Possible option depending on position of USA. No decision June 2005, to be decided Sept. 2005 | 5 | 4 P | TBD (Ce ?) |
| # 32) Stop lamp illumination: to be included in GTR? yes/no? | April 2005 decision held up by Selective braking GRRF discussion needed? Discussed June 2005, no decision. | 1 | 4 P | TBD (Ce?) |