INCOHERENCES BETWEEN THE DEFINITIONS IN R.E.3, THE 1968 VIENNA CONVENTION AND UNECE REGULATIONS

## Transmitted by the representative of the European Communities

| Problem | Remark / Example |
| --- | --- |
| Definitions of R.E.3 are not always legally binding | Many Regulations make no reference to R.E.3; e.g. Reg. 48 |
| No definition of scope (maybe not a problem if fully compensated by Regulations on installation – to be checked) | Most lighting components; as a consequence each contracting party can define the scope by foreseeing the use of the component in the respective national law |
| Definition refers to various versions of R.E.3 (1) or Vienna Convention on Road Traffic (2); separate definition diverging from all other definition (3); Reference to two definitions without the same borderline between L1 and L3 (4); it gets even more complicated when amendments to the Regulations are taken into account: It is unclear whether an amendment makes automatically and implicitly reference to a more recent version of R.E.3 or not. | (1) e.g. Regulations Nos.73, 74, 75, 78, 79, … 111. (2) e.g. Regulations Nos.56, 60, 74, 76; imprecise due to definition-options and the frequently used term “power-driven vehicle”; weight limit for L5. (3) e.g. Regulation No.41 (no 50-ccm-criteria); Regulation No.46 and 48 (>25 km/h); (4) e.g. Regulation No.50 para. 2.1. |
| Lack of harmonised use of the Vienna Convention term “motor vehicle” | Most Regulations (e.g. Nos.51, 110) use “Motor vehicles” like in the Vienna Convention, thus excluding Tractors. But Regulation No.4 (p. 14 + Am. 3, 4) and Regulation No.1/Rev.4 (Annex 2 at para. 20) don’t |
| Unclear use of the Vienna Convention term “power driven vehicle” | e.g.: Are agricultural tractors included in Regulation No.24 or not? |
| Unsystematic use of the Vienna Convention terms “power-driven vehicle” and “motor vehicle” | e.g. Regulations Nos.98 and 99; maybe all the Regulations on lighting (except installation Regulation.) |
| Contradictory definitions of scope | See Regulation No.96 (for agricultural tractors) referring to Regulation.No.49 (not mentioning these) as an alternative |
| Ambiguous definitions. Like tyres “designed mainly, but not only for…”; “designed primarily, but not only, for…” | e.g. strict application of Regulations Nos. 30, 54, 75; excludes components “designed only for …”; the relevant criteria must be the use, but not the intention of the manufacturer when designing the product |
| Scope gives options to the contracting parties like “motor cycles and vehicles treated as such” | e.g. Regulations Nos.50, 56, 57, 72, 82 |
| Mixing of Scope definitions and requirements by cross-references, making it unclear as to which Reg. applies | Regulation No.17 making reference to Regulation No.80, the latter making reference to only one para of Regulation No.17; Regulation No.80 making reference to para 7.4. of Regulation No.14 for definition of scope whereas Regulation No.14 makes only reference to Regulation No.80 for an individual requirement |
| Dynamic definition references | RegulationNo.4 to Regulation No.48 |
| Other cases of scope uncertainty | Many Regulations, e.g. with respect to the inclusion of MG and NG vehicles; |
| Useless and misleading specifications of M1 vehicles in 8.1 of R.E.3 | See Multi-purpose vehicles: 1. double negation 2. What about 7 seats? |
| L-vehicle definition excludes 4-wheelers; no L6/7 | See R.E.3 |
| Risk of country code updating in individual legal acts being incomplete |  |
| No clear limit between M and L vehicles; except R.E.5: all 4-wheelers are M | How about L5/6/7 beyond 400/550kg? L6/7 will be covered by UNECE 1958 soon; many other Regulations apply to 4-wheelers. Do they apply to L6/7 at the same time? New light eco-vehicles with bodywork underway? |

___________