# Technical requirements for the design of "sleeping coaches" in the Russian Federation

Technical requirements

Nº 451740.02-2003

- Changes of design

  - Requirements for sleeping places (bunks) in a bus.

    - All bunks must be oriented only laterally.

    - Each bunk must have direct access to a gangway from at least one side. The width of the gangway must match the gangway width of a class III bus according to ECE Regulation No. 36.03.

    - The bunks must be equipped with a safety board not less then 15 cm high and at least 40 cm long, located in the middle third of bunk  length.

    - The safety board must withstand transverse static load of at least 700N.

    - The safety board must not have any dangerous brims or protrusions.

    - It is possible to use two-point safety belts if safety boards are not installed.

    - If a bunk is placed higher than 100 cm above the floor, it must have stairs and handles (hand-rails) to ease access to the bunk.  The stairs must not make the gangway narrower than the minimal required width.

    - The top of a bunk  must be soft.

    - Each bunk must have not less than 80 cm of  free space above without compressing the soft bunk .

    - The upper bunk surface must be located not less then 30 cm above the gangway floor.

    - The width of a bunk must not be less then 42 cm.

    - The length of a bunk must be 170 cm or more.

    - Neighbouring in the lateral direction bunks must be separated by a partition-wall at least 20 cm high above the uncompressed bunk surface.

    - Recommended bunk placement plans are available in appendix A.

- - - - -