Comparison of Head Restraint Regulations FMVSS 202

(Current standard, Final Rule, and ECE 17)

| Head Restraint Component | U.S. – FMVSS 202 (current) | US FMVSS 202 Final Rule | ECE 17 | Comments |
| --- | --- | --- | --- | --- |
| # A. Application |  |  |  |  |
| # 1. Vehicles |  |  |  |  |
|  | Front outboard seating positions in passenger cars, MPVs and trucks with a GVWR  4536 kg | Front outboard and rear outboard (optional) seating positions in passenger cars, MPVs and trucks with a GVWR  4536 kg, with added exclusion for seating position adjacent to aisle on buses (more than 10 seats) | Front outboard and rear (optional) seating positions in vehicles of categories M1 and N, and of vehicles of categories M2 and M3, not covered by Regulation No. 80 | -If HR present in rear seat, ECE 17 and 202 Final Rule regulates. -ECE 17 regulates rear center head restraints if available. |
| 2. Requirements |  |  |  |  |
| a. Height |  |  |  |  |
| 1. Front outboard |  |  |  |  |
| A. Fixed | At least 700 mm above H-point as measured parallel to the torso reference line. | Increased to 800 mm above H-point and measured with a SAE J826 manikin. Seat back angle set at 25 degrees. Seat cushion at highest position. | Same height as FR, but measured from R-point. Seat back angle is 25 degrees or manufacturer specified. Seat cushion at lowest position | Different seat set-up and measuring techniques used. |
| B. Adjustable | Same as 202-fixed | Must achieve a height of 800 mm and cannot be adjusted below 750 mm. Measured with a SAE J826 manikin. Seat back angle set at 25 degrees. Seat cushion in highest position. | Same height as FR, but measured from R-point and at manufacturer’s suggested angle or 25 degrees. Seat cushion in highest position. | Different seat set-up and measuring techniques used. |

| Head Restraint Component | U.S. – FMVSS 202 (current) |  | US FMVSS 202 Final Rule |  | ECE 17 |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| a. Height (cont.) 2. Rear outboard | (202 Final Rule: Rear head restraint means a rear seat back, or any independently adjustable seat component attached to or adjacent to a seat back, that has a height equal or greater than 700 mm, in any position of backset and height adjustment.) |  |  |  |  |  |  |
| A. Fixed | Not specified |  | If provided, minimum height of 750 mm above H-point. Measured with SAE J826 Manikin. |  | If provided, same height as FR, but measured from R-Point |  | Different seat set-up and measuring techniques used. |
| B. Adjustable | Not specified |  | If provided, no adjustment below 750 mm from H-point. Measured with SAE J826 Manikin. |  | If provided, same as FR, but measured from R-Point |  | Different seat set-up and measuring techniques used. |
| 3. Rear Center |  |  |  |  |  |  |  |
|  | Not specified |  | Not specified |  | If provided, minimum height of 700 mm above R-point |  |  |
| b. Backset |  |  |  |  |  |  |  |
| 1. Front outboard positions | Not specified | Backset limited to a maximum 55 mm as measured with HRMD. Head restraint in at any height adjustment between 750 and 800 mm, inclusive. Seat back angle set at 25 degrees. Seat cushion at highest position. |  | No backset specified, but there is a general requirement for the seat back angle to be set at manufacturer’s suggested angle or 25 degrees and the seat cushion to be in the lowest position. |  | Different seat set-up and measuring techniques used. |  |

| Head Restraint Component | U.S. – FMVSS 202 (current) |  | US FMVSS 202 Final Rule | ECE 17 |  | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| c. Width |  |  |  |  |  |  |
| 1. Front outboard | Minimum of 171 mm on single seats and 254 mm on bench seats | Minimum of 170 mm on single seats (outboard seats with no seat in between) and 254 mm on bench seats (outboard seats with seat in between). |  | Minimum of 170 mm for all seat types. | US requires wider HRs on front outboard seats with a center seat between them. |  |
| 2. Rear outboard | Not specified | If provided, minimum of 170 mm for all seat types |  | If provided, minimum of 170 mm. |  |  |
| d. Height of adjustable head restraint front surface |  |  |  |  |  |  |
|  | Not specified |  | Not specified | Minimum height of 100 mm |  |  |
| e. Gaps |  |  |  |  |  |  |
| 1. All outboard positions | Not specified | In all positions, gap between HR and seat back and within the HR is  60 mm. A 165 mm sphere is pressed against the gap with a load no more than 5 N |  | -In lowest position, gap is  25, with no reference to backset adjustment. Measured along straight line between HR and seat back. -In other positions the gap  60 mm as measured with 165 mm dia. sphere. -Gaps larger than 60 mm are allowed if they pass the energy absorption test. |  | -ECE 17/25 does not specify load placed on the sphere to measure gap. -ECE 17/25 measures the gap between the HR in the lowest position and seat back differently from the gaps in the HR. -Larger gaps allowed by ECE, but must be tested. |

| Head Restraint Component | U.S. – FMVSS 202 (current) | US FMVSS 202 Final Rule | ECE 17 | Comments |
| --- | --- | --- | --- | --- |
| f. HR Adjustment Retention Devices (locks) |  |  |  |  |
| 1. Height | Not specified | Must maintain height in highest position and at 800 mm and 750 mm for front and rear seats (if HR provided), respectively, while a downward force is applied. Seat back is rigidly constrained. | If adjustable, requires automatic locking system (ECE 17, 5.1.1). No downward test required. | ECE has no downward testing requirement. |
| 2. Backset | Not specified | Under applied rearward moment, while adjusted to 800mm for front and 750mm for rear (if provided), HR must maintain any position of backset adjustment. Seat back is rigidly constrained. | Not specified. |  |
| g. Removability |  |  |  |  |
| 1. Front | Not specified | Can be removed with deliberate action distinct from any act necessary for adjustment. | Same as 202 FR |  |
| 2. Rear | Not specified | Can be removed with deliberate action distinct from any act necessary for adjustment. | Same as 202 FR |  |

| Head Restraint Component | U.S. – FMVSS 202 (current) | US FMVSS 202 Final Rule | ECE 17 | Comments |
| --- | --- | --- | --- | --- |
| h. Clearance |  |  |  |  |
|  | Not specified | 25 mm clear space allowed where rear HRs, when seat is occupied, interfere with roofline or rear window. | If HR provided, 25 mm clear space allowed where interference with vehicle structure. Seat does not need to be occupied. Minimum height of 700mm must be maintained. | -In ECE the 25 mm gap is measured from any vehicle structure, not just roofline or rear window as in FR. -ECE requires a minimum seat height if HR is present. FR defines a rear HR as having a height greater than 700 mm |
| i. Non-use positions |  |  |  |  |
| 1. Front | Not specified | Not allowed | Allowed, provided HR automatically returns to proper position when seat is occupied. |  |
| 2. Rear | Not specified | Allowed, provided HR automatically returns to proper position when seat is occupied or the HR is rotated a minimum of 60 forward or rearward. | Allowed as long as non-use position is “clearly recognizable to the occupant”. | US rule defines “clearly recognizable” as being rotated forward or rearward 60. |
| j. Radius of Curvature |  |  |  |  |
|  | Not specified | In NPRM, requirement was same as ECE 17. Requirement was deleted in final rule. | Parts of front and rear of HR shall not exhibit a radius of curvature less than 5 mm. | Deleted in FR because enforcement outweighs benefits. No commenter had info to support reg. |

| Head Restraint Component | U.S. – FMVSS 202 (current) | US FMVSS 202 Final Rule | # ECE 17 | Comments |
| --- | --- | --- | --- | --- |
| k. Energy Absorption |  |  |  |  |
|  | Not specified | Front of HR impacted with head form at v=24.1 km/h. 3ms deceleration of head form must not exceed 80gs . Impactor is linear head form with mass of 6.8 kg. | Similar to FR: Uses pendulum impactor with same weight and velocity as linear impactor. Front and rear of HR tested. | Tests in ECE and FR are functionally equivalent. Except FR does not test rear of HR. |
| l. Displacement Test Procedures |  |  |  |  |
|  | Load is applied to back pan of seat, load is applied to head restraint after seat load is removed. 102 mm of displacement allowed with 373 Nm moment. Load is increased until 890N or seat back fails. Use spherical or cylindrical form to apply load. | Test procedure modified from 202. Seat back and HR loaded together. Moments and displacements same. Maximum load the same, seat back cannot fail. Use spherical form to apply load | Same load and displacement requirements as FR. | FR provides a detailed test procedure, including load hold times. |
| m. Dynamic sled test (optional) |  |  |  |  |
|  | Seat accelerated so the pulse falls in a corridor defined by 2-½ sine waves with amplitudes of 78 m/s2 and 86 m/s2. Corridor cannot be met. 95th male dummy used, max rotation 45. | New corridor based on scaled version 208 sled test. Target pulse the same as 202. 50th male dummy used in any seat, HR adjusted midway between lowest and highest position and any backset position. 12 max rotation. | Not specified |  |