| # UNITED # NATIONS |  | ## E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL ECE/TRANS/WP.15/AC.2/21/Add.2 24 March 2006 #### ENGLISH Original: ENGLISH AND FRENCH |

ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

Working Party on the Transport of Dangerous Goods

Joint Meeting of Experts on the Regulations

annexed to the European Agreement concerning

the International Carriage of Dangerous Goods

by Inland Waterways (ADN) */

## REPORT OF THE JOINT MEETING OF EXPERTS ON ITS TENTH SESSION*

# (23-25 January 2006)

Addendum 2

Consolidated version  of Table C of the Regulations

annexed to ADN adopted by the Joint Meeting of Experts

#### Prepared by the secretariat

The secretariat reproduces below the consolidated version of Table C of the Regulations annexed to ADN adopted by the Joint Meeting of Experts at its ninth and tenth sessions. (see documents TRANS/WP.15AC.2/19, TRANS/WP.15AC.2/19/Add.1, ECE/TRANS/WP.15/AC.2/2006/3, ECE/ TRANS /WP.15/ AC.2/2006/4, ECE/TRANS/WP.15/AC.2/21 and ECE/TRANS/WP.15AC.2/ 21/Add.1).

_________________________

*/ 	This meeting is organized jointly by the Economic Commission for Europe and the Central Commission for the Navigation of the Rhine.

| UN No. or substance identification No. | Name and description |  |  | Class | Classification code |  | Packing group |  | Dangers |  | Type of tank vessel |  | Cargo tank design |  | Cargo tank type |  | Cargo tank equipment |  | Opening pressure of the high-velocity vent valve in kPa |  | Maximum degree of filling in % |  | Relative density at 20 °C |  | Type of sampling device |  | Pump room below deck permitted |  | Temperature class |  | Explosion group |  | Anti- explosion protection required |  | Equipment required |  | Number of cones/blue lights |  | Additional requirements/Remarks |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (1) | (2) |  |  | 3 (a) | 3 (b) |  | (4) |  | (5) |  | (6) |  | (7) |  | (8) |  | (9) |  | (10) |  | (11) |  | (12) |  | (13) |  | (14) |  | (15) |  | (16) |  | (17) |  | (18) |  | (19) |  | (20) |  |
| 1005 | AMMONIA, ANHYDROUS |  |  | 2 | 2TC |  |  |  | 2.3+8+2.1 |  | G |  | 1 |  | 1 |  | 3 |  |  |  | 91 |  |  |  | 1 |  |  |  | T1 |  | II A |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 1; 31 |  |
| 1010 | 1,2-BUTADIENE, STABILIZED |  |  | 2 | 2F |  |  |  | 2.1+unst. |  | G |  | 1 |  | 1 |  |  |  |  |  | 91 |  |  |  | 1 |  | yes |  | T2 |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 2; 3; 31 |  |
| 1010 | 1,3-BUTADIENE, STABILIZED |  |  | 2 | 2F |  |  |  | 2.1+unst. |  | G |  | 1 |  | 1 |  |  |  |  |  | 91 |  |  |  | 1 |  | yes |  | T2 |  | II B |  | yes |  | PP, EX, A |  | 1 |  | 2; 3; 31 |  |
| 1010 | BUTADIENE STABILIZED or BUTADIENES AND HYDROCARBON MIXTURE, STABILIZED, having a vapour pressure at 70 °C not exceeding 1.1 MPa (11 bar) and a density at 50 °C not lower than 0.525 kg/l |  |  | 2 | 2F |  |  |  | 2.1+unst. |  | G |  | 1 |  | 1 |  |  |  |  |  | 91 |  |  |  | 1 |  | yes |  | T2 |  | II B |  | yes |  | PP, EX, A |  | 1 |  | 2; 3; 31 |  |
| 1011 | BUTANE |  |  | 2 | 2F |  |  |  | 2.1 |  | G |  | 1 |  | 1 |  |  |  |  |  | 91 |  |  |  | 1 |  | yes |  | T2 |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 31 |  |
| 1012 | 1-BUTYLENE |  |  | 2 | 2F |  |  |  | 2.1 |  | G |  | 1 |  | 1 |  |  |  |  |  | 91 |  |  |  | 1 |  | yes |  | T2 |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 31 |  |
| 1020 | CHLOROPENTAFLUORO-ETHANE (REFRIGERANT GAS R 115) |  |  | 2 | 2A |  |  |  | 2.2 |  | G |  | 1 |  | 1 |  |  |  |  |  | 91 |  |  |  | 1 |  | yes |  |  |  |  |  | no |  | PP |  | 0 |  | 31 |  |
| 1030 | 1,1-DIFLUOROETHANE (REFRIGERANT GAS R 152a) |  |  | 2 | 2F |  |  |  | 2.1 |  | G |  | 1 |  | 1 |  |  |  |  |  | 91 |  |  |  | 1 |  | yes |  | T1 |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 31 |  |
| 1033 | DIMETHYL ETHER |  |  | 2 | 2F |  |  |  | 2.1 |  | G |  | 1 |  | 1 |  |  |  |  |  | 91 |  |  |  | 1 |  | yes |  | T3 |  | II B |  | yes |  | PP, EX, A |  | 1 |  | 31 |  |
| 1038 | ETHYLENE, REFRIGERATED LIQUID |  |  | 2 | 3F |  |  |  | 2.1 |  | G |  | 1 |  | 1 |  | 1 |  |  |  | 95 |  |  |  | 1 |  | no |  | T1 |  | II B |  | yes |  | PP, EX, A |  | 1 |  | 31 |  |
| 1040 | ETHYLENE OXIDE WITH NITROGEN up to a total pressure of 1 MPa (10 bar) at 50 °C |  |  | 2 | 2TF |  |  |  | 2.3+2.1 |  | G |  | 1 |  | 1 |  |  |  |  |  | 91 |  |  |  | 1 |  | no |  | T2 |  | II B |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 2; 3; 11; 31 |  |
| 1055 | ISOBUTYLENE |  |  | 2 | 2F |  |  |  | 2.1 |  | G |  | 1 |  | 1 |  |  |  |  |  | 91 |  |  |  | 1 |  | yes |  | T21) |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 31 |  |
| 1063 | METHYL CHLORIDE (REFRIGERANT GAS R 40) |  |  | 2 | 2F |  |  |  | 2.1 |  | G |  | 1 |  | 1 |  |  |  |  |  | 91 |  |  |  | 1 |  | yes |  | T1 |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 31 |  |
| 1077 | PROPYLENE |  |  | 2 | 2F |  |  |  | 2.1 |  | G |  | 1 |  | 1 |  |  |  |  |  | 91 |  |  |  | 1 |  | yes |  | T1 |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 31 |  |
| 1083 | TRIMETHYLAMINE, ANHYDROUS |  |  | 2 | 2F |  |  |  | 2.1 |  | G |  | 1 |  | 1 |  |  |  |  |  | 91 |  |  |  | 1 |  | yes |  | T4 |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 31 |  |
| 1086 | VINYL CHLORIDE, STABILIZED |  |  | 2 | 2F |  |  |  | 2.1+unst. |  | G |  | 1 |  | 1 |  |  |  |  |  | 91 |  |  |  | 1 |  | yes |  | T2 |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 2; 3; 13; 31 |  |
| 1088 | ACETAL |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.83 |  | 3 |  | yes |  | T3 |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1089 | ACETALDEHYDE (ethanal) |  |  | 3 | F1 |  | I |  | 3 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  | 0.78 |  | 1 |  | yes |  | T4 |  | II A |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1090 | ACETONE |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.79 |  | 3 |  | yes |  | T1 |  | II A |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1092 | ACROLEINE, STABILIZED |  |  | 6.1 | TF1 |  | I |  | 6.1+3+unst. |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 0.84 |  | 1 |  | no |  | T32) |  | II B |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 2; 3; 5; 23 |  |
| 1093 | ACRYLONITRILE, STABILIZED |  |  | 3 | FT1 |  | I |  | 3+6.1+unst. |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 0.8 |  | 1 |  | no |  | T1 |  | II B |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 3; 5; 23 |  |
| 1098 | ALLYL ALCOHOL |  |  | 6.1 | TF1 |  | I |  | 6.1+3 |  | C |  | 2 |  | 2 |  |  |  | 40 |  | 95 |  | 0.85 |  | 1 |  | no |  | T2 |  | II B |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  |  |  |
| 1100 | ALLYL CHLORIDE |  |  | 3 | FT1 |  | I |  | 3+6.1 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 0.94 |  | 1 |  | no |  | T2 |  | II A |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 23 |  |
| 1105 | PENTANOLS (n- pentanol) |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.81 |  | 3 |  | yes |  | T2 |  | II A |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 1106 | AMYLAMINE (n- amylamine) |  |  | 3 | FC |  | II |  | 3+8 |  | C |  | 2 |  | 2 |  |  |  | 40 |  | 95 |  | 0.76 |  | 2 |  | yes |  | T43) |  | II A7) |  | yes |  | PP, EP, EX, A |  | 1 |  |  |  |
| 1107 | AMYL CHLORIDES (l- chloropentane) |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 40 |  | 95 |  | 0.88 |  | 2 |  | yes |  | T3 |  | II A |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1107 | AMYL CHLORIDES (1-chloro-3-methylbutane) |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 45 |  | 95 |  | 0.89 |  | 2 |  | yes |  | T3 |  | II A |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1107 | AMYL CHLORIDES (2-chloro-2-methylbutane) |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  | 0.87 |  | 2 |  | yes |  | T2 |  | II A |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1107 | AMYL CHLORIDES (1-chloro-2,2-dimethylpropane) |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  | 0.87 |  | 2 |  | yes |  | T32) |  | II A |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1107 | AMYL CHLORIDES |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  | 0.9 |  | 1 |  | yes |  | T32) |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 27 |  |
| 1108 | 1-PENTENE (n-AMYLENE) |  |  | 3 | F1 |  | I |  | 3 |  | N |  | 1 |  | 1 |  |  |  |  |  | 97 |  | 0.64 |  | 1 |  | yes |  | T3 |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1114 | BENZENE |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 0.88 |  | 2 |  | yes |  | T1 |  | II A |  | yes |  | PP, EP, EX, TOX, A |  | 1 |  | 6: +10 °C; 17; 23 |  |
| 1120 | BUTANOLS (tert- butyl alcohol) |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  | 2 |  | 10 |  | 97 |  | 0.79 |  | 3 |  | yes |  | T1 |  | II A7) |  | yes |  | PP, EX, A |  | 1 |  | 7; 17 |  |
| 1120 | BUTANOLS (sec- butyl alcohol) |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.81 |  | 3 |  | yes |  | T2 |  | II B7) |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 1120 | BUTANOLS (n- butyl alcohol) |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.81 |  | 3 |  | yes |  | T2 |  | II B |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 1123 | BUTYL ACETATES (sec- butyl acetate) |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.86 |  | 3 |  | yes |  | T2 |  | II A7) |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1123 | BUTYL ACETATES (n-butyl acetate) |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.86 |  | 3 |  | yes |  | T2 |  | II A |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 1125 | n-BUTYLAMINE |  |  | 3 | FC |  | II |  | 3+8 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 0.75 |  | 2 |  | yes |  | T2 |  | II A |  | yes |  | PP, EP, EX, A |  | 1 |  | 23 |  |
| 1127 | CHLOROBUTANES (1- chlorobutane) |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 0.89 |  | 2 |  | yes |  | T3 |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 23 |  |
| 1127 | CHLOROBUTANES (2- chlorobutane) |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 0.87 |  | 2 |  | yes |  | T43) |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 23 |  |
| 1127 | CHLOROBUTANES (1-chloro-2-methylpropane) |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 0.88 |  | 2 |  | yes |  | T43) |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 23 |  |
| 1127 | CHLOROBUTANES (2-chloro-2-methylpropane) |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 0.84 |  | 2 |  | yes |  | T1 |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 23 |  |
| 1127 | CHLOROBUTANES |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  | 0.89 |  | 1 |  | yes |  | T43) |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 27 |  |
| 1129 | BUTYRALDEHYDE (n- butyraldehyde) |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 0.8 |  | 2 |  | yes |  | T4 |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 15; 23 |  |
| 1131 | CARBON DISULPHIDE |  |  | 3 | FT1 |  | I |  | 3+6.1 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 1.26 |  | 1 |  | no |  | T6 |  | II C |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 2; 9; 23 |  |
| 1134 | CHLOROBENZENE (phenyl chloride) |  |  | 3 | F1 |  | III |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 30 |  | 95 |  | 1.11 |  | 2 |  | yes |  | T1 |  | II A8) |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 1135 | ETHYLENE CHLOROHYDRIN (2- chloroethanol) |  |  | 6.1 | TF1 |  | I |  | 6.1+3 |  | C |  | 2 |  | 2 |  |  |  | 30 |  | 95 |  | 1.21 |  | 1 |  | no |  | T2 |  | II A8) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  |  |  |
| 1143 | CROTONALDEHYDE or CROTONALDEHYDE, STABILIZED |  |  | 6.1 | TF1 |  | I |  | 6.1+3+unst. |  | C |  | 2 |  | 2 |  |  |  | 40 |  | 95 |  | 0.85 |  | 1 |  | no |  | T3 |  | II B |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 3; 5; 15 |  |
| 1145 | CYCLOHEXANE |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.78 |  | 3 |  | yes |  | T3 |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 6: +11 °C; 17 |  |
| 1146 | CYCLOPENTANE |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.75 |  | 3 |  | yes |  | T2 |  | II A |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1150 | 1,2-DICHLOROETHYLENE (cis-1,2-DICHLOROETHYLENE) |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 1.28 |  | 2 |  | yes |  | T21) |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 23 |  |
| 1150 | 1,2-DICHLOROETHYLENE (trans-1,2-DICHLOROETHYLENE) |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 1.26 |  | 2 |  | yes |  | T2 |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 23 |  |
| 1153 | ETHYLENE GLYCOL DIETHYL ETHER |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.84 |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 1154 | DIETHYLAMINE |  |  | 3 | FC |  | II |  | 3+8 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 0.7 |  | 2 |  | yes |  | T2 |  | II A |  | yes |  | PP, EP, EX, A |  | 1 |  | 23 |  |
| 1155 | DIETHYL ETHER |  |  | 3 | F1 |  | I |  | 3 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  | 0.71 |  | 1 |  | yes |  | T4 |  | II B |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1157 | DIISOBUTYL KETONE |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.81 |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 1159 | DIISOPROPYL ETHER |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.72 |  | 3 |  | yes |  | T2 |  | II A |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1160 | DIMETHYLAMINE AQUEOUS SOLUTION |  |  | 3 | FC |  | II |  | 3+8 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 0.82 |  | 2 |  | yes |  | T2 |  | II B4) |  | yes |  | PP, EP, EX, A |  | 1 |  | 23 |  |
| 1163 | DIMETHYLHYDRAZINE, UNSYMMETRICAL |  |  | 6.1 | TFC |  | I |  | 6.1+3+8 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 0.78 |  | 1 |  | no |  | T3 |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 23 |  |
| 1165 | DIOXANE |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 1.03 |  | 3 |  | yes |  | T2 |  | II B |  | yes |  | PP, EX, A |  | 1 |  | 6: +14 °C; 17 |  |
| 1167 | DIVINYL ETHER, STABILIZED |  |  | 3 | F1 |  | I |  | 3+unst. |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  | 0.77 |  | 1 |  | yes |  | T2 |  | II B7) |  | yes |  | PP, EX, A |  | 1 |  | 2; 3 |  |
| 1170 | ETHANOL (ETHYL ALCOHOL) or ETHANOL SOLUTION (ETHYL ALCOHOL SOLUTION), aqueous solution with more than 70 % alcohol by volume |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.79 - 0.87 |  | 3 |  | yes |  | T2 |  | II B |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1170 | ETHANOL SOLUTION (ETHYL ALCOHOL SOLUTION), aqueous solution with more than 24 % and not more than 70 % alcohol by volume |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.87 - 0.96 |  | 3 |  | yes |  | T2 |  | II B |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 1171 | ETHYLENE GLYCOL MONOETHYL ETHER |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.93 |  | 3 |  | yes |  | T3 |  | II B |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 1172 | ETHYLENE GLYCOL MONOETHYL ETHER ACETATE |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.98 |  | 3 |  | yes |  | T2 |  | II A |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 1173 | ETHYL ACETATE |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.9 |  | 3 |  | yes |  | T1 |  | II A |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1175 | ETHYLBENZENE |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.87 |  | 3 |  | yes |  | T2 |  | II B |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1177 | 2-ETHYLBUTYL ACETATE |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.88 |  | 3 |  | yes |  | T3 |  | II A |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 1184 | ETHYLENE DICHLORIDE (1,2-dichloroethane) |  |  | 3 | FT1 |  | II |  | 3+6.1 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  | 1.25 |  | 2 |  | no |  | T2 |  | II A |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  |  |  |
| 1188 | ETHYLENE GLYCOL MONOMETHYL ETHER |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.97 |  | 3 |  | yes |  | T3 |  | II B |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 1191 | OCTYL ALDEHYDES (2- ethylcapronaldehyde) |  |  | 3 | F1 |  | III |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 30 |  | 95 |  | 0.82 |  | 2 |  | yes |  | T4 |  | II A |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 1191 | OCTYL ALDEHYDES (n- octaldehyde) |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.82 |  | 3 |  | yes |  | T3 |  | II B4) |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 1193 | ETHYL METHYL KETONE (METHYL ETHYL KETONE) |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.8 |  | 3 |  | yes |  | T1 |  | II A |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1198 | FORMALDEHYDE SOLUTION, FLAMMABLE |  |  | 3 | FC |  | III |  | 3+8 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 1.09 |  | 3 |  | yes |  | T2 |  | II B |  | yes |  | PP, EP, EX, A |  | 0 |  | 34 |  |
| 1199 | FURALDEHYDES (a- furaldehyde) or furfuraldehydes (a- furfurylaldehyde) |  |  | 6.1 | TF1 |  | II |  | 6.1+3 |  | C |  | 2 |  | 2 |  |  |  | 25 |  | 95 |  | 1.16 |  | 2 |  | no |  | T32) |  | II B |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 15 |  |
| 1202 | GAS OIL or DIESEL FUEL or HEATING OIL (LIGHT) (flash-point not more than 60 °C) |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 4 |  | 2 |  |  |  |  |  | 97 |  | < 0.85 |  | 3 |  | yes |  |  |  |  |  | no |  | PP |  | 0 |  |  |  |
| 1202 | GAS OIL complying with standard EN 590: 2004 or DIESEL FUEL or HEATING OIL (LIGHT) with flash-point as specified in EN 590:2004 |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 4 |  | 2 |  |  |  |  |  | 97 |  | 0.82 – 0.85 |  | 3 |  | yes |  |  |  |  |  | no |  | PP |  | 0 |  |  |  |
| 1202 | GAS OIL or DIESEL FUEL or HEATING OIL (LIGHT) (flash-point more than 60 °C but not more than 100 °C) |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 4 |  | 2 |  |  |  |  |  | 97 |  | < 1.1 |  | 3 |  | yes |  |  |  |  |  | no |  | PP |  | 0 |  |  |  |
| 1203 | MOTOR SPIRIT or GASOLINE or PETROL |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.68 - 0.7210) |  | 3 |  | yes |  | T3 |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 14 |  |
| 1203 | MOTOR SPIRIT or GASOLINE or PETROL, with more than 10 % benzene boiling point < 60 °C |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | yes |  | T3 |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 29 |  |
| 1203 | MOTOR SPIRIT or GASOLINE or PETROL with more than 10 % benzene 60 °C < boiling point < 85 °C |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 2 |  | yes |  | T3 |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 23; 29 |  |
| 1203 | MOTOR SPIRIT or GASOLINE or PETROL with more than 10 % benzene 85 °C < boiling point < 115 °C |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  |  |  | 2 |  | yes |  | T3 |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 29 |  |
| 1203 | MOTOR SPIRIT or GASOLINE or PETROL with more than 10 % benzene boiling point > 115 °C |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  |  |  | 2 |  | yes |  | T3 |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 29 |  |
| 1206 | HEPTANES (n- heptane) |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.68 |  | 3 |  | yes |  | T3 |  | II A |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1208 | HEXANES (n- hexane) |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.66 |  | 3 |  | yes |  | T3 |  | II A |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1212 | ISOBUTANOL or ISOBUTYL ALCOHOL |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.8 |  | 3 |  | yes |  | T2 |  | II A |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 1213 | ISOBUTYL ACETATE |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.87 |  | 3 |  | yes |  | T2 |  | II A7) |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1214 | ISOBUTYLAMINE |  |  | 3 | FC |  | II |  | 3+8 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 0.73 |  | 2 |  | yes |  | T2 |  | II A |  | yes |  | PP, EP, EX, A |  | 1 |  | 23 |  |
| 1216 | ISOOCTENE |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.73 |  | 3 |  | yes |  | T3 |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1218 | ISOPRENE, STABILIZED |  |  | 3 | F1 |  | I |  | 3+unst. |  | N |  | 1 |  | 1 |  |  |  |  |  | 95 |  | 0.68 |  | 1 |  | yes |  | T3 |  | II B |  | yes |  | PP, EX, A |  | 1 |  | 2; 3; 5; 16 |  |
| 1219 | ISOPROPANOL or ISOPROPYL ALCOHOL |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.78 |  | 3 |  | yes |  | T2 |  | II A |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1220 | ISOPROPYLE ACETATE |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.88 |  | 3 |  | yes |  | T2 |  | II A |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1221 | ISOPROPYLAMINE |  |  | 3 | FC |  | I |  | 3+8 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  | 0.69 |  | 1 |  | yes |  | T2 |  | II A7) |  | yes |  | PP, EP, EX, A |  | 1 |  |  |  |
| 1223 | KEROSENE |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | < 0.83 |  | 3 |  | yes |  | T3 |  | II A |  | yes |  | PP, EX, A |  | 0 |  | 14 |  |
| 1224 | KETONES, LIQUID, N.O.S. 110 kPa < vp50 < 175 kPa |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 50 |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 27; 29 |  |
| 1224 | KETONES, LIQUID, N.O.S. 110 kPa < vp50 < 150 kPa |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  | 3 |  | 10 |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 27; 29 |  |
| 1224 | KETONES, LIQUID, N.O.S. vp50 < 110 kPa |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 27; 29 |  |
| 1224 | KETONES, LIQUID, N.O.S. |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 0 |  | 14; 27 |  |
| 1229 | MESITYL OXYDE |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.85 |  | 3 |  | yes |  | T2 |  | II B4) |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 1230 | METHANOL |  |  | 3 | FT1 |  | II |  | 3+6.1 |  | N |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 0.79 |  | 2 |  | yes |  | T2 |  | II A |  | yes |  | PP, EP, EX, TOX, A |  | 1 |  | 23 |  |
| 1231 | METHYL ACETATE |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.93 |  | 3 |  | yes |  | T1 |  | II A |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1235 | METHYLAMINE, AQUEOUS SOLUTION |  |  | 3 | FC |  | II |  | 3+8 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  |  |  | 2 |  | yes |  | T2 |  | II A |  | yes |  | PP, EP, EX, A |  | 1 |  |  |  |
| 1243 | METHYLE FORMATE |  |  | 3 | F1 |  | I |  | 3 |  | N |  | 1 |  | 1 |  |  |  |  |  | 97 |  | 0.97 |  | 1 |  | yes |  | T2 |  | II A |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1244 | METHYLHYDRAZINE |  |  | 6.1 | TFC |  | I |  | 6.1+3+8 |  | C |  | 2 |  | 2 |  |  |  | 45 |  | 95 |  | 0.88 |  | 1 |  | no |  | T4 |  | II C5) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  |  |  |
| 1245 | METHYL ISOBUTYL KETONE |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.8 |  | 3 |  | yes |  | T1 |  | II A |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1247 | METHYL METHACRYLATE MONOMER, STABILIZED |  |  | 3 | F1 |  | II |  | 3+unst. |  | C |  | 2 |  | 2 |  |  |  | 40 |  | 95 |  | 0.94 |  | 1 |  | yes |  | T2 |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 3; 5; 16 |  |
| 1262 | OCTANES (n- octane) |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.7 |  | 3 |  | yes |  | T3 |  | II A |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1264 | PARALDEHYDE |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.99 |  | 3 |  | yes |  | T3 |  | II A7) |  | yes |  | PP, EX, A |  | 0 |  | 6: +16 °C; 17 |  |
| 1265 | PENTANES, liquid (2- methylbutane) |  |  | 3 | F1 |  | I |  | 3 |  | N |  | 1 |  | 1 |  |  |  |  |  | 97 |  | 0.62 |  | 1 |  | yes |  | T2 |  | II A |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1265 | PENTANES, liquid (n- pentane) |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 50 |  | 97 |  | 0.63 |  | 3 |  | yes |  | T3 |  | II A |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1265 | PENTANES, liquid (n- pentane) |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  | 3 |  | 10 |  | 97 |  | 0.63 |  | 3 |  | yes |  | T3 |  | II A |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1267 | PETROLEUM CRUDE OIL vp50 > 175 kPa |  |  | 3 | F1 |  | I |  | 3 |  | N |  | 1 |  | 1 |  |  |  |  |  | 97 |  |  |  | 1 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 29 |  |
| 1267 | PETROLEUM CRUDE OIL vp50 > 175 kPa |  |  | 3 | F1 |  | I |  | 3 |  | N |  | 2 |  | 2 |  | 1 |  | 50 |  | 97 |  |  |  | 2 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 29 |  |
| 1267 | PETROLEUM CRUDE OIL with more than 10 % benzene vp50 > 175 kPa |  |  | 3 | F1 |  | I |  | 3 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 29 |  |
| 1267 | PETROLOLEUM CRUDE OIL 110 kPa < vp50 < 175 kPa |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 50 |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 29 |  |
| 1267 | PETROLEUM CRUDE OIL 110 kPa < vp50 < 150 kPa |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  | 3 |  | 10 |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 29 |  |
| 1267 | PETROLEUM CRUDE OIL with more than 10 % benzene 110 kPa < vp50 < 175 kPa |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 29 |  |
| 1267 | PETROLEUM CRUDE OIL vp50 ≤ 110 kPa |  |  | 3 | F1 |  | I |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  |  |  | 3 |  | yes |  | T4 3) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 29 |  |
| 1267 | PETROLEUM CRUDE OIL vp50 < 110 kPa |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 29 |  |
| 1267 | PETROLEUM CRUDE OIL with more than 10 % benzene vp50 ≤ 110 kPa boiling point ≤ 60 °C |  |  | 3 | F1 |  | I |  | 3 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | yes |  | T4 3) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 29 |  |
| 1267 | PETROLEUM CRUDE OIL with more than 10 % benzene vp50 ≤ 110 kPa boiling point ≤ 60 °C |  |  | 3 | F1 |  | I |  | 3 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 1 |  | yes |  | T4 3) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 23; 29 |  |
| 1267 | PETROLEUM CRUDE OIL with more than 10 % benzene vp50 < 110 kPa boiling point < 60 °C |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 29 |  |
| 1267 | PETROLEUM CRUDE OIL with more than 10 % benzene vp50 < 110 kPa boiling point < 60 °C |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 1 |  | yes |  | T4 3) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 23; 29; 38 |  |
| 1267 | PETROLEUM CRUDE OIL with more than 10 % benzene vp50 < 110 kPa 60 °C < boiling point < 85 °C |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 2 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 23; 29 |  |
| 1267 | PETROLEUM CRUDE OIL with more than 10 % benzene vp50 < 110 kPa 85 °C < boiling point < 115 °C |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  |  |  | 2 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 29 |  |
| 1267 | PETROLEUM CRUDE OIL with more than 10 % benzene vp50 < 110 kPa boiling point > 115 °C |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  |  |  | 2 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 29 |  |
| 1267 | PETROLEUM CRUDE OIL |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 0 |  | 14 |  |
| 1268 | PETROLEUM DISTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. vp50 > 175 kPa |  |  | 3 | F1 |  | I |  | 3 |  | N |  | 1 |  | 1 |  |  |  |  |  | 97 |  |  |  | 1 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 27; 29 |  |
| 1268 | PETROLEUM DISTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S vp50 > 175 kPa |  |  | 3 | F1 |  | I |  | 3 |  | N |  | 2 |  | 2 |  | 1 |  | 50 |  | 97 |  |  |  | 2 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 27; 29 |  |
| 1268 | PETROLEUM DISTILLATES, N.O.S. with more than 10 % BENZENE or PETROLEUM PRODUCTS, N.O.S. with more than 10 % benzene vp50 > 175 kPa |  |  | 3 | F1 |  | I |  | 3 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 27; 29 |  |
| 1268 | PETROLEUM DISTILLATES, N.O.S or PETROLEUM PRODUCTS, N.O.S. 110 kPa < vp50 ≤ 175 kPa |  |  | 3 | F1 |  | I |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 50 |  | 97 |  |  |  | 3 |  | yes |  | T4 3) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 27; 29 |  |
| 1268 | PETROLEUM DISTILLATES, N.O.S or PETROLEUM PRODUCTS, N.O.S. 110 kPa < vp50 ≤ 150 kPa |  |  | 3 | F1 |  | I |  | 3 |  | N |  | 2 |  | 2 |  | 3 |  | 10 |  | 97 |  |  |  | 3 |  | yes |  | T4 3) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 27; 29 |  |
| 1268 | PETROLEUM DISTILLATES, N.O.S or PETROLEUM PRODUCTS, N.O.S. 110 kPa < vp50 < 175 kPa |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 50 |  | 97 |  |  |  | 3 |  | yes |  | T4 3) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14 ; 27 ; 29 |  |
| 1268 | PETROLEUM DISTILLATES, N.O.S. with more than 10 % benzene or PETROLEUM PRODUCTS, N.O.S. with more than 10 % benzene 110 kPa < vp50 < 175 kPa |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | yes |  | T4 3) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 27 ; 29 |  |
| 1268 | PETROLEUM DISTILLATES, N.O.S or PETROLEUM PRODUCTS, N.O.S. 110 kPa < vp50 < 150 kPa |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  | 3 |  | 10 |  | 97 |  |  |  | 3 |  | yes |  | T4 3) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14 ; 27 ; 29 |  |
| 1268 | PETROLEUM DISTILLATES, N.O.S or PETROLEUM PRODUCTS, N.O.S. vp50 ≤ 110 kPa |  |  | 3 | F1 |  | I |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  |  |  | 3 |  | yes |  | T4 3) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 27; 29 |  |
| 1268 | PETROLEUM DISTILLATES, N.O.S or PETROLEUM PRODUCTS, N.O.S. vp50 < 110 kPa |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  |  |  | 3 |  | yes |  | T4 3) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14 ; 27 ; 29 |  |
| 1268 | PETROLEUM DISTILLATES, N.O.S. with more than 10 % benzene or PETROLEUM PRODUCTS, N.O.S. with more than 10 % benzene vp50 ≤ 110 kPa boiling point ≤ 60 °C |  |  | 3 | F1 |  | I |  | 3 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | yes |  | T4 3) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 27; 29 |  |
| 1268 | PETROLEUM DISTILLATES, N.O.S. with more than 10 % benzene or PETROLEUM PRODUCTS, N.O.S. with more than 10 % benzene vp50 ≤ 110 kPa boiling point ≤ 60 °C |  |  | 3 | F1 |  | I |  | 3 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 1 |  | yes |  | T4 3) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 23; 27; 29 |  |
| 1268 | PETROLEUM DISTILLATES, N.O.S. with more than 10 % benzene or PETROLEUM PRODUCTS, N.O.S. with more than 10 % benzene vp50 ≤ 110 kPa boiling point ≤ 60 °C |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | yes |  | T4 3) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 27 ; 29 |  |
| 1268 | PETROLEUM DISTILLATES, N.O.S. with more than 10 % benzene or PETROLEUM PRODUCTS, N.O.S. with more than 10 % benzene vp50 ≤ 110 kPa boiling point ≤ 60 °C |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 1 |  | yes |  | T4 3) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 23; 27; 29; 38 |  |
| 1268 | PETROLEUM DISTILLATES, N.O.S. with more than 10 % benzene or PETROLEUM PRODUCTS, N.O.S. with more than 10 % benzene vp50 < 110 kPa 60°C < boiling point < 85 °C |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 0.77 |  | 2 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 23; 27; 29 |  |
| 1268 | PETROLEUM DISTILATES, N.O.S. with more than 10 % BENZENE or PETROLEUM PRODUCTS, N.O.S., benzene heart cut with more than 10 % benzene vp50 < 110 kPa 60 °C < boiling point < 85 °C |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 2 |  | yes |  | T3 |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 23; 27; 29 |  |
| 1268 | PETROLEUM DISTILLATES, N.O.S. with more than 10 % BENZENE or PETROLEUM PRODUCTS, N.O.S, with more than 10 % benzene, vp50 < 110 kPa 85 °C < boiling point < 115 °C |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  |  |  | 2 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 27; 29 |  |
| 1268 | PETROLEUM DISTILLATES, N.O.S. with more than 10 % benzene or PETROLEUM PRODUCTS, N.O.S with more than 10 % benzene vp50 < 110 kPa boiling point > 115 °C |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  |  |  | 2 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 27; 29 |  |
| 1268 | PETROLEUM DISTILLATES, N.O.S. or PETROLEUM PRODUCTS, N.O.S. |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 0 |  | 14; 27 |  |
| 1268 | PETROLEUM DISTILLATES, N.O.S or PETROLEUM PRODUCTS, N.O.S. ( naphtha) 110 kPa < vp50 < 175 kPa |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 50 |  | 97 |  | 0.735 |  | 3 |  | yes |  | T3 |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 14; 27; 29 |  |
| 1268 | PETROLEUM DISTILLATES, N.O.S or PETROLEUM PRODUCTS, N.O.S. ( naphtha) 110 kPa < vp50 < 150 kPa |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  | 3 |  | 10 |  | 97 |  | 0.735 |  | 3 |  | yes |  | T3 |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 14; 29 |  |
| 1268 | PETROLEUM DISTILLATES, N.O.S or PETROLEUM PRODUCTS, N.O.S. ( naphtha) vp50 < 110 kPa |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.735 |  | 3 |  | yes |  | T3 |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 14; 29 |  |
| 1268 | PETROLEUM DISTILLATES, N.O.S, or PETROLEUM PRODUCTS, N.O.S. ( benzene heart cut) vp50 < 110 kPa |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.765 |  | 3 |  | yes |  | T3 |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 14; 29 |  |
| 1274 | n-PROPANOL or n-PROPYL ALCOHOL |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.8 |  | 3 |  | yes |  | T2 |  | II B |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1274 | n-PROPANOL or n-PROPYL ALCOHOL |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.8 |  | 3 |  | yes |  | T2 |  | II B |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 1275 | PROPIONALDEHYDE |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 0.81 |  | 2 |  | yes |  | T4 |  | II B |  | yes |  | PP, EX, A |  | 1 |  | 15; 23 |  |
| 1276 | n-PROPYL ACETATE |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.88 |  | 3 |  | yes |  | T1 |  | II A |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1277 | PROPYLAMINE (1-aminopropane) |  |  | 3 | FC |  | II |  | 3+8 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 0.72 |  | 2 |  | yes |  | T32) |  | II A |  | yes |  | PP, EP, EX, A |  | 1 |  | 23 |  |
| 1278 | 1-CHLOROPROPANE (propyl chloride) |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 0.89 |  | 2 |  | yes |  | T1 |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 23 |  |
| 1279 | 1,2-DICHLOROPROPANEor propyl dichloride |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 45 |  | 95 |  | 1.16 |  | 2 |  | yes |  | T1 |  | II A8) |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1280 | PROPYLENE OXIDE |  |  | 3 | F1 |  | I |  | 3+unst. |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  | 0.83 |  | 1 |  | yes |  | T2 |  | II B |  | yes |  | PP, EX, A |  | 1 |  | 2; 12; 31 |  |
| 1282 | PYRIDINE |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.98 |  | 3 |  | yes |  | T1 |  | II A8) |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1289 | SODIUM METHYLATE SOLUTION in alcohol |  |  | 3 | FC |  | III |  | 3+8 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.969 |  | 3 |  | yes |  | T2 |  | II A |  | yes |  | PP, EP, EX, A |  | 0 |  | 34 |  |
| 1294 | TOLUENE |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.87 |  | 3 |  | yes |  | T1 |  | II A8) |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1296 | TRIETHYLAMINE |  |  | 3 | FC |  | II |  | 3+8 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  | 0.73 |  | 2 |  | yes |  | T3 |  | II A8) |  | yes |  | PP, EP, EX, A |  | 1 |  |  |  |
| 1300 | TURPENTINE SUBSTITUTE |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.78 |  | 3 |  | yes |  | T3 |  | II B4) |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 1301 | VINYL ACETATE, STABILIZED |  |  | 3 | F1 |  | II |  | 3+unst. |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.93 |  | 2 |  | yes |  | T2 |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 3; 5; 16 |  |
| 1307 | XYLENES (o- xylene) |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.88 |  | 3 |  | yes |  | T1 |  | II A |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 1307 | XYLENES (m- xylene) |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.86 |  | 3 |  | yes |  | T1 |  | II A |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 1307 | XYLENES (p- xylene) |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  | 2 |  |  |  | 97 |  | 0.86 |  | 3 |  | yes |  | T1 |  | II A |  | yes |  | PP, EX, A |  | 0 |  | 6: +17 °C; 17 |  |
| 1307 | XYLENES (mixture with melting point < 0° C) |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  | T1 |  | II A |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1307 | XYLENES (mixture with melting point < 0° C) |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  | T1 |  | II A |  | yes |  | PP, EX, A |  | 0 |  | TRANS/WP.15/AC.2/19/Add.1 page 9 |  |
| 1307 | XYLENES (mixture with melting point > 0° C < 13° C) |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  | 2 |  |  |  | 97 |  |  |  | 3 |  | yes |  | T1 |  | II A |  | yes |  | PP, EX, A |  | 0 |  | 6: +17° C; 17 |  |
| 1541 | ACETONE CYANOHYDRIN, STABILIZED |  |  | 6.1 | T1 |  | I |  | 6.1+unst. |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  | 0.932 |  | 1 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 3 |  |
| 1545 | ALLYL ISOTHIOCYANATE, STABILIZED |  |  | 6.1 | TF1 |  | II |  | 6.1+3+unst. |  | C |  | 2 |  | 2 |  |  |  | 30 |  | 95 |  | 1.02 |  | 1 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 2; 3 |  |
| 1547 | ANILINE |  |  | 6.1 | T1 |  | II |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 25 |  | 95 |  | 1.02 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  |  |  |
| 1578 | CHLORONITROBENZENES, SOLID, MOLTEN (p- chloronitrobenzene) |  |  | 6.1 | T2 |  | II |  | 6.1 |  | C |  | 2 |  | 1 |  | 2 |  | 25 |  | 95 |  | 1.37 |  | 2 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 7; 17; 26 |  |
| 1578 | CHLORONITROBENZENES, SOLID, MOLTEN (p- chloronitrobenzene) |  |  | 6.1 | T2 |  | II |  | 6.1 |  | C |  | 2 |  | 1 |  | 4 |  | 25 |  | 95 |  | 1.37 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 7; 17; 20: +112 °C; 26 |  |
| 1591 | o-DICHLOROBENZENE |  |  | 6.1 | T1 |  | III |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 25 |  | 95 |  | 1.32 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 0 |  |  |  |
| 1593 | DICHLOROMETHANE (methyl chloride) |  |  | 6.1 | T1 |  | III |  | 6.1 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 1.33 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 0 |  | 23 |  |
| 1594 | DIETHYLSULPHATE |  |  | 6.1 | T1 |  | II |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 25 |  | 95 |  | 1.18 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  |  |  |
| 1595 | DIMETHYL SULPHATE |  |  | 6.1 | TC1 |  | I |  | 6.1+8 |  | C |  | 2 |  | 2 |  |  |  | 25 |  | 95 |  | 1.33 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  |  |  |
| 1604 | ETHYLENEDIAMINE |  |  | 8 | CF1 |  | II |  | 8+3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.9 |  | 3 |  | yes |  | T2 |  | II A |  | yes |  | PP, EP, EX, A |  | 1 |  | 6: +12 °C; 17; 34 |  |
| 1605 | ETHYLENE DIBROMIDE |  |  | 6.1 | T1 |  | I |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 30 |  | 95 |  | 2.18 |  | 1 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 6: +14 °C; 17 |  |
| 1648 | ACETONITRILE (methyl cyanide) |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.78 |  | 3 |  | yes |  | T1 |  | II A |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1662 | NITROBENZENE |  |  | 6.1 | T1 |  | II |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 25 |  | 95 |  | 1.21 |  | 2 |  | no |  | T1 |  | II B |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 6:+10 °C ; 17 |  |
| 1663 | NITROPHENOLS |  |  | 6.1 | T2 |  | III |  | 6.1 |  | C |  | 2 |  | 2 |  | 2 |  | 25 |  | 95 |  |  |  | 2 |  | no |  | T1 |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 0 |  | 7; 17 |  |
| 1663 | NITROPHENOLS |  |  | 6.1 | T2 |  | III |  | 6.1 |  | C |  | 2 |  | 2 |  | 4 |  | 25 |  | 95 |  |  |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 0 |  | 7; 17; 20: +65 °C |  |
| 1664 | NITROTOLUENES, LIQUID (o- nitrotoluene) |  |  | 6.1 | T1 |  | II |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 25 |  | 95 |  | 1.16 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 17 |  |
| 1708 | TOLUIDINES, LIQUID (o- toluidine) |  |  | 6.1 | T1 |  | II |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 25 |  | 95 |  | 1 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  |  |  |
| 1708 | TOLUIDINES, LIQUID (m- toluidine) |  |  | 6.1 | T1 |  | II |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 25 |  | 95 |  | 1.03 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  |  |  |
| 1710 | TRICHLOROETHYLENE |  |  | 6.1 | T1 |  | III |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  | 1.46 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 0 |  | 15 |  |
| 1715 | ACETIC ANHYDRIDE |  |  | 8 | CF1 |  | II |  | 8+3 |  | N |  | 2 |  | 3 |  |  |  | 10 |  | 97 |  | 1.08 |  | 3 |  | yes |  | T2 |  | II A |  | yes |  | PP, EP, EX, A |  | 1 |  | 34 |  |
| 1717 | ACETYL CHLORIDE |  |  | 3 | FC |  | II |  | 3+8 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 1.1 |  | 2 |  | yes |  | T2 |  | II A8) |  | yes |  | PP, EP, EX, A |  | 1 |  | 23 |  |
| 1718 | BUTYL ACIDE PHOSPHATE |  |  | 8 | C3 |  | III |  | 8 |  | N |  | 4 |  | 3 |  |  |  |  |  | 97 |  | 0.98 |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 34 |  |
| 1719 | CAUSTIC ALKALI LIQUID, N.O.S. |  |  | 8 | C5 |  | II |  | 8 |  | N |  | 4 |  | 2 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 27; 30; 34 |  |
| 1719 | CAUSTIC ALKALI LIQUID, N.O.S. |  |  | 8 | C5 |  | III |  | 8 |  | N |  | 4 |  | 2 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 27; 30; 34 |  |
| 1738 | BENZYL CHLORIDE |  |  | 6.1 | TC1 |  | II |  | 6.1+8+3 |  | C |  | 2 |  | 2 |  |  |  | 25 |  | 95 |  | 1.1 |  | 2 |  | no |  | T1 |  | II A8) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  |  |  |
| 1742 | BORON TRIFLUORIDE ACETIC ACID COMPLEX, LIQUID |  |  | 8 | C3 |  | II |  | 8 |  | N |  | 4 |  | 2 |  |  |  |  |  | 97 |  | 1.35 |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 34 |  |
| 1750 | CHLORACETIC ACID SOLUTION |  |  | 6.1 | TC1 |  | II |  | 6.1+8 |  | C |  | 2 |  | 2 |  | 2 |  | 25 |  | 95 |  | 1.58 |  | 2 |  | no |  | T1 |  | II A |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 7; 17 |  |
| 1750 | CHLORACETIC ACID SOLUTION |  |  | 6.1 | TC1 |  | II |  | 6.1+8 |  | C |  | 2 |  | 1 |  | 4 |  | 25 |  | 95 |  | 1.58 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 7; 17; 20: +111 °C; 26 |  |
| 1760 | CORROSIVE LIQUID, N.O.S. |  |  | 8 | C9 |  | I |  | 8 |  | N |  | 2 |  | 3 |  |  |  | 10 |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 27; 34 |  |
| 1760 | CORROSIVE LIQUID, N.O.S. |  |  | 8 | C9 |  | II |  | 8 |  | N |  | 2 |  | 3 |  |  |  | 10 |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 27; 34 |  |
| 1760 | CORROSIVE LIQUID, N.O.S. |  |  | 8 | C9 |  | III |  | 8 |  | N |  | 4 |  | 3 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 27; 34 |  |
| 1760 | CORROSIVE LIQUID, N.O.S. ( sodium mercaptobenzothiazole, 50 % aqueous solution) |  |  | 8 | C9 |  | II |  | 8 |  | N |  | 4 |  | 2 |  |  |  |  |  | 97 |  | 1.25 |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 34 |  |
| 1760 | CORROSIVE LIQUID, N.O.S. ( fatty alcohol, C12-C14) |  |  | 8 | C9 |  | III |  | 8 |  | N |  | 4 |  | 2 |  |  |  |  |  | 97 |  | 0.89 |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 34 |  |
| 1760 | CORROSIVE LIQUID, N.O.S. ( ethylene diaminetetraacetic acid, tetrasodium salt, 40 % aqueous solution) |  |  | 8 | C9 |  | III |  | 8 |  | N |  | 4 |  | 2 |  |  |  |  |  | 97 |  | 1.28 |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 34 |  |
| 1764 | DICHLOROACETIC ACID |  |  | 8 | C3 |  | II |  | 8 |  | N |  | 3 |  | 3 |  |  |  |  |  | 97 |  | 1.56 |  | 3 |  | yes |  | T1 |  | II A |  | yes |  | PP, EP, EX, A |  | 0 |  | 17; 34 |  |
| 1778 | FLUOROSILICIC ACID |  |  | 8 | C1 |  | II |  | 8 |  | N |  | 2 |  | 3 |  |  |  | 10 |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 34 |  |
| 1779 | FORMIC ACID with more than 85% acid by mass |  |  | 8 | CF1 |  | II |  | 8+3 |  | N |  | 2 |  | 3 |  |  |  | 10 |  | 97 |  | 1.22 |  | 3 |  | yes |  | T1 |  | II A |  | yes |  | PP, EP, EX, A |  | 1 |  | 6: +12 °C; 17; 34 |  |
| 1780 | FUMARYL CHLORIDE |  |  | 8 | C3 |  | II |  | 8 |  | N |  | 2 |  | 3 |  |  |  | 10 |  | 97 |  | 1.41 |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 8; 34 |  |
| 1783 | HEXAMETHYLENEDIAMINE SOLUTION |  |  | 8 | C7 |  | II |  | 8 |  | N |  | 3 |  | 2 |  | 2 |  |  |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, A |  | 0 |  | 7; 17; 34 |  |
| 1783 | HEXAMETHYLENEDIAMINE SOLUTION |  |  | 8 | C7 |  | III |  | 8 |  | N |  | 3 |  | 2 |  | 2 |  |  |  | 97 |  |  |  | 3 |  | yes |  | T3 |  | II B4) |  | yes |  | PP, EP, EX, A |  | 0 |  | 7; 17; 34 |  |
| 1789 | HYDROCHLORIC ACID |  |  | 8 | C1 |  | II |  | 8 |  | N |  | 2 |  | 3 |  |  |  | 10 |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 34 |  |
| 1789 | HYDROCHLORIC ACID |  |  | 8 | C1 |  | III |  | 8 |  | N |  | 4 |  | 3 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 34 |  |
| 1805 | PHOSPHORIC ACID, SOLUTION, with more than 80 % (volume) acid |  |  | 8 | C1 |  | III |  | 8 |  | N |  | 4 |  | 3 |  | 2 |  |  |  | 95 |  | >1.6 |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 7; 17; 22; 34 |  |
| 1805 | PHOSPHORIC ACID, SOLUTION, with 80 % (volume) acid, OR LESS |  |  | 8 | C1 |  | III |  | 8 |  | N |  | 4 |  | 3 |  |  |  |  |  | 97 |  | 1.00 - 1.6 |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 22; 34 |  |
| 1814 | POTASSIUM HYDROXIDE SOLUTION |  |  | 8 | C5 |  | II |  | 8 |  | N |  | 4 |  | 2 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 30; 34 |  |
| 1814 | POTASSIUM HYDROXIDE SOLUTION |  |  | 8 | C5 |  | III |  | 8 |  | N |  | 4 |  | 2 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 30; 34 |  |
| 1823 | SODIUM HYDROXIDE, SOLID, molten |  |  | 8 | C6 |  | II |  | 8 |  | N |  | 4 |  | 1 |  | 4 |  |  |  | 95 |  | 2.13 |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 7; 17; 34 |  |
| 1824 | SODIUM HYDROXIDE SOLUTION |  |  | 8 | C5 |  | II |  | 8 |  | N |  | 4 |  | 2 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 30; 34 |  |
| 1824 | SODIUM HYDROXIDE SOLUTION |  |  | 8 | C5 |  | III |  | 8 |  | N |  | 4 |  | 2 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 30; 34 |  |
| 1830 | SULPHURIC ACID with more than 51 % acid |  |  | 8 | C1 |  | II |  | 8 |  | N |  | 4 |  | 3 |  |  |  |  |  | 97 |  | 1.4 - 1.84 |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 8; 22; 30; 34 |  |
| 1831 | SULPHURIC ACID, FUMING |  |  | 8 | CT1 |  | I |  | 8+6.1 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  | 1.94 |  | 1 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 8 |  |
| 1832 | SULPHURIC ACID, SPENT |  |  | 8 | C1 |  | II |  | 8 |  | N |  | 4 |  | 3 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 8; 30; 34 |  |
| 1846 | CARBON TETRACHLORIDE |  |  | 6.1 | T1 |  | II |  | 6.1 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 1.59 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 23 |  |
| 1848 | PROPIONIC ACID with not less than 10% and less than 90% acid by mass |  |  | 8 | C3 |  | III |  | 8 |  | N |  | 3 |  | 3 |  |  |  |  |  | 97 |  | 0.99 |  | 3 |  | yes |  | T1 |  | II A7) |  | yes |  | PP, EP, EX, A |  | 0 |  | 34 |  |
| 1863 | FUEL, AVIATION, TURBINE ENGINE vp50 > 175 kPa |  |  | 3 | F1 |  | I |  | 3 |  | N |  | 1 |  | 1 |  |  |  |  |  | 97 |  |  |  | 1 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 29 |  |
| 1863 | FUEL, AVIATION, TURBINE ENGINE vp50 > 175 kPa |  |  | 3 | F1 |  | I |  | 3 |  | N |  | 2 |  | 2 |  | 1 |  | 50 |  | 97 |  |  |  | 2 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 29 |  |
| 1863 | FUEL, AVIATION, TURBINE ENGINE with more than 10 % benzene vp50 > 175 kPa |  |  | 3 | F1 |  | I |  | 3 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 29 |  |
| 1863 | FUEL, AVIATION, TURBINE ENGINE 110 kPa < vp50 < 175 kPa |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 50 |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 29 |  |
| 1863 | FUEL, AVIATION, TURBINE ENGINE 110 kPa < vp50 < 150 kPa |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  | 3 |  | 10 |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 29 |  |
| 1863 | FUEL, AVIATION, TURBINE ENGINE with more than 10 % benzene 110 kPa < vp50 < 175 kPa |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 29 |  |
| 1863 | FUEL, AVIATION, TURBINE ENGINE vp50 < 110 kPa |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 29 |  |
| 1863 | FUEL, AVIATION, TURBINE ENGINE with more than 10 % benzene vp50 < 110 kPa boiling point < 60 °C |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 29 |  |
| 1863 | FUEL, AVIATION, TURBINE ENGINE with more than 10 % benzene vp50 < 110 kPa 60 °C < boiling point < 85 °C |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 2 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 29 |  |
| 1863 | FUEL, AVIATION, TURBINE ENGINE with more than 10 % benzene vp50 < 110 kPa 85 °C < boiling point < 115 °C |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  |  |  | 2 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 29 |  |
| 1863 | FUEL, AVIATION, TURBINE ENGINE with more than 10 % benzene vp50 < 110 kPa boiling point > 115 °C |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  |  |  | 2 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 29 |  |
| 1863 | FUEL, AVIATION, TURBINE ENGINE |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 0 |  | 14 |  |
| 1888 | CHLOROFORM |  |  | 6.1 | T1 |  | III |  | 6.1 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 1.48 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 0 |  | 23 |  |
| 1897 | TETRACHLOROETHYLENE |  |  | 6.1 | T1 |  | III |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  | 1.62 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 0 |  |  |  |
| 1912 | METHYL CHLORIDE AND METHYLENE CHLORIDE MIXTURE |  |  | 2 | 2F |  |  |  | 2.1 |  | G |  | 1 |  | 1 |  |  |  |  |  | 91 |  |  |  | 1 |  | yes |  | T1 |  | II A8) |  | yes |  | PP, EX, A |  | 1 |  | 31 |  |
| 1915 | CYCLOHEXANONE |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.95 |  | 3 |  | yes |  | T2 |  | II A |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 1917 | ETHYL ACRYLATE, STABILIZED |  |  | 3 | F1 |  | II |  | 3+unst. |  | C |  | 2 |  | 2 |  |  |  | 40 |  | 95 |  | 0.92 |  | 1 |  | yes |  | T2 |  | II B |  | yes |  | PP, EX, A |  | 1 |  | 3; 5 |  |
| 1918 | ISOPROPYLBENZENE (cumene) |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.86 |  | 3 |  | yes |  | T2 |  | II A8) |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 1919 | METHYL ACRYLATE, STABILIZED |  |  | 3 | F1 |  | II |  | 3+unst. |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 0.95 |  | 1 |  | yes |  | T2 |  | II B |  | yes |  | PP, EX, A |  | 1 |  | 3; 5; 23 |  |
| 1920 | NONANES |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.70 - 0.75 |  | 3 |  | yes |  | T3 |  | II A |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 1922 | PYRROLIDINE |  |  | 3 | FC |  | II |  | 3+8 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  | 0.86 |  | 2 |  | yes |  | T2 |  | II A |  | yes |  | PP, EP, EX, A |  | 1 |  |  |  |
| 1965 | HYDROCARBON GAS MIXTURE, LIQUEFIED, N.O.S., (mixture A) |  |  | 2 | 2F |  |  |  | 2.1 |  | G |  | 1 |  | 1 |  |  |  |  |  | 91 |  |  |  | 1 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 31 |  |
| 1965 | HYDROCARBON GAS MIXTURE, LIQUEFIED, N.O.S., (mixture A0) |  |  | 2 | 2F |  |  |  | 2.1 |  | G |  | 1 |  | 1 |  |  |  |  |  | 91 |  |  |  | 1 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 31 |  |
| 1965 | HYDROCARBON GAS MIXTURE, LIQUEFIED, N.O.S., (mixture A01) |  |  | 2 | 2F |  |  |  | 2.1 |  | G |  | 1 |  | 1 |  |  |  |  |  | 91 |  |  |  | 1 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 31 |  |
| 1965 | HYDROCARBON GAS MIXTURE, LIQUEFIED, N.O.S., (mixture A02) |  |  | 2 | 2F |  |  |  | 2.1 |  | G |  | 1 |  | 1 |  |  |  |  |  | 91 |  |  |  | 1 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 31 |  |
| 1965 | HYDROCARBON GAS MIXTURE, LIQUEFIED, N.O.S., (mixture A1) |  |  | 2 | 2F |  |  |  | 2.1 |  | G |  | 1 |  | 1 |  |  |  |  |  | 91 |  |  |  | 1 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 31 |  |
| 1965 | HYDROCARBON GAS MIXTURE, LIQUEFIED, N.O.S., (mixture B) |  |  | 2 | 2F |  |  |  | 2.1 |  | G |  | 1 |  | 1 |  |  |  |  |  | 91 |  |  |  | 1 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 31 |  |
| 1965 | HYDROCARBON GAS MIXTURE, LIQUEFIED, N.O.S., (mixture B1) |  |  | 2 | 2F |  |  |  | 2.1 |  | G |  | 1 |  | 1 |  |  |  |  |  | 91 |  |  |  | 1 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 31 |  |
| 1965 | HYDROCARBON GAS MIXTURE, LIQUEFIED, N.O.S., (mixture B2) |  |  | 2 | 2F |  |  |  | 2.1 |  | G |  | 1 |  | 1 |  |  |  |  |  | 91 |  |  |  | 1 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 31 |  |
| 1965 | HYDROCARBON GAS MIXTURE, LIQUEFIED, N.O.S., (mixture C) |  |  | 2 | 2F |  |  |  | 2.1 |  | G |  | 1 |  | 1 |  |  |  |  |  | 91 |  |  |  | 1 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 31 |  |
| 1969 | ISOBUTANE |  |  | 2 | 2F |  |  |  | 2.1 |  | G |  | 1 |  | 1 |  |  |  |  |  | 91 |  |  |  | 1 |  | yes |  | T21) |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 31 |  |
| 1978 | PROPANE |  |  | 2 | 2F |  |  |  | 2.1 |  | G |  | 1 |  | 1 |  |  |  |  |  | 91 |  |  |  | 1 |  | yes |  | T1 |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 31 |  |
| 1986 | ALCOHOLS, FLAMMABLE, TOXIC, N.O.S. boiling point < 60 °C |  |  | 3 | FT1 |  | I |  | 3+6.1 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 27; 29 |  |
| 1986 | ALCOHOLS, FLAMMABLE, TOXIC, N.O.S. 60 °C < boiling point < 85 °C |  |  | 3 | FT1 |  | II |  | 3+6.1 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 2 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 23; 27; 29 |  |
| 1986 | ALCOHOLS, FLAMMABLE, TOXIC, N.O.S. 85 °C < boiling point < 115 °C |  |  | 3 | FT1 |  | II |  | 3+6.1 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  |  |  | 2 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 27; 29 |  |
| 1986 | ALCOHOLS, FLAMMABLE, TOXIC, N.O.S. boiling point > 115 °C |  |  | 3 | FT1 |  | II |  | 3+6.1 |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  |  |  | 2 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 27; 29 |  |
| 1986 | ALCOHOLS, FLAMMABLE, TOXIC, N.O.S. 60 °C < boiling point < 85 °C |  |  | 3 | FT1 |  | III |  | 3+6.1 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 2 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 0 |  | 23; 27; 29 |  |
| 1986 | ALCOHOLS, FLAMMABLE, TOXIC, N.O.S. 85 °C < boiling point < 115 °C |  |  | 3 | FT1 |  | III |  | 3+6.1 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  |  |  | 2 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 0 |  | 27; 29 |  |
| 1986 | ALCOHOLS, FLAMMABLE, TOXIC, N.O.S. boiling point > 115 °C |  |  | 3 | FT1 |  | III |  | 3+6.1 |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  |  |  | 2 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 0 |  | 27; 29 |  |
| 1987 | ALCOHOLS, N.O.S. 110 kPa < vp50 < 175 kPa |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 50 |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 27; 29 |  |
| 1987 | ALCOHOLS, N.O.S. 110 kPa < vp50 < 150 kPa |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  | 3 |  | 10 |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 27; 29 |  |
| 1987 | ALCOHOLS, N.O.S. vp50 < 110 kPa |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 27; 29 |  |
| 1987 | ALCOHOLS, N.O.S. ( tert-butanol 90 % (mass)/methanol 10 % (mass) mixture) |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  |  |  | 3 |  | yes |  | T1 |  | II A |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 1987 | ALCOHOLS, N.O.S. |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 0 |  | 14; 27 |  |
| 1987 | ALCOHOLS, N.O.S. (CYCLOHEXANOL) |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  | 2 |  |  |  | 95 |  | 0.95 |  | 3 |  | yes |  | T3 |  | II A |  | yes |  | PP, EX, A |  | 0 |  | 7; 17 |  |
| 1987 | ALCOHOLS, N.O.S. (CYCLOHEXANOL) |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  | 4 |  |  |  | 95 |  | 0.95 |  | 3 |  | yes |  |  |  |  |  | no |  | PP |  | 0 |  | 7; 17; 20: +46 °C |  |
| 1989 | ALDEHYDES, FLAMMABLE, N.O.S. 110 kPa < vp50 < 175 kPa |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 50 |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 27; 29 |  |
| 1989 | ALDEHYDES, FLAMMABLE, N.O.S. 110 kPa < vp50 < 150 kPa |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  | 3 |  | 10 |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 27; 29 |  |
| 1989 | ALDEHYDES, FLAMMABLE, N.O.S. vp50 < 110 kPa |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 27; 29 |  |
| 1989 | ALDEHYDES, FLAMMABLE, N.O.S. |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 0 |  | 14; 27 |  |
| 1991 | CHLOROPRENE, STABILIZED |  |  | 3 | FT1 |  | I |  | 3+6.1+unst. |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 0.96 |  | 1 |  | no |  | T2 |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 3; 5; 23 |  |
| 1992 | FLAMMABLE LIQUID, TOXIC, N.O.S. boiling point < 60 °C |  |  | 3 | FT1 |  | I |  | 3+6.1 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 27; 29 |  |
| 1992 | FLAMMABLE LIQUID, TOXIC, N.O.S. boiling point < 60 °C |  |  | 3 | FT1 |  | II |  | 3+6.1 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 27; 29 |  |
| 1992 | FLAMMABLE LIQUID, TOXIC, N.O.S. 60 °C < boiling point < 85 °C |  |  | 3 | FT1 |  | II |  | 3+6.1 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 2 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 23; 27; 29 |  |
| 1992 | FLAMMABLE LIQUID, TOXIC, N.O.S. 85 °C < boiling point < 115 °C |  |  | 3 | FT1 |  | II |  | 3+6.1 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  |  |  | 2 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 27; 29 |  |
| 1992 | FLAMMABLE LIQUID, TOXIC, N.O.S. boiling point > 115 °C |  |  | 3 | FT1 |  | II |  | 3+6.1 |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  |  |  | 2 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 27; 29 |  |
| 1992 | FLAMMABLE LIQUID, TOXIC, N.O.S. boiling point < 60 °C |  |  | 3 | FT1 |  | III |  | 3+6.1 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 0 |  | 27; 29 |  |
| 1992 | FLAMMABLE LIQUID, TOXIC, N.O.S. 60 °C < boiling point < 85 °C |  |  | 3 | FT1 |  | III |  | 3+6.1 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 2 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 0 |  | 23; 27; 29 |  |
| 1992 | FLAMMABLE LIQUID, TOXIC, N.O.S. 85 °C < boiling point < 115 °C |  |  | 3 | FT1 |  | III |  | 3+6.1 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  |  |  | 2 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 0 |  | 27; 29 |  |
| 1992 | FLAMMABLE LIQUID, TOXIC, N.O.S. boiling point > 115 °C |  |  | 3 | FT1 |  | III |  | 3+6.1 |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  |  |  | 2 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 0 |  | 27; 29 |  |
| 1993 | FLAMMABLE LIQUID, N.O.S. vp50 >175 kPa |  |  | 3 | F1 |  | I |  | 3 |  | N |  | 1 |  | 1 |  |  |  |  |  | 97 |  |  |  | 1 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 27; 29 |  |
| 1993 | FLAMMABLE LIQUID, N.O.S. vp50 >175 kPa |  |  | 3 | F1 |  | I |  | 3 |  | N |  | 2 |  | 2 |  | 1 |  | 50 |  | 97 |  |  |  | 2 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 27; 29 |  |
| 1993 | FLAMMABLE LIQUID, N.O.S. with more than 10 % benzene vp50 >175 kPa |  |  | 3 | F1 |  | I |  | 3 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 27; 29 |  |
| 1993 | FLAMMABLE LIQUID, N.O.S. 110 kPa < vp50 < 175 kPa |  |  | 3 | F1 |  | I |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 50 |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 27; 29 |  |
| 1993 | FLAMMABLE LIQUID, N.O.S. 110 kPa < vp50 < 175 kPa |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 50 |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 27; 29 |  |
| 1993 | FLAMMABLE LIQUID, N.O.S. 110 kPa < vp50 < 150 kPa |  |  | 3 | F1 |  | I |  | 3 |  | N |  | 2 |  | 2 |  | 3 |  | 10 |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 27; 29 |  |
| 1993 | FLAMMABLE LIQUID, N.O.S. 110 kPa < vp50 < 150 kPa |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  | 3 |  | 10 |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 27; 29 |  |
| 1993 | FLAMMABLE LIQUID, N.O.S. with more than 10 % benzene 110 kPa < vp50 < 175 kPa |  |  | 3 | F1 |  | I |  | 3 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 27; 29 |  |
| 1993 | FLAMMABLE LIQUID, N.O.S. with more than 10 % benzene 110 kPa < vp50 < 175 kPa |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 27; 29 |  |
| 1993 | FLAMMABLE LIQUID, N.O.S. vp50 < 110 kPa |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 27; 29 |  |
| 1993 | FLAMMABLE LIQUID, N.O.S. with more than 10 % benzene vp50 < 110 kPa boiling point < 60 °C |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 27; 29 |  |
| 1993 | FLAMMABLE LIQUID, N.O.S. with more than 10 % benzene vp50 < 110 kPa 60 °C < boiling point < 85 °C |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 2 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 23; 27; 29 |  |
| 1993 | FLAMMABLE LIQUID, N.O.S. with more than 10 % benzene vp50 < 110 kPa 85 °C < boiling point < 115 °C |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  |  |  | 2 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 27; 29 |  |
| 1993 | FLAMMABLE LIQUID, N.O.S. with more than 10 % benzene vp50 < 110 kPa boiling point > 115 °C |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  |  |  | 2 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 27; 29 |  |
| 1993 | FLAMMABLE LIQUID, N.O.S. |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 0 |  | 14; 27 |  |
| 1993 | FLAMMABLE LIQUID, N.O.S. with more than 10 % benzene 60 °C < boiling point < 85 °C |  |  | 3 | F1 |  | III |  | 3 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 2 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 0 |  | 23; 27; 29 |  |
| 1993 | FLAMMABLE LIQUID, N.O.S. with more than 10 % benzene 85 °C < boiling point < 115 °C |  |  | 3 | F1 |  | III |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  |  |  | 2 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 0 |  | 27; 29 |  |
| 1993 | FLAMMABLE LIQUID, N.O.S. with more than 10 % benzene boiling point > 115 °C |  |  | 3 | F1 |  | III |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  |  |  | 2 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 0 |  | 27; 29 |  |
| 1993 | FLAMMABLE LIQUID, N.O.S. (cyclohexanone/cyclohexanol mixture) |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.95 |  | 3 |  | yes |  | T3 |  | II A |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 1999 | TARS, LIQUID, including road asphalt and oils, bitumen and cut backs |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 4 |  | 2 |  | 2 |  |  |  | 97 |  |  |  | 3 |  | yes |  | T3 |  | II A7) |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 2014 | HYDROGEN PEROXIDE, AQUEOUS SOLUTION with not less than 20 % but not more than 60 % hydrogen peroxide (stabilized as necessary) |  |  | 5.1 | OC1 |  | II |  | 5.1+8+unst. |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  | 1.2 |  | 2 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 3; 33 |  |
| 2021 | CHLOROPHENOLS, LIQUID (2- chlorophenol) |  |  | 6.1 | T1 |  | III |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 25 |  | 95 |  | 1.23 |  | 2 |  | no |  | T1 |  | II A7) |  | yes |  | PP, EP, EX, TOX, A |  | 0 |  | 6: +10 °C; 17 |  |
| 2022 | CRESILIC ACID |  |  | 6.1 | TC1 |  | II |  | 6.1+8 |  | C |  | 2 |  | 2 |  |  |  | 25 |  | 95 |  | 1.03 |  | 2 |  | no |  | T1 |  | II A |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 6: +16 °C; 17 |  |
| 2023 | EPICHLORHYDRINE |  |  | 6.1 | TF1 |  | II |  | 6.1+3 |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  | 1.18 |  | 2 |  | no |  | T2 |  | II B |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 5 |  |
| 2031 | NITRIC ACID, other than red fuming, with more than 70 % acid |  |  | 8 | CO1 |  | I |  | 8+5.1 |  | N |  | 2 |  | 3 |  |  |  | 10 |  | 97 |  | 1.41 (at 68 % HNO3) |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 34 |  |
| 2031 | NITRIC ACID, other than red fuming, with not more than 70 % acid |  |  | 8 | CO1 |  | II |  | 8 |  | N |  | 2 |  | 3 |  |  |  | 10 |  | 97 |  | 1.5111) (at 68 % HNO3) |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 34 |  |
| 2032 | NITRIC ACID, RED FUMING |  |  | 8 | COT |  | I |  | 8+5.1+6.1 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  | 1.51 |  | 1 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  |  |  |
| 2045 | ISOBUTYRALDEHYDE (ISOBUTYL ALDEHYDE) |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 0.79 |  | 2 |  | yes |  | T4 |  | II A7) |  | yes |  | PP, EX, A |  | 1 |  | 15; 23 |  |
| 2046 | CYMENES |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.88 |  | 3 |  | yes |  | T2 |  | II A |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 2047 | DICHLOROPROPENES (2,3- dichloroprop-1-ene) |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 45 |  | 95 |  | 1.2 |  | 2 |  | yes |  | T1 |  | II A |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 2047 | DICHLOROPROPENES ( mixtures of 2,3-dichloroprop-1-ene and 1,3-dichloroprop-1-ene) |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 45 |  | 95 |  | 1.23 |  | 2 |  | yes |  | T21) |  | II A |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 2047 | DICHLOROPROPENES ( mixtures of 2,3-dichloroprop-1-ene and 1,3-dichloroprop-1-ene) |  |  | 3 | F1 |  | III |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 45 |  | 95 |  | 1.23 |  | 2 |  | yes |  | T21) |  | II A |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 2047 | DICHLOROPROPENES (1,3- dichloroprop-1-ene) |  |  | 3 | F1 |  | III |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 40 |  | 95 |  | 1.23 |  | 2 |  | yes |  | T21) |  | II A7) |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 2048 | DICYCLOPENTADIENE |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  | 2 |  |  |  | 95 |  | 0.94 |  | 3 |  | yes |  | T1 |  | II B4) |  | yes |  | PP, EX, A |  | 0 |  | 7; 17 |  |
| 2050 | DIISOBUTYLENE, ISOMERIC COMPOUNDS |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.72 |  | 3 |  | yes |  | T32) |  | II A7) |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 2051 | 2-DIMETHYLAMINOETHANOL |  |  | 8 | CF1 |  | II |  | 8+3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.89 |  | 3 |  | yes |  | T3 |  | II A |  | yes |  | PP, EP, EX, A |  | 1 |  | 34 |  |
| 2053 | METHYL ISOBUTIL CARBINOL |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.81 |  | 3 |  | yes |  | T2 |  | II B4) |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 2054 | MORPHOLINE |  |  | 8 | CF1 |  | I |  | 8+3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 1 |  | 3 |  | yes |  | T3 |  | II A |  | yes |  | PP, EP, EX, A |  | 1 |  | 34 |  |
| 2055 | STYRENE MONOMER, STABILIZED |  |  | 3 | F1 |  | III |  | 3+unst. |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.91 |  | 3 |  | yes |  | T1 |  | II A |  | yes |  | PP, EX, A |  | 0 |  | 3; 5; 16 |  |
| 2056 | TETRAHYDROFURAN |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.89 |  | 3 |  | yes |  | T3 |  | II B |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 2057 | TRIPROPYLÈNE |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.744 |  | 3 |  | yes |  | T3 |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 2057 | TRIPROPYLENE |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.73 |  | 3 |  | yes |  | T3 |  | II B4) |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 2078 | TOLUENE DIISOCYANATE (and isomeric mixtures) (2,4- toluene diisocyanate) |  |  | 6.1 | T1 |  | II |  | 6.1 |  | C |  | 2 |  | 2 |  | 2 |  | 25 |  | 95 |  | 1.22 |  | 2 |  | no |  | T1 |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 2; 7; 8; 17 |  |
| 2078 | TOLUENE DIISOCYANATE (and isomeric mixtures) (2,4- toluene diisocyanate) |  |  | 6.1 | T1 |  | II |  | 6.1 |  | C |  | 2 |  | 1 |  | 4 |  | 25 |  | 95 |  | 1.22 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 2; 7; 8; 17; 20: +112 °C; 26 |  |
| 2079 | DIETHYLENETRIAMINE |  |  | 8 | C7 |  | II |  | 8 |  | N |  | 4 |  | 2 |  |  |  |  |  | 97 |  | 0.96 |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 34 |  |
| 2205 | ADIPONITRILE |  |  | 6.1 | T1 |  | III |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 25 |  | 95 |  | 0.96 |  | 2 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 0 |  | 17 |  |
| 2206 | ISOCYANATES, TOXIC, N.O.S. (4- chlorophenyl isocyanate) |  |  | 6.1 | T1 |  | II |  | 6.1 |  | C |  | 2 |  | 2 |  | 4 |  | 25 |  | 95 |  | 1.25 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 7; 17 |  |
| 2209 | FORMALDEHYDE SOLUTION with not less than 25 % formaldehyde |  |  | 8 | C9 |  | III |  | 8 |  | N |  | 4 |  | 2 |  |  |  |  |  | 97 |  | 1.09 |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 15; 34 |  |
| 2215 | MALEIC ANHYDRIDE, MOLTEN |  |  | 8 | C3 |  | III |  | 8 |  | N |  | 3 |  | 3 |  | 2 |  |  |  | 95 |  | 0.93 |  | 3 |  | yes |  | T2 |  | II B4) |  | yes |  | PP, EP, EX, A |  | 0 |  | 7; 17; 34 |  |
| 2215 | MALEIC ANHYDRIDE, MOLTEN |  |  | 8 | C3 |  | III |  | 8 |  | N |  | 3 |  | 3 |  | 4 |  |  |  | 95 |  | 0.93 |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 7; 17; 20: +88 °C; 34 |  |
| 2218 | ACRYLIC ACID, STABILIZED |  |  | 8 | CF1 |  | II |  | 8+3+unst. |  | C |  | 2 |  | 2 |  | 4 |  | 30 |  | 95 |  | 1.05 |  | 1 |  | yes |  | T2 |  | II A7) |  | yes |  | PP, EP, EX, A |  | 1 |  | 3; 4; 5; 17 |  |
| 2227 | n-BUTYL METHACRYLATE, STABILIZED |  |  | 3 | F1 |  | III |  | 3+unst. |  | C |  | 2 |  | 2 |  |  |  | 25 |  | 95 |  | 0.9 |  | 1 |  | yes |  | T3 |  | II A |  | yes |  | PP, EX, A |  | 0 |  | 3; 5 |  |
| 2238 | CHLOROTOLUENES (m- chlorotoluene) |  |  | 3 | F1 |  | III |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 30 |  | 95 |  | 1.08 |  | 2 |  | yes |  | T1 |  | II A7) |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 2238 | CHLOROTOLUENES (o- chlorotoluene) |  |  | 3 | F1 |  | III |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 30 |  | 95 |  | 1.08 |  | 2 |  | yes |  | T1 |  | II A7) |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 2238 | CHLOROTOLUENES (p- chlorotoluene) |  |  | 3 | F1 |  | III |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 30 |  | 95 |  | 1.07 |  | 2 |  | yes |  | T1 |  | II A7) |  | yes |  | PP, EX, A |  | 0 |  | 6: +11 °C; 17 |  |
| 2241 | CYCLOHEPTANE |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.81 |  | 3 |  | yes |  | T43) |  | II A |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 2247 | n-DECANE |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.73 |  | 3 |  | yes |  | T4 |  | II A |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 2248 | DI-n-BUTYLAMINE |  |  | 8 | CF1 |  | II |  | 8+3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.76 |  | 3 |  | yes |  | T3 |  | II A7) |  | yes |  | PP, EP, EX, A |  | 1 |  | 34 |  |
| 2259 | TRIETHYLENETETRAMINE |  |  | 8 | C7 |  | II |  | 8 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.98 |  | 3 |  | yes |  | T2 |  | II B4) |  | yes |  | PP, EP, EX, A |  | 1 |  | 34 |  |
| 2263 | DIMETHYLCYCLOHEXANES (cis-1,4- dimethylcyclohexaneS) |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  | 0.78 |  | 2 |  | yes |  | T43) |  | II A7) |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 2263 | DIMETHYLCYCLOHEXANES (trans-1,4- dimethylcyclohexaneS) |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  | 0.76 |  | 2 |  | yes |  | T43) |  | II A7) |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 2264 | N,N-DIMETHYLCYCLO- HEXYLAMINE |  |  | 8 | CF1 |  | II |  | 8+3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.85 |  | 3 |  | yes |  | T3 |  | II B4) |  | yes |  | PP, EP, EX, A |  | 1 |  | 34 |  |
| 2265 | N,N-DIMETHYLFORMAMIDE |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.95 |  | 3 |  | yes |  | T2 |  | II A |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 2266 | DIMETHYL-N-PROPYLAMINE |  |  | 3 | FC |  | II |  | 3+8 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 0.72 |  | 2 |  | yes |  | T4 |  | II A |  | yes |  | PP, EP, EX, A |  | 1 |  | 23 |  |
| 2276 | 2-ETHYLHEXYLAMINE |  |  | 3 | FC |  | III |  | 3+8 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.79 |  | 3 |  | yes |  | T3 |  | II A7) |  | yes |  | PP, EP, EX, A |  | 0 |  | 34 |  |
| 2278 | n-HEPTENE |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.7 |  | 3 |  | yes |  | T3 |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 2280 | HEXAMETHYLENEDIAMINE, SOLID, molten |  |  | 8 | C8 |  | III |  | 8 |  | N |  | 3 |  | 3 |  | 2 |  |  |  | 95 |  | 0.83 |  | 3 |  | yes |  | T3 |  | II B4) |  | yes |  | PP, EP, EX, A |  | 0 |  | 7; 17; 34 |  |
| 2280 | HEXAMETHYLENEDIAMINE, SOLID, molten |  |  | 8 | C8 |  | III |  | 8 |  | N |  | 3 |  | 3 |  | 4 |  |  |  | 95 |  | 0.83 |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 7; 17; 20: +66 °C; 34 |  |
| 2282 | HEXANOLS |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.83 |  | 3 |  | yes |  | T3 |  | II A |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 2286 | PENTAMETHYLHEPTANE |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.75 |  | 3 |  | yes |  | T2 |  | II A7) |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 2288 | ISOHEXENES |  |  | 3 | F1 |  | II |  | 3+unst. |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 0.735 |  | 2 |  | yes |  | T2 |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 3; 23 |  |
| 2289 | ISOPHORONEDIAMINE |  |  | 8 | C7 |  | III |  | 8 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.92 |  | 3 |  | yes |  | T2 |  | II A |  | yes |  | PP, EP, EX, A |  | 0 |  | 17; 34 |  |
| 2302 | 5-METHYLHEXAN-2-ONE |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0,81 |  | 3 |  | yes |  | T1 |  | II A |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 2303 | ISOPROPENYLBENZENE |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.91 |  | 3 |  | yes |  | T2 |  | II B |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 2309 | OCTADIENE (1,7- octadiene) |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.75 |  | 3 |  | yes |  | T3 |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 2311 | PHENETIDINES |  |  | 6.1 | T1 |  | III |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 25 |  | 95 |  | 1.07 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 0 |  | 6: +7 °C; 17 |  |
| 2312 | PHENOL, MOLTEN |  |  | 6.1 | T1 |  | II |  | 6.1 |  | C |  | 2 |  | 2 |  | 4 |  | 25 |  | 95 |  | 1.07 |  | 2 |  | no |  | T1 |  | II A8) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 7; 17 |  |
| 2312 | PHENOL, MOLTEN |  |  | 6.1 | T1 |  | II |  | 6.1 |  | C |  | 2 |  | 2 |  | 4 |  | 25 |  | 95 |  | 1.07 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 7; 17; 20: +67 °C |  |
| 2320 | TETRAETHYLENEPENTAMINE |  |  | 8 | C7 |  | III |  | 8 |  | N |  | 4 |  | 2 |  |  |  |  |  | 97 |  | 1 |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 34 |  |
| 2321 | TRICHLOROBENZENES, LIQUID (1,2,4- trichlorobenzene) |  |  | 6.1 | T1 |  | III |  | 6.1 |  | C |  | 2 |  | 2 |  | 2 |  | 25 |  | 95 |  | 1.45 |  | 2 |  | no |  | T1 |  | II A |  | yes |  | PP, EP, EX, TOX, A |  | 0 |  | 7; 17 |  |
| 2321 | TRICHLOROBENZENES, LIQUID (1,2,4- trichlorobenzene) |  |  | 6.1 | T1 |  | III |  | 6.1 |  | C |  | 2 |  | 2 |  | 4 |  | 25 |  | 95 |  | 1.45 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 0 |  | 7; 17; 20: +95 °C |  |
| 2323 | TRIETHYL PHOSPHITE |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.8 |  | 3 |  | yes |  | T3 |  | II B4) |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 2324 | TRIISOBUTYLENE |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.76 |  | 3 |  | yes |  | T2 |  | II B4) |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 2325 | 1,3,5-TRIMETHYLBENZENE |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.87 |  | 3 |  | yes |  | T1 |  | II A |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 2333 | ALLYL ACETATE |  |  | 3 | FT1 |  | II |  | 3+6.1 |  | C |  | 2 |  | 2 |  |  |  | 40 |  | 95 |  | 0.93 |  | 2 |  | no |  | T2 |  | II A7) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  |  |  |
| 2348 | BUTYL ACRYLATES, STABILIZED (n- butylacrylate, stabilized) |  |  | 3 | F1 |  | III |  | 3+unst. |  | C |  | 2 |  | 2 |  |  |  | 30 |  | 95 |  | 0.9 |  | 1 |  | yes |  | T3 |  | II B |  | yes |  | PP, EX, A |  | 0 |  | 3; 5 |  |
| 2350 | BUTYL METHYL ETHER |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.74 |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 2356 | 2-CHLOROPROPANE |  |  | 3 | F1 |  | I |  | 3 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 0.86 |  | 2 |  | yes |  | T1 |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 23 |  |
| 2357 | CYCLOHEXYLAMINE |  |  | 8 | CF1 |  | II |  | 8+3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.86 |  | 3 |  | yes |  | T3 |  | II A8) |  | yes |  | PP, EP, EX, A |  | 1 |  | 34 |  |
| 2362 | 1,1-DICHLOROETHANE |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 1.17 |  | 2 |  | yes |  | T2 |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 23 |  |
| 2370 | 1-HEXENE |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.67 |  | 3 |  | yes |  | T3 |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 2381 | DIMÉTHYL DISULPHIDE |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 40 |  | 95 |  | 1.063 |  | 2 |  | yes |  | T2 |  | IIB |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 2382 | DIMETHYLHYDRAZINE, SYMMETRICAL |  |  | 6.1 | TF1 |  | I |  | 6.1+3 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  | 0.83 |  | 1 |  | yes |  | T43) |  | II C |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  |  |  |
| 2383 | DIPROPYLAMINE |  |  | 3 | FC |  | II |  | 3+8 |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  | 0.74 |  | 2 |  | no |  | T3 |  | II A |  | yes |  | PP, EP, EX, A |  | 1 |  |  |  |
| 2397 | 3-METHYLBUTAN-2-ONE |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.81 |  | 3 |  | yes |  | T1 |  | II A |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 2398 | METHYL tert-BUTYL ETHER |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.74 |  | 3 |  | yes |  | T1 |  | II A |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 2404 | PROPIONITRILE |  |  | 3 | FT1 |  | II |  | 3+6.1 |  | C |  | 2 |  | 2 |  |  |  | 45 |  | 95 |  | 0.78 |  | 2 |  | no |  | T19) |  | II A |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  |  |  |
| 2414 | THIOPHENE |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 1.06 |  | 3 |  | yes |  | T2 |  | II A |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 2430 | ALKYLPHENOLS, SOLID, N.O.S. ( nonylphenol, isomeric mixture, molten) |  |  | 8 | C4 |  | II |  | 8 |  | N |  | 3 |  | 3 |  | 2 |  |  |  | 95 |  | 0.95 |  | 3 |  | yes |  | T2 |  | II A7) |  | yes |  | PP, EP, EX, A |  | 0 |  | 7; 17; 34 |  |
| 2430 | ALKYLPHENOLS, SOLID, N.O.S. ( nonylphenol, isomeric mixture, molten) |  |  | 8 | C4 |  | II |  | 8 |  | N |  | 3 |  | 1 |  | 4 |  |  |  | 95 |  | 0.95 |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 7; 17; 20: +125 °C; 34 |  |
| 2432 | N,N-DIETHYLANILINE |  |  | 6.1 | T1 |  | III |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 25 |  | 95 |  | 0.93 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 0 |  |  |  |
| 2448 | SULPHUR, MOLTEN |  |  | 4.1 | F3 |  | III |  | 4.1 |  | N |  | 4 |  | 1 |  | 4 |  |  |  | 95 |  | 2.07 |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP, TOX*, A |  | 0 |  | * Toximeter for H2S; 7; 20: +150 °C; 28; 32 |  |
| 2458 | HEXADIENES |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.72 |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 2477 | METHYL ISOTHIOCYANATE |  |  | 6.1 | TF1 |  | I |  | 6.1+3 |  | C |  | 2 |  | 2 |  | 2 |  | 35 |  | 95 |  | 1.0711) |  | 2 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 7; 17 |  |
| 2485 | n-BUTYL ISOCYANATE |  |  | 6.1 | TF1 |  | I |  | 6.1+3 |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  | 0.89 |  | 1 |  | no |  | T2 |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  |  |  |
| 2486 | ISOBUTYL ISOCYANATE |  |  | 3 | FT1 |  | II |  | 3+6.1 |  | C |  | 2 |  | 2 |  |  |  | 40 |  | 95 |  |  |  | 2 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  |  |  |
| 2487 | PHENYL ISOCYANATE |  |  | 6.1 | TF1 |  | I |  | 6.1+3 |  | C |  | 2 |  | 2 |  |  |  | 25 |  | 95 |  | 1.1 |  | 1 |  | no |  | T1 |  | II A |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  |  |  |
| 2490 | DICHLOROISOPROPYL ETHER |  |  | 6.1 | T1 |  | II |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 25 |  | 95 |  | 1.11 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  |  |  |
| 2491 | ETHANOLAMINE or ETHANOLAMINE SOLUTION |  |  | 8 | C7 |  | III |  | 8 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 1.02 |  | 3 |  | yes |  | T2 |  | II B4) |  | yes |  | PP, EP, EX, A |  | 0 |  | 17; 34 |  |
| 2493 | HEXAMETHYLENEIMINE |  |  | 3 | FC |  | II |  | 3+8 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.88 |  | 3 |  | yes |  | T32) |  | II A |  | yes |  | PP, EP, EX, A |  | 1 |  | 34 |  |
| 2496 | PROPIONIC ANHYDRIDE |  |  | 8 | C3 |  | III |  | 8 |  | N |  | 4 |  | 3 |  |  |  |  |  | 97 |  | 1.02 |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 34 |  |
| 2518 | 1,5,9-CYCLODODECATRIENE |  |  | 6.1 | T1 |  | III |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 25 |  | 95 |  | 0.9 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 0 |  |  |  |
| 2527 | ISOBUTYL ACRYLATE, STABILIZED |  |  | 3 | F1 |  | III |  | 3+unst. |  | C |  | 2 |  | 2 |  |  |  | 30 |  | 95 |  | 0.89 |  | 1 |  | yes |  | T2 |  | II B9) |  | yes |  | PP, EX, A |  | 0 |  | 3; 5 |  |
| 2528 | ISOBUTYL ISOBUTYRATE |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.86 |  | 3 |  | yes |  | T2 |  | II A |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 2531 | METHACRYLIC ACID, STABILIZED |  |  | 8 | C3 |  | II |  | 8+unst. |  | C |  | 2 |  | 2 |  | 4 |  | 25 |  | 95 |  | 1.02 |  | 1 |  | yes |  | T2 |  | II B4) |  | yes |  | PP, EP, EX, A |  | 0 |  | 3; 4; 5; 17 |  |
| 2564 | TRICHLOROACETIC ACID SOLUTION |  |  | 8 | C3 |  | II |  | 8 |  | N |  | 3 |  | 3 |  | 2 |  |  |  | 95 |  | 1.6211) |  | 3 |  | yes |  | T1 |  | II A7) |  | yes |  | PP, EP, EX, A |  | 0 |  | 7; 17; 22; 34 |  |
| 2564 | TRICHLOROACETIC ACID SOLUTION |  |  | 8 | C3 |  | III |  | 8 |  | N |  | 4 |  | 3 |  |  |  |  |  | 97 |  | 1.6211) |  | 3 |  | yes |  | T1 |  | II A7) |  | yes |  | PP, EP, EX, A |  | 0 |  | 22; 34 |  |
| 2574 | TRICRESYL PHOSPHATE with more than 3 % ortho isomer |  |  | 6.1 | T1 |  | II |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 25 |  | 95 |  | 1.18 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  |  |  |
| 2579 | PIPERAZINE, molten |  |  | 8 | C8 |  | III |  | 8 |  | N |  | 3 |  | 3 |  | 2 |  |  |  | 95 |  | 0.9 |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 7; 17; 34 |  |
| 2582 | FERRIC CHLORIDE SOLUTION |  |  | 8 | C1 |  | III |  | 8 |  | N |  | 4 |  | 3 |  |  |  |  |  | 97 |  | 1.45 |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 22; 30; 34 |  |
| 2586 | ALKYLSULFONIC ACIDS, LIQUID or ARLSULFONIC ACIDS, LIQUID with not more than 5 % free sulphuric acid |  |  | 8 | C3 |  | III |  | 8 |  | N |  | 4 |  | 3 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 34 |  |
| 2608 | NITROPROPANES |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 1 |  | 3 |  | yes |  | T2 |  | II B7) |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 2615 | ETHYL PROPYL ETHER |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.73 |  | 3 |  | yes |  | T43) |  | II A7) |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 2618 | VINYLTOLUENES, STABILIZED |  |  | 3 | F1 |  | III |  | 3+unst. |  | C |  | 2 |  | 2 |  |  |  | 25 |  | 95 |  | 0.92 |  | 1 |  | yes |  | T1 |  | II B4) |  | yes |  | PP, EX, A |  | 0 |  | 3; 5 |  |
| 2651 | 4,4’-DIAMINODIPHENYL-METHANE |  |  | 6.1 | T2 |  | III |  | 6.1 |  | C |  | 2 |  | 2 |  | 2 |  | 25 |  | 95 |  | 1 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 0 |  | 7; 17 |  |
| 2672 | AMMONIA SOLUTION (relative density between 0.880 and 0.957 at 15 °C in water, with more than 10 % but not more than 35 % ammonia) |  |  | 8 | C5 |  | III |  | 8 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.8810) - 0.9610) |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 34 |  |
| 2683 | AMMONIUM SULPHIDE SOLUTION |  |  | 8 | CFT |  | II |  | 8+3+6.1 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  |  |  | 2 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 15; 16 |  |
| 2693 | BISULPHITES, AQUEOUS SOLUTION, N.O.S. |  |  | 8 | C1 |  | III |  | 8 |  | N |  | 4 |  | 3 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 27; 34 |  |
| 2709 | BUTYLBENZENES |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.87 |  | 3 |  | yes |  | T2 |  | II A |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 2733 | AMINES, FLAMMABLE, CORROSIVE, N.O.S. or POLYAMINES, FLAMMABLE, CORROSIVE, N.O.S. (2- aminobutane) |  |  | 3 | FC |  | II |  | 3+8 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 0.72 |  | 2 |  | yes |  | T43) |  | II A |  | yes |  | PP, EP, EX, A |  | 1 |  | 23 |  |
| 2735 | AMINES, LIQUID, CORROSIVE, N.O.S. or POLYAMINES, LIQUID, CORROSIVE, N.O.S. |  |  | 8 | C7 |  | I |  | 8 |  | N |  | 4 |  | 2 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 27; 34 |  |
| 2735 | AMINES, LIQUID, CORROSIVE, N.O.S. or POLYAMINES, LIQUID, CORROSIVE, N.O.S. |  |  | 8 | C7 |  | II |  | 8 |  | N |  | 4 |  | 2 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 27; 34 |  |
| 2735 | AMINES, LIQUID, CORROSIVE, N.O.S. or POLYAMINES, LIQUID, CORROSIVE, N.O.S. |  |  | 8 | C7 |  | III |  | 8 |  | N |  | 4 |  | 2 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 27; 34 |  |
| 2754 | N-ETHYLTOLUIDINES ( N-ethyl-o-toluidine) |  |  | 6.1 | T1 |  | II |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 25 |  | 95 |  | 0.94 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  |  |  |
| 2754 | N-ETHYLTOLUIDINES ( N-ethyl-m-toluidine) |  |  | 6.1 | T1 |  | II |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 25 |  | 95 |  | 0.94 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  |  |  |
| 2754 | N-ETHYLTOLUIDINES (N-ethyl-o-toluidine and N-ethyl-m-toluidine MIXTURES) |  |  | 6.1 | T1 |  | II |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 25 |  | 95 |  | 0.94 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  |  |  |
| 2754 | N-ETHYLTOLUIDINES ( N-ethyl-p-toluidine) |  |  | 6.1 | T1 |  | II |  | 6.1 |  | C |  | 2 |  | 2 |  | 2 |  | 25 |  | 95 |  | 0.94 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 7; 17 |  |
| 2785 | 4-THIAPENTANAL (3-MÉTHYLMERCAPTO- PROPIONALDÉHYDE) |  |  | 6.1 | T1 |  | III |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 25 |  | 95 |  | 1.04 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 0 |  |  |  |
| 2789 | ACETIC ACID, GLACIAL or ACETIC ACID SOLUTION, more than 80 % acid, by mass |  |  | 8 | CF1 |  | II |  | 8+3 |  | N |  | 2 |  | 3 |  | 2 |  | 10 |  | 95 |  | 1.05 (with 100 % acid) |  | 3 |  | yes |  | T1 |  | II A |  | yes |  | PP, EP, EX, A |  | 1 |  | 7 ; 17 ; 34 |  |
| 2790 | ACETIC ACID SOLUTION, not less than 50 % but not more than 80 % acid, by mass |  |  | 8 | C3 |  | II |  | 8 |  | N |  | 2 |  | 3 |  |  |  | 10 |  | 95 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 34 |  |
| 2790 | ACETIC ACID SOLUTION, more than 10 % and less than 50 % acid, by mass |  |  | 8 | C3 |  | III |  | 8 |  | N |  | 2 |  | 3 |  |  |  | 10 |  | 95 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 34 |  |
| 2796 | BATTERY FLUID, ACID |  |  | 8 | C1 |  | II |  | 8 |  | N |  | 4 |  | 3 |  |  |  |  |  | 97 |  | 1.00 - 1.84 |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 8; 22; 30; 34 |  |
| 2796 | SULPHURIC ACID with not more than 51 % acid |  |  | 8 | C1 |  | II |  | 8 |  | N |  | 4 |  | 3 |  |  |  |  |  | 97 |  | 1.00 - 1.41 |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 8; 22; 30; 34 |  |
| 2797 | BATTERY FLUID, ALKALI |  |  | 8 | C5 |  | II |  | 8 |  | N |  | 4 |  | 3 |  |  |  |  |  | 97 |  | 1.00 - 2.13 |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 22; 30; 34 |  |
| 2810 | TOXIC LIQUID, ORGANIC, N.O.S. boiling point < 60 °C |  |  | 6.1 | T1 |  | I |  | 6.1 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 27; 29 |  |
| 2810 | TOXIC LIQUID, ORGANIC, N.O.S. 60 °C < boiling point < 85 °C |  |  | 6.1 | T1 |  | I |  | 6.1 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 1 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 23; 27; 29 |  |
| 2810 | TOXIC LIQUID, ORGANIC, N.O.S. 85 °C < boiling point < 115 °C |  |  | 6.1 | T1 |  | I |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  |  |  | 1 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 27; 29 |  |
| 2810 | TOXIC LIQUID, ORGANIC, N.O.S. boiling point > 115 °C |  |  | 6.1 | T1 |  | I |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  |  |  | 1 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 27; 29 |  |
| 2810 | TOXIC LIQUID, ORGANIC, N.O.S. boiling point < 60 °C |  |  | 6.1 | T1 |  | II |  | 6.1 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 27; 29 |  |
| 2810 | TOXIC LIQUID, ORGANIC, N.O.S. 60 °C < boiling point < 85 °C |  |  | 6.1 | T1 |  | II |  | 6.1 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 23; 27; 29 |  |
| 2810 | TOXIC LIQUID, ORGANIC, N.O.S. 85 °C < boiling point < 115 °C |  |  | 6.1 | T1 |  | II |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  |  |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 27; 29 |  |
| 2810 | TOXIC LIQUID, ORGANIC, N.O.S. boiling point > 115 °C |  |  | 6.1 | T1 |  | II |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  |  |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 27; 29 |  |
| 2810 | TOXIC LIQUID, ORGANIC, N.O.S. boiling point < 60 °C |  |  | 6.1 | T1 |  | III |  | 6.1 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 0 |  | 27; 29 |  |
| 2810 | TOXIC LIQUID, ORGANIC, N.O.S. 60 °C < boiling point < 85 °C |  |  | 6.1 | T1 |  | III |  | 6.1 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 0 |  | 23; 27; 29 |  |
| 2810 | TOXIC LIQUID, ORGANIC, N.O.S. 85 °C < boiling point < 115 °C |  |  | 6.1 | T1 |  | III |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  |  |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 0 |  | 27; 29 |  |
| 2810 | TOXIC LIQUID, ORGANIC, N.O.S. boiling point > 115 °C |  |  | 6.1 | T1 |  | III |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  |  |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 0 |  | 27; 29 |  |
| 2811 | TOXIC SOLID, ORGANIC, N.O.S. (1,2,3- trichlorobenzene, molten) |  |  | 6.1 | T2 |  | III |  | 6.1 |  | C |  | 2 |  | 2 |  | 2 |  | 25 |  | 95 |  |  |  | 2 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 0 |  | 7; 17; 22 |  |
| 2811 | TOXIC SOLID, ORGANIC, N.O.S. (1,2,3- trichlorobenzene, molten) |  |  | 6.1 | T2 |  | III |  | 6.1 |  | C |  | 2 |  | 2 |  | 4 |  | 25 |  | 95 |  |  |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 0 |  | 7; 17; 20: +92 °C; 22 |  |
| 2811 | TOXIC SOLID, ORGANIC, N.O.S. (1,3,5- trichlorobenzene, molten) |  |  | 6.1 | T2 |  | III |  | 6.1 |  | C |  | 2 |  | 2 |  | 2 |  | 25 |  | 95 |  |  |  | 2 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 0 |  | 7; 17; 22 |  |
| 2811 | TOXIC SOLID, ORGANIC, N.O.S. (1,3,5- trichlorobenzene, molten) |  |  | 6.1 | T2 |  | III |  | 6.1 |  | C |  | 2 |  | 2 |  | 4 |  | 25 |  | 95 |  |  |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 0 |  | 7; 17; 20: +92 °C; 22 |  |
| 2815 | N-AMINOETHYLPIPERAZINE |  |  | 8 | C7 |  | III |  | 8 |  | N |  | 4 |  | 2 |  |  |  |  |  | 97 |  | 0.98 |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 34 |  |
| 2820 | BUTYRIC ACID |  |  | 8 | C3 |  | III |  | 8 |  | N |  | 2 |  | 3 |  |  |  | 10 |  | 97 |  | 0.96 |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 34 |  |
| 2829 | CAPROIC ACID |  |  | 8 | C3 |  | III |  | 8 |  | N |  | 4 |  | 3 |  |  |  |  |  | 97 |  | 0.92 |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 34 |  |
| 2831 | 1,1,1-TRICHLOROETHANE |  |  | 6.1 | T1 |  | III |  | 6.1 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  | 1.34 |  | 2 |  | yes |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 0 |  | 23 |  |
| 2850 | PROPYLENE TETRAMER |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 4 |  | 2 |  |  |  |  |  | 97 |  | 0.76 |  | 3 |  | yes |  |  |  |  |  | no |  | PP |  | 0 |  |  |  |
| 2874 | FURFURYL ALCOHOL |  |  | 6.1 | T1 |  | III |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 25 |  | 95 |  | 1.13 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 0 |  |  |  |
| 2904 | PHENOLATES, LIQUID |  |  | 8 | C9 |  | III |  | 8 |  | N |  | 4 |  | 2 |  |  |  |  |  | 97 |  | 1.13-1.18 |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 34 |  |
| 2920 | CORROSIVE LIQUID, FLAMMABLE, N.O.S. (2- propanol and didecyldimethylammonium chloride, aqueous solution) |  |  | 8 | CF1 |  | II |  | 8+3 |  | N |  | 3 |  | 3 |  |  |  |  |  | 95 |  | 0.95 |  | 3 |  | yes |  | T3 |  | II A |  | yes |  | PP, EP, EX, A |  | 1 |  | 34 |  |
| 2920 | CORROSIVE LIQUID, FLAMMABLE, N.O.S. ( aqueous solution of hexadecyltrimethyl-ammonium chloride (50 %) and ethanol (35 %)) |  |  | 8 | CF1 |  | II |  | 8+3 |  | N |  | 2 |  | 3 |  |  |  | 10 |  | 95 |  | 0.9 |  | 3 |  | yes |  | T2 |  | II B |  | yes |  | PP, EP, EX, A |  | 1 |  | 6: +7 °C; 17; 34 |  |
| 2922 | CORROSIVE LIQUID, TOXIC, N.O.S. boiling point < 60 °C |  |  | 8 | CT1 |  | I |  | 8+6.1 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 27; 29 |  |
| 2922 | CORROSIVE LIQUID, TOXIC, N.O.S. 60 °C < boiling point < 85 °C |  |  | 8 | CT1 |  | I |  | 8+6.1 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 1 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 23; 27; 29 |  |
| 2922 | CORROSIVE LIQUID, TOXIC, N.O.S. 85 °C < boiling point < 115 °C |  |  | 8 | CT1 |  | I |  | 8+6.1 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  |  |  | 1 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 27; 29 |  |
| 2922 | CORROSIVE LIQUID, TOXIC, N.O.S. boiling point > 115 °C |  |  | 8 | CT1 |  | I |  | 8+6.1 |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  |  |  | 1 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 27; 29 |  |
| 2922 | CORROSIVE LIQUID, TOXIC, N.O.S. boiling point < 60 °C |  |  | 8 | CT1 |  | II |  | 8+6.1 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 27; 29 |  |
| 2922 | CORROSIVE LIQUID, TOXIC, N.O.S. 60 °C < boiling point < 85 °C |  |  | 8 | CT1 |  | II |  | 8+6.1 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 23; 27; 29 |  |
| 2922 | CORROSIVE LIQUID, TOXIC, N.O.S. 85 °C < boiling point < 115 °C |  |  | 8 | CT1 |  | II |  | 8+6.1 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  |  |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 27; 29 |  |
| 2922 | CORROSIVE LIQUID, TOXIC, N.O.S. boiling point > 115 °C |  |  | 8 | CT1 |  | II |  | 8+6.1 |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  |  |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 27; 29 |  |
| 2922 | CORROSIVE LIQUID, TOXIC, N.O.S. boiling point < 60 °C |  |  | 8 | CT1 |  | III |  | 8+6.1 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 0 |  | 27; 29 |  |
| 2922 | CORROSIVE LIQUID, TOXIC, N.O.S. 60 °C < boiling point < 85 °C |  |  | 8 | CT1 |  | III |  | 8+6.1 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 0 |  | 23; 27; 29 |  |
| 2922 | CORROSIVE LIQUID, TOXIC, N.O.S. 85 °C < boiling point < 115 °C |  |  | 8 | CT1 |  | III |  | 8+6.1 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  |  |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 0 |  | 27; 29 |  |
| 2922 | CORROSIVE LIQUID, TOXIC, N.O.S. boiling point > 115 °C |  |  | 8 | CT1 |  | III |  | 8+6.1 |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  |  |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 0 |  | 27; 29 |  |
| 2924 | FLAMMABLE LIQUID, CORROSIVE, N.O.S. boiling point < 60 °C |  |  | 3 | FC |  | I |  | 3+8 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, A |  | 1 |  | 27; 29 |  |
| 2924 | FLAMMABLE LIQUID, CORROSIVE, N.O.S. boiling point < 60 °C |  |  | 3 | FC |  | II |  | 3+8 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, A |  | 1 |  | 27; 29 |  |
| 2924 | FLAMMABLE LIQUID, CORROSIVE, N.O.S. 60 °C < boiling point < 85 °C |  |  | 3 | FC |  | II |  | 3+8 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 2 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, A |  | 1 |  | 23; 27; 29 |  |
| 2924 | FLAMMABLE LIQUID, CORROSIVE, N.O.S. 85 °C < boiling point < 115 °C |  |  | 3 | FC |  | II |  | 3+8 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  |  |  | 2 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, A |  | 1 |  | 27; 29 |  |
| 2924 | FLAMMABLE LIQUID, CORROSIVE, N.O.S. boiling point > 115 °C |  |  | 3 | FC |  | II |  | 3+8 |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  |  |  | 2 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, A |  | 1 |  | 27; 29 |  |
| 2924 | FLAMMABLE LIQUID, CORROSIVE, N.O.S. |  |  | 3 | FC |  | III |  | 3+8 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  | T4 3) |  | II B4) |  | yes |  | PP, EP, EX, A |  | 0 |  | 27 ; 34 |  |
| 2924 | FLAMMABLE LIQUID, CORROSIVE, N.O.S. ( aqueous solution of dialkyl-(C8-C18)-dimethylammonium chloride and 2-propanol) |  |  | 3 | FC |  | II |  | 3+8 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  | 0.88 |  | 2 |  | yes |  | T2 |  | II A |  | yes |  | PP, EP, EX, A |  | 1 |  |  |  |
| 2927 | TOXIC LIQUID, CORROSIVE, ORGANIC, N.O.S. boiling point < 60 °C |  |  | 6.1 | TC1 |  | I |  | 6.1+8 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 27; 29 |  |
| 2927 | TOXIC LIQUID, CORROSIVE, ORGANIC, N.O.S. 60 °C < boiling point < 85 °C |  |  | 6.1 | TC1 |  | I |  | 6.1+8 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 1 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 23; 27; 29 |  |
| 2927 | TOXIC LIQUID, CORROSIVE, ORGANIC, N.O.S. 85 °C < boiling point < 115 °C |  |  | 6.1 | TC1 |  | I |  | 6.1+8 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  |  |  | 1 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 27; 29 |  |
| 2927 | TOXIC LIQUID, CORROSIVE, ORGANIC, N.O.S. boiling point > 115 °C |  |  | 6.1 | TC1 |  | I |  | 6.1+8 |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  |  |  | 1 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 27; 29 |  |
| 2927 | TOXIC LIQUID, CORROSIVE, ORGANIC, N.O.S. boiling point < 60 °C |  |  | 6.1 | TC1 |  | II |  | 6.1+8 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 27; 29 |  |
| 2927 | TOXIC LIQUID, CORROSIVE, ORGANIC, N.O.S. 60 °C < boiling point < 85 °C |  |  | 6.1 | TC1 |  | II |  | 6.1+8 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 23; 27; 29 |  |
| 2927 | TOXIC LIQUID, CORROSIVE, ORGANIC, N.O.S. 85 °C < boiling point < 115 °C |  |  | 6.1 | TC1 |  | II |  | 6.1+8 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  |  |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 27; 29 |  |
| 2927 | TOXIC LIQUID, CORROSIVE, ORGANIC, N.O.S. boiling point > 115 °C |  |  | 6.1 | TC1 |  | II |  | 6.1+8 |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  |  |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 27; 29 |  |
| 2929 | TOXIC LIQUID, FLAMMABLE, ORGANIC, N.O.S. |  |  | 6.1 | TF1 |  | I |  | 6.1+3 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 27 |  |
| 2929 | TOXIC LIQUID, FLAMMABLE, ORGANIC, N.O.S. boiling point < 60 °C |  |  | 6.1 | TF1 |  | I |  | 6.1+3 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 27; 29 |  |
| 2929 | TOXIC LIQUID, FLAMMABLE, ORGANIC, N.O.S. 60 °C < boiling point < 85 °C |  |  | 6.1 | TF1 |  | I |  | 6.1+3 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 1 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 23; 27; 29 |  |
| 2929 | TOXIC LIQUID, FLAMMABLE, ORGANIC, N.O.S. 85 °C < boiling point < 115 °C |  |  | 6.1 | TF1 |  | I |  | 6.1+3 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  |  |  | 1 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 27; 29 |  |
| 2929 | TOXIC LIQUID, FLAMMABLE, ORGANIC, N.O.S. boiling point > 115 °C |  |  | 6.1 | TF1 |  | I |  | 6.1+3 |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  |  |  | 1 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 27; 29 |  |
| 2929 | TOXIC LIQUID, FLAMMABLE, ORGANIC, N.O.S. boiling point < 60 °C |  |  | 6.1 | TF1 |  | II |  | 6.1+3 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 27; 29 |  |
| 2929 | TOXIC LIQUID, FLAMMABLE, ORGANIC, N.O.S. 60 °C < boiling point < 85 °C |  |  | 6.1 | TF1 |  | II |  | 6.1+3 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 2 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 23; 27; 29 |  |
| 2929 | TOXIC LIQUID, FLAMMABLE, ORGANIC, N.O.S. 85 °C < boiling point < 115 °C |  |  | 6.1 | TF1 |  | II |  | 6.1+3 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  |  |  | 2 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 27; 29 |  |
| 2929 | TOXIC LIQUID, FLAMMABLE, ORGANIC, N.O.S. boiling point > 115 °C |  |  | 6.1 | TF1 |  | II |  | 6.1+3 |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  |  |  | 2 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 27; 29 |  |
| 2935 | ETHYL-2-CHLORO-PROPIONATE |  |  | 3 | F1 |  | III |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 30 |  | 95 |  | 1.08 |  | 2 |  | yes |  | T43) |  | II A |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 2947 | ISOPROPYL CHLOROACETATE |  |  | 3 | F1 |  | III |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 30 |  | 95 |  | 1.09 |  | 2 |  | yes |  | T43) |  | II A |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 2966 | THIOGLYCOL |  |  | 6.1 | T1 |  | II |  | 6.1 |  | C |  | 2 |  | 2 |  | 3 |  | 25 |  | 95 |  | 1.12 |  | 2 |  | no |  |  |  |  |  | no |  | PP,EP, TOX,A |  | 2 |  |  |  |
| 2983 | ETHYLENE OXIDE AND PROPYLENE OXIDE MIXTURE, with not more than 30 % ethylene oxide |  |  | 3 | FT1 |  | I |  | 3+6.1+unst. |  | C |  | 1 |  | 1 |  | 3 |  |  |  | 95 |  | 0.85 |  | 1 |  | no |  | T2 |  | II B |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 2; 3; 12; 31 |  |
| 2984 | HYDROGEN PEROXIDE AQUEOUS SOLUTION with not less than 8%,but less than 20% hydrogen peroxide (stabilized as necessary) |  |  | 5.1 | O1 |  | III |  | 5.1+ unst. |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  | 1.06 |  | 2 |  | yes |  |  |  |  |  | no |  | PP |  | 0 |  | 3; 33 |  |
| 3077 | ENVIRONMENTALLY HAZARDOUS SUBSTANCE, SOLID, N.O.S., molten, (alkylamine (C12 to C18)) |  |  | 9 | M7 |  | III |  | 9 |  | N |  | 4 |  | 3 |  | 2 |  |  |  | 95 |  | 0.79 |  | 3 |  | yes |  |  |  |  |  | no |  | PP |  | 0 |  | 7; 17 |  |
| 3079 | METHACRYLONITRILE, STABILIZED |  |  | 3 | FT1 |  | I |  | 3+6.1+unst. |  | C |  | 2 |  | 2 |  |  |  | 45 |  | 95 |  | 0.8 |  | 1 |  | no |  | T1 |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 3; 5 |  |
| 3082 | ENVIRONMENTALLY HAZARDOUS SUBSTANCE, LIQUID, N.O.S. |  |  | 9 | M6 |  | III |  | 9 |  | N |  | 4 |  | 3 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP |  | 0 |  | 22; 27 |  |
| 3082 | ENVIRONMENTALLY HAZARDOUS SUBSTANCE, LIQUID, N.O.S. ( bilge water) |  |  | 9 | M6 |  | III |  | 9 |  | N |  | 4 |  | 2 |  |  |  |  |  | 97 |  |  |  |  |  | yes |  |  |  |  |  | no |  | PP |  | 0 |  |  |  |
| 3092 | 1-METHOXY-2-PROPANOL |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 0.92 |  | 3 |  | yes |  | T3 |  | II B |  | yes |  | PP, EX, A |  | 0 |  |  |  |
| 3145 | ALKYLPHENOLS, LIQUID, N.O.S. (including C2-C12 homologues) |  |  | 8 | C3 |  | II |  | 8 |  | N |  | 4 |  | 3 |  |  |  |  |  | 97 |  | 0.95 |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 34 |  |
| 3145 | ALKYLPHENOLS, LIQUID, N.O.S. (including C2-C12 homologues) |  |  | 8 | C3 |  | III |  | 8 |  | N |  | 4 |  | 3 |  |  |  |  |  | 97 |  | 0.95 |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 34 |  |
| 3175 | SOLIDS CONTAINING FLAMMABLE LIQUID, N.O.S., molten, having a flash-point up to 60 °C (2- propanol and dialkyl-(C12 to C18)-dimethylammonium chloride) |  |  | 4.1 | F1 |  | II |  | 4.1 |  | N |  | 3 |  | 3 |  | 4 |  |  |  | 95 |  | 0.86 |  | 3 |  | yes |  | T2 |  | II A |  | yes |  | PP, EX, A |  | 1 |  | 7; 17 |  |
| 3256 | ELEVATED TEMPERATURE LIQUID, FLAMMABLE, N.O.S. with flash-point above 60 °C, at or above its flash-point |  |  | 3 | F2 |  | III |  | 3 |  | N |  | 3 |  | 2 |  | 2 |  |  |  | 95 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 0 |  | 7; 27 |  |
| 3256 | ELEVATED TEMPERATURE LIQUID, FLAMMABLE, N.O.S. with flash-point above 60 °C, at or above its flash-point ( carbon black reedstock) ( pyrolysis oil) |  |  | 3 | F2 |  | III |  | 3 |  | N |  | 3 |  | 2 |  | 2 |  |  |  | 95 |  |  |  | 3 |  | yes |  | T1 |  | II B |  | yes |  | PP, EX, A |  | 0 |  | 7 |  |
| 3256 | ELEVATED TEMPERATURE LIQUID, FLAMMABLE, N.O.S. with flash-point above 60 °C, at or above its flash-point ( pyrolysis oil A) |  |  | 3 | F2 |  | III |  | 3 |  | N |  | 3 |  | 2 |  | 2 |  |  |  | 95 |  |  |  | 3 |  | yes |  | T1 |  | II B |  | yes |  | PP, EX, A |  | 0 |  | 7 |  |
| 3256 | ELEVATED TEMPERATURE LIQUID, FLAMMABLE, N.O.S. with flash-point above 60 °C, at or above its flash-point ( residual oil) |  |  | 3 | F2 |  | III |  | 3 |  | N |  | 3 |  | 2 |  | 2 |  |  |  | 95 |  |  |  | 3 |  | yes |  | T1 |  | II B |  | yes |  | PP, EX, A |  | 0 |  | 7 |  |
| 3256 | ELEVATED TEMPERATURE LIQUID, FLAMMABLE, N.O.S. with flash-point above 60 °C, at or above its flash-point ( mixture of crude naphthaline) |  |  | 3 | F2 |  | III |  | 3 |  | N |  | 3 |  | 2 |  | 2 |  |  |  | 95 |  |  |  | 3 |  | yes |  | T1 |  | II B |  | yes |  | PP, EX, A |  | 0 |  | 7 |  |
| 3256 | ELEVATED TEMPERATURE LIQUID, FLAMMABLE, N.O.S. with flash-point above 60 °C, at or above its flash-point ( creosote oil) |  |  | 3 | F2 |  | III |  | 3 |  | N |  | 3 |  | 2 |  | 2 |  |  |  | 95 |  |  |  | 3 |  | yes |  | T2 |  | II B |  | yes |  | PP, EX, A |  | 0 |  | 7 |  |
| 3256 | ELEVATED TEMPERATURE LIQUID, FLAMMABLE, N.O.S. with flash-point above 60 °C, at or above its flash-point (Low QI Pitch) |  |  | 3 | F2 |  | III |  | 3 |  | N |  | 3 |  | 1 |  | 4 |  |  |  | 95 |  | 1.1-1.3 |  | 3 |  | yes |  | T2 |  | II B |  | yes |  | PP, EX, A |  | 0 |  | 7 |  |
| 3257 | ELEVATED TEMPERATURE LIQUID, N.O.S. at or above 100 °C and below its flash-point (including molten metals, molten salts, etc.) |  |  | 9 | M9 |  | III |  | 9 |  | N |  | 4 |  | 1 |  | 4 |  |  |  | 95 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP |  | 0 |  | 7; 20: +115 °C; 22; 24; 25; 27 |  |
| 3257 | ELEVATED TEMPERATURE LIQUID, N.O.S. at or above 100 °C and below its flash-point (including molten metals, molten salts, etc.) |  |  | 9 | M9 |  | III |  | 9 |  | N |  | 4 |  | 1 |  | 4 |  |  |  | 95 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP |  | 0 |  | 7; 20:+225 °C; 22; 24; 27 |  |
| 3259 | AMINES, SOLID, CORROSIVE, N.O.S. ( monoalkyl-(C12 to C18)-amine acetate, MOLTEN) |  |  | 8 | C8 |  | III |  | 8 |  | N |  | 4 |  | 3 |  | 2 |  |  |  | 95 |  | 0.87 |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 7; 17; 34 |  |
| 3264 | CORROSIVE LIQUID, ACIDIC, INORGANIC, N.O.S. |  |  | 8 | C1 |  | I |  | 8 |  | N |  | 2 |  | 3 |  |  |  | 10 |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 27; 34 |  |
| 3264 | CORROSIVE LIQUID, ACIDIC, INORGANIC, N.O.S. |  |  | 8 | C1 |  | II |  | 8 |  | N |  | 2 |  | 3 |  |  |  | 10 |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 27; 34 |  |
| 3264 | CORROSIVE LIQUID, ACIDIC, INORGANIC, N.O.S. |  |  | 8 | C1 |  | III |  | 8 |  | N |  | 4 |  | 3 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 27; 34 |  |
| 3264 | CORROSIVE LIQUID, ACIDIC, INORGANIC, N.O.S. ( aqueous solution of phosphoric acid and citric acid) |  |  | 8 | C1 |  | I |  | 8 |  | N |  | 2 |  | 3 |  |  |  | 10 |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 34 |  |
| 3264 | CORROSIVE LIQUID, ACIDIC, INORGANIC, N.O.S. ( aqueous solution of phosphoric acid and citric acid) |  |  | 8 | C1 |  | II |  | 8 |  | N |  | 4 |  | 3 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 34 |  |
| 3264 | CORROSIVE LIQUID, ACIDIC, INORGANIC, N.O.S. ( aqueous solution of phosphoric acid and citric acid) |  |  | 8 | C1 |  | III |  | 8 |  | N |  | 4 |  | 3 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 34 |  |
| 3265 | CORROSIVE LIQUID, ACIDIC, ORGANIC, N.O.S. |  |  | 8 | C3 |  | I |  | 8 |  | N |  | 2 |  | 3 |  |  |  | 10 |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 27; 34 |  |
| 3265 | CORROSIVE LIQUID, ACIDIC, ORGANIC, N.O.S. |  |  | 8 | C3 |  | II |  | 8 |  | N |  | 2 |  | 3 |  |  |  | 10 |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 27; 34 |  |
| 3265 | CORROSIVE LIQUID, ACIDIC, ORGANIC, N.O.S. |  |  | 8 | C3 |  | III |  | 8 |  | N |  | 4 |  | 3 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 27; 34 |  |
| 3266 | CORROSIVE LIQUID, BASIC, INORGANIC, N.O.S. |  |  | 8 | C5 |  | I |  | 8 |  | N |  | 4 |  | 2 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 27; 34 |  |
| 3266 | CORROSIVE LIQUID, BASIC, INORGANIC, N.O.S. |  |  | 8 | C5 |  | II |  | 8 |  | N |  | 4 |  | 2 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 27; 34 |  |
| 3266 | CORROSIVE LIQUID, BASIC, INORGANIC, N.O.S. |  |  | 8 | C5 |  | III |  | 8 |  | N |  | 4 |  | 2 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 27; 34 |  |
| 3267 | CORROSIVE LIQUID, BASIC, ORGANIC, N.O.S. |  |  | 8 | C7 |  | I |  | 8 |  | N |  | 4 |  | 2 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 27; 34 |  |
| 3267 | CORROSIVE LIQUID, BASIC, ORGANIC, N.O.S. |  |  | 8 | C7 |  | II |  | 8 |  | N |  | 4 |  | 2 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 27; 34 |  |
| 3267 | CORROSIVE LIQUID, BASIC, ORGANIC, N.O.S. |  |  | 8 | C7 |  | III |  | 8 |  | N |  | 4 |  | 2 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP, EP |  | 0 |  | 27; 34 |  |
| 3271 | ETHERS, N.O.S. vp50 < 110 kPa |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14, 27; 29 |  |
| 3271 | ETHERS, N.O.S. (tert- amylmethyl ether) |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.77 |  | 3 |  | yes |  | T2 |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  |  |  |
| 3271 | ETHERS, N.O.S. |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 0 |  | 14, 27 |  |
| 3272 | ESTERS, N.O.S. vp50 < 110 kPa |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.77 |  | 3 |  | yes |  | T2 |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14, 27; 29 |  |
| 3272 | ESTERS, N.O.S. |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 0 |  | 14, 27 |  |
| 3276 | NITRILES, TOXIC, LIQUID, N.O.S. (2- methylglutaronitrile) |  |  | 6.1 | T1 |  | II |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.95 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  |  |  |
| 3286 | FLAMMABLE LIQUID, TOXIC, CORROSIVE, N.O.S. boiling point< 60 °C |  |  | 3 | FTC |  | I |  | 3+6.1+8 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 27; 29 |  |
| 3286 | FLAMMABLE LIQUID, TOXIC, CORROSIVE, N.O.S. boiling point < 60 °C |  |  | 3 | FTC |  | II |  | 3+6.1+8 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 27; 29 |  |
| 3286 | FLAMMABLE LIQUID, TOXIC, CORROSIVE, N.O.S. 60 °C < boiling point < 85 °C |  |  | 3 | FTC |  | II |  | 3+6.1+8 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 2 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 23, 27; 29 |  |
| 3286 | FLAMMABLE LIQUID, TOXIC, CORROSIVE, N.O.S. 85 °C < boiling point < 115 °C |  |  | 3 | FTC |  | II |  | 3+6.1+8 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  |  |  | 2 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 27; 29 |  |
| 3286 | FLAMMABLE LIQUID, TOXIC, CORROSIVE, N.O.S. boiling point > 115 °C |  |  | 3 | FTC |  | II |  | 3+6.1+8 |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  |  |  | 2 |  | no |  | T43) |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 27; 29 |  |
| 3287 | TOXIC LIQUID, INORGANIC, N.O.S. boiling point < 60 °C |  |  | 6.1 | T4 |  | I |  | 6.1 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 27; 29 |  |
| 3287 | TOXIC LIQUID, INORGANIC, N.O.S. 60 °C < boiling point < 85 °C |  |  | 6.1 | T4 |  | I |  | 6.1 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 1 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 23, 27; 29 |  |
| 3287 | TOXIC LIQUID, INORGANIC, N.O.S. 85 °C < boiling point < 115 °C |  |  | 6.1 | T4 |  | I |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  |  |  | 1 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 27; 29 |  |
| 3287 | TOXIC LIQUID, INORGANIC, N.O.S. boiling point > 115 °C |  |  | 6.1 | T4 |  | I |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  |  |  | 1 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 27; 29 |  |
| 3287 | TOXIC LIQUID, INORGANIC, N.O.S. boiling point < 60 °C |  |  | 6.1 | T4 |  | II |  | 6.1 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 27; 29 |  |
| 3287 | TOXIC LIQUID, INORGANIC, N.O.S. 60 °C < boiling point < 85 °C |  |  | 6.1 | T4 |  | II |  | 6.1 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 23, 27; 29 |  |
| 3287 | TOXIC LIQUID, INORGANIC, N.O.S. 85 °C < boiling point < 115 °C |  |  | 6.1 | T4 |  | II |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  |  |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 27; 29 |  |
| 3287 | TOXIC LIQUID, INORGANIC, N.O.S. boiling point > 115 °C |  |  | 6.1 | T4 |  | II |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  |  |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 27; 29 |  |
| 3287 | TOXIC LIQUID, INORGANIC, N.O.S. boiling point < 60 °C |  |  | 6.1 | T4 |  | III |  | 6.1 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 0 |  | 27; 29 |  |
| 3287 | TOXIC LIQUID, INORGANIC, N.O.S. 60 °C < boiling point < 85 °C |  |  | 6.1 | T4 |  | III |  | 6.1 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 0 |  | 23, 27; 29 |  |
| 3287 | TOXIC LIQUID, INORGANIC, N.O.S. 85 °C < boiling point < 115 °C |  |  | 6.1 | T4 |  | III |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  |  |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 0 |  | 27; 29 |  |
| 3287 | TOXIC LIQUID, INORGANIC, N.O.S. boiling point > 115 °C |  |  | 6.1 | T4 |  | III |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  |  |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 0 |  | 27; 29 |  |
| 3287 | TOXIC LIQUID, INORGANIC, N.O.S. ( sodium dichromate solution) |  |  | 6.1 | T4 |  | III |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 30 |  | 95 |  | 1.68 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 0 |  |  |  |
| 3289 | TOXIC LIQUID, CORROSIVE, INORGANIC, N.O.S. boiling point < 60 °C |  |  | 6.1 | TC3 |  | I |  | 6.1+8 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 27; 29 |  |
| 3289 | TOXIC LIQUID, CORROSIVE, INORGANIC, N.O.S. 60 °C < boiling point < 85 °C |  |  | 6.1 | TC3 |  | I |  | 6.1+8 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 23, 27; 29 |  |
| 3289 | TOXIC LIQUID, CORROSIVE, INORGANIC, N.O.S. 85 °C < boiling point < 115 °C |  |  | 6.1 | TC3 |  | I |  | 6.1+8 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  |  |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 27; 29 |  |
| 3289 | TOXIC LIQUID, CORROSIVE, INORGANIC, N.O.S. boiling point > 115 °C |  |  | 6.1 | TC3 |  | I |  | 6.1+8 |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  |  |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 27; 29 |  |
| 3289 | TOXIC LIQUID, CORROSIVE, INORGANIC, N.O.S. boiling point < 60 °C |  |  | 6.1 | TC3 |  | II |  | 6.1+8 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 27; 29 |  |
| 3289 | TOXIC LIQUID, CORROSIVE, INORGANIC, N.O.S. 60 °C < boiling point < 85 °C |  |  | 6.1 | TC3 |  | II |  | 6.1+8 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 23, 27; 29 |  |
| 3289 | TOXIC LIQUID, CORROSIVE, INORGANIC, N.O.S. 85 °C < boiling point < 115 °C |  |  | 6.1 | TC3 |  | II |  | 6.1+8 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  |  |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 27; 29 |  |
| 3289 | TOXIC LIQUID, CORROSIVE, INORGANIC, N.O.S. boiling point > 115 °C |  |  | 6.1 | TC3 |  | II |  | 6.1+8 |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  |  |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 27; 29 |  |
| 3295 | HYDROCARBONS, LIQUID, N.O.S. vp50 > 175 kPa |  |  | 3 | F1 |  | I |  | 3 |  | N |  | 1 |  | 1 |  |  |  |  |  | 97 |  |  |  | 1 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 27; 29 |  |
| 3295 | HYDROCARBONS, LIQUID, N.O.S. vp50 > 175 kPa |  |  | 3 | F1 |  | I |  | 3 |  | N |  | 2 |  | 2 |  | 1 |  | 50 |  | 97 |  |  |  | 1 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 27; 29 |  |
| 3295 | HYDROCARBONS, LIQUID, N.O.S. 110 kPa < vp50 < 175 kPa |  |  | 3 | F1 |  | I |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 50 |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 27; 29 |  |
| 3295 | HYDROCARBONS, LIQUID, N.O.S. 110 kPa < vp50 < 150 kPa |  |  | 3 | F1 |  | I |  | 3 |  | N |  | 2 |  | 2 |  | 3 |  | 10 |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 27; 29 |  |
| 3295 | HYDROCARBONS, LIQUID, N.O.S. 110 kPa < vp50 < 175 kPa |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 50 |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 27; 29 |  |
| 3295 | HYDROCARBONS, LIQUID, N.O.S. 110 kPa < vp50 < 150 kPa |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  | 3 |  | 10 |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 27; 29 |  |
| 3295 | HYDROCARBONS, LIQUID, N.O.S. vp50 ≤ 110 kPa |  |  | 3 | F1 |  | I |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  |  |  | 3 |  | yes |  | T4 3) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 27; 29 |  |
| 3295 | HYDROCARBONS, LIQUID, N.O.S. vp50 < 110 kPa |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14; 27; 29 |  |
| 3295 | HYDROCARBONS, LIQUID, N.O.S. |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 0 |  | 14; 27 |  |
| 3295 | HYDROCARBONS, LIQUID, N.O.S. (1- octen) |  |  | 3 | F1 |  | II |  | 3 |  | N |  | 2 |  | 2 |  |  |  | 10 |  | 97 |  | 0.71 |  | 3 |  | yes |  | T3 |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 14 |  |
| 3295 | HYDROCARBONS, LIQUID, N.O.S. ( polycyclic aromatic hydrocarbons mixture) |  |  | 3 | F1 |  | III |  | 3 |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  | 1.08 |  | 3 |  | yes |  | T1 |  | II A |  | yes |  | PP, EX, A |  | 0 |  | 14 |  |
| 3295 | HYDROCARBONS, LIQUID, N.O.S. with more than 10 % benzene vp50 > 175 kPa |  |  | 3 | F1 |  | I |  | 3 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 27; 29 |  |
| 3295 | HYDROCARBONS, LIQUID, N.O.S. with more than 10 % benzene 110 kPa < vp50 < 175 kPa |  |  | 3 | F1 |  | I |  | 3 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 27; 29 |  |
| 3295 | HYDROCARBONS, LIQUID, N.O.S. with more than 10 % benzene vp50 < 110 kPa boiling point < 60 °C |  |  | 3 | F1 |  | I |  | 3 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | yes |  | T4 3) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 27; 29 |  |
| 3295 | HYDROCARBONS, LIQUID, N.O.S. with more than 10 % benzene vp50 < 110 kPa boiling point < 60 °C |  |  | 3 | F1 |  | I |  | 3 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 2 |  | yes |  | T4 3) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 23; 27; 29 |  |
| 3295 | HYDROCARBONS, LIQUID, N.O.S. with more than 10 % benzene 110 kPa < vp50 < 175 kPa |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 27; 29 |  |
| 3295 | HYDROCARBONS, LIQUID, N.O.S. with more than 10 % benzene vp50 < 110 kPa boiling point < 60 °C |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 27; 29 |  |
| 3295 | HYDROCARBONS, LIQUID, N.O.S. with more than 10 % benzene vp50 < 110 kPa boiling point < 60 °C |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 1 |  | yes |  | T4 3) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 23; 27; 29; 38 |  |
| 3295 | HYDROCARBONS, LIQUID, N.O.S. with more than 10 % benzene vp50 < 110 kPa 60 °C < boiling point < 85 °C |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 2 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 23; 27; 29 |  |
| 3295 | HYDROCARBONS, LIQUID, N.O.S. with more than 10 % benzene vp50 < 110 kPa 85 °C < boiling point < 115 °C |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  |  |  | 2 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 27; 29 |  |
| 3295 | HYDROCARBONS, LIQUID, N.O.S. with more than 10 % benzene vp50 < 110 kPa boiling point > 115 °C |  |  | 3 | F1 |  | II |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  |  |  | 2 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 1 |  | 27; 29 |  |
| 3295 | HYDROCARBONS, LIQUID, N.O.S. with more than 10 % benzene vp50 < 110 kPa 60 °C < boiling point < 85 °C |  |  | 3 | F1 |  | III |  | 3 |  | C |  | 2 |  | 2 |  | 3 |  | 50 |  | 95 |  |  |  | 2 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 0 |  | 23; 27; 29 |  |
| 3295 | HYDROCARBONS, LIQUID, N.O.S. with more than 10 % benzene vp50 < 110 kPa 85 °C < boiling point < 115 °C |  |  | 3 | F1 |  | III |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 50 |  | 95 |  |  |  | 2 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 0 |  | 27; 29 |  |
| 3295 | HYDROCARBONS, LIQUID, N.O.S. with more than 10 % benzene vp50 < 110 kPa boiling point < 115 °C |  |  | 3 | F1 |  | III |  | 3 |  | C |  | 2 |  | 2 |  |  |  | 35 |  | 95 |  |  |  | 2 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 0 |  | 27; 29 |  |
| 3412 | FORMIC ACID with not less than 10% but not more than 85% acid by mass |  |  | 8 | C3 |  | II |  | 8 |  | N |  | 2 |  | 3 |  |  |  | 10 |  | 97 |  | 1.22 |  | 3 |  | yes |  | T1 |  | II A |  | yes |  | PP, EP, EX, A |  | 1 |  | 6: +12 °C; 17; 34 |  |
| 3412 | FORMIC ACID with not less than 5% but less than 10% acid by mass |  |  | 8 | C3 |  | III |  | 8 |  | N |  | 2 |  | 3 |  |  |  | 10 |  | 97 |  | 1.22 |  | 3 |  | yes |  | T1 |  | II A |  | yes |  | PP, EP, EX, A |  | 1 |  | 6: +12 °C; 17; 34 |  |
| 3426 | ACRYLAMIDE, SOLUTION |  |  | 6.1 | T2 |  | III |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 30 |  | 95 |  | 1.03 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 0 |  | 3; 5; 16 |  |
| 3429 | CHLOROTOLUIDINES, LIQUID |  |  | 6.1 | T1 |  | III |  | 6.1 |  | C |  | 2 |  | 2 |  |  |  | 25 |  | 95 |  | 1.15 |  | 2 |  | no |  | T1 |  | II A7) |  | yes |  | PP, EP, EX, TOX, A |  | 0 |  | 6: +6 °C; 17 |  |
| 3446 | NITROTOLUENES, SOLID, MOLTEN (p- nitrotoluene, molten) |  |  | 6.1 | T2 |  | II |  | 6.1 |  | C |  | 2 |  | 2 |  | 2 |  | 25 |  | 95 |  | 1.16 |  | 2 |  | no |  | T2 |  | II B4) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 7; 17 |  |
| 3446 | NITROTOLUENES, SOLID, MOLTEN (p- nitrotoluene, molten) |  |  | 6.1 | T2 |  | II |  | 6.1 |  | C |  | 2 |  | 2 |  | 4 |  | 25 |  | 95 |  | 1.16 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 7; 17; 20: +88 °C |  |
| 3451 | TOLUIDINES. SOLID, MOLTEN (p- toluidine, molten) |  |  | 6.1 | T2 |  | II |  | 6.1 |  | C |  | 2 |  | 2 |  | 2 |  | 25 |  | 95 |  | 1.05 |  | 2 |  | no |  | T1 |  | II A8) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 7; 17 |  |
| 3451 | TOLUIDINES, SOLID (p- toluidine, molten) |  |  | 6.1 | T2 |  | II |  | 6.1 |  | C |  | 2 |  | 2 |  | 4 |  | 25 |  | 95 |  | 1.05 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 7; 17; 20: +60 °C |  |
| 3455 | CRESOLS, SOLID, molten |  |  | 6.1 | TC2 |  | II |  | 6.1+8 |  | C |  | 2 |  | 2 |  | 2 |  | 25 |  | 95 |  | 1.03 - 1.05 |  | 2 |  | no |  | T1 |  | II A8) |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 7; 17 |  |
| 3455 | CRESOLS, SOLID, molten |  |  | 6.1 | TC2 |  | II |  | 6.1+8 |  | C |  | 2 |  | 2 |  | 4 |  | 25 |  | 95 |  | 1.03 - 1.05 |  | 2 |  | no |  |  |  |  |  | no |  | PP, EP, TOX, A |  | 2 |  | 7; 17; 20: +66 °C |  |
| 3463 | PROPIONIC ACID with not less than 90% acid by mass |  |  | 8 | CF1 |  | II |  | 8+3 |  | N |  | 3 |  | 3 |  |  |  |  |  | 97 |  | 0.99 |  | 3 |  | yes |  | T1 |  | II A7) |  | yes |  | PP, EP, EX, A |  | 0 |  | 34 |  |
| 9000 | AMMONIA, ANHYDROUS, DEEPLY refrigerated |  |  | 2 | 3TC |  |  |  | 2.1+2.3+8 |  | G |  | 1 |  | 1 |  | 1; 3 |  |  |  | 95 |  |  |  | 1 |  | yes |  | T1 |  | II A |  | yes |  | PP, EP, EX, TOX, A |  | 2 |  | 1; 31 |  |
| 9001 | SUBSTANCES WITH a flash-point above 60 °C handed over for carriage or carried at a temperature within a range of 15K below theIR flash-point or SUBSTANCES WITH a flash-point >60 °C, , heated TO LESS THAN 15 K FROM THE flash-point |  |  | 3 | F3 |  |  |  |  |  | N |  | 3 |  | 2 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  | T43) |  | II B4) |  | yes |  | PP, EX, A |  | 0 |  | 27 |  |
| 9002 | SUBSTANCES having a self-ignition temperature < 200 °C and not otherwise mentioned |  |  | 3 | F4 |  |  |  | 3 |  | C |  | 1 |  | 1 |  |  |  |  |  | 95 |  |  |  | 1 |  | yes |  | T4 |  | II B4) |  | yes |  | PP,EX,A |  | 0 |  |  |  |
| 9003 | SUBSTANCES with a flash-point above 60 °C but not more than 100 °C or SUBSTANCES WHERE 61° C < flash-point ≤ 100° C, which are not affected to another class |  |  | 9 |  |  |  |  |  |  | N |  | 4 |  | 2 |  |  |  |  |  | 97 |  |  |  | 3 |  | yes |  |  |  |  |  | no |  | PP |  | 0 |  | 27 |  |
| 9003 | SUBSTANCES with a flash-point above 60 °C but not more than 100 °C or SUBSTANCES WHERE 61° C < flash-point < 100 °C, which are not affected to another class (ETHYLENE GLYCOL MONOBUTYL ETHER) |  |  | 9 |  |  |  |  |  |  | N |  | 4 |  | 2 |  |  |  |  |  | 97 |  | 0.9 |  | 3 |  | yes |  |  |  |  |  | no |  | PP |  | 0 |  |  |  |
| 9003 | SUBSTANCES with a flash-point above 60 °C but not more than 100 °C or SUBSTANCES WHERE 61° C < flash-point < 100 °C, which are not affected to another class (2-ethylhexyl-acrylate) |  |  | 9 |  |  |  |  |  |  | N |  | 4 |  | 2 |  |  |  |  |  | 97 |  | 0.89 |  | 3 |  | yes |  |  |  |  |  | no |  | PP |  | 0 |  | 3; 5; 16 |  |
| 9004 | DIPHENYLMETHANE- 4.4’-DIISOCYANATE |  |  | 9 |  |  |  |  |  |  | N |  | 2 |  | 3 |  | 4 |  | 10 |  | 95 |  | 1.2111) |  | 3 |  | yes |  |  |  |  |  | no |  | PP |  | 0 |  | 7; 8; 17; 19 |  |