| UNITED NATIONS |  | E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL ECE/TRANS/WP.29/GRE/2006/20 23 January 2006 Original: FRENCH ENGLISH AND FRENCH ONLY |

ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations (WP.29)

Working Party on Lighting and Light-Signalling (GRE)
(Fifty-sixth session, 4-7 April 2006,
agenda item 4.10.)

# PROPOSAL FOR DRAFT AMENDMENTS TO REGULATION No. 48

## (Installation of lighting and light-signalling devices)

## Transmitted by the expert from France

Note:	The text reproduced below was prepared by the expert from France in order to specify the number of rear end-outline marker lamps which may be fitted to vehicles.  The modifications to the current text of the Regulation are marked in bold.

_____________________________
Note:	This document is distributed to the Experts on Lighting and Light-Signalling only.

GE.06-20314  (E)    080206    100206

A.	PROPOSAL

Paragraph 6.13.2., amend to read:

“6.13.2.	Number

Front:	Two visible from the front

Rear:	Two visible from the front or four visible from the rear when fitted in accordance with paragraph 6.13.4.2.”

Paragraph 6.13.4.2., amend to read:

“6.13.4.2.	In height:

Front:

…

Rear:	At the maximum height compatible with the requirements relating to the width, design and operational requirements of the vehicle and to the symmetry of the lamps.

If two end-outline marker lamps are fitted on the highest part of the vehicle, two additional lamps grouped with the other rear lamps may be fitted.”

###### B.	JUSTIFICATION

This document is intended to improve the fitting of end-outline marker lamps.

Some rear signalling lamps on lorries or trailers group together all the prescribed functions:

Direction indicators, rear position lamps, stop lamps, reversing lamps, rear fog lamps, in some cases side-marker lamps, rear and side retro-reflectors and very often end-outline marker lamps.

In such cases, it is not possible to fit another end-outline marker lamp on the highest part of the vehicle, and this has a serious impact on safety, since it is the highest end-outline marker lamp which is most visible.

The proposed amendment will make it possible to leave the end-outline marker lamp grouped with the other lamps on the lower part of the vehicle and to fit another end-outline marker lamp on the highest part of the vehicle.

-----