| ## UNITED NATIONS |  | # E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL ECE/TRANS/WP.29/GRE/2006/27 23 January 2006 Original: ENGLISH ENGLISH AND FRENCH ONLY |

ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations (WP.29)

Working Party on Lighting and Light-Signalling (GRE)

(Fifty-sixth session, 4-7 April 2006,

agenda item 4.4.)

PROPOSAL FOR DRAFT AMENDMENTS TO REGULATION No. 48

(Installation of lighting and light-signalling devices)

Transmitted by the experts from the Working Party "Brussels 1952" (GTB)

and the International Organization of Motor Vehicle Manufacturers (OICA)

Note:	  The text reproduced below was prepared as a consolidated proposal by the experts from GTB and OICA in order to propose conditions for tell-tale activation in a case of failure of an individual LED.  The document is prepared upon the GRE request at its fifty-fourth session (TRANS/WP.29/GRE/54, para. 20) and fifty-fifth session (TRANS/WP.29/GRE/55, para. 14).  The modifications to the current text of the Regulation (up to Supplement 13 to the 02 series of amendments) are marked in bold characters.

This proposal should be considered in conjunction with ECE/TRANS/WP.29/GRE/2006/26 under agenda item 7.

A.	PROPOSAL

Paragraph 6.5.8., amend to read:

"6.5.8.	Tell-tale

Operating tell-tale mandatory for ..... or rear direction-indicator lamps.

It shall be activated either:

-	in case of a failure of any one of the light sources incorporated in such direction indicator lamps, or

-	when any such direction indicator lamp, as a consequence of a failure of one or more of such light sources, ceases to meet at least the minimum luminous intensity required in the axis of reference as described in the communication form according to Regulation No. 6.

The method of activation shall be stated in the communication according to Annex 1 to this Regulation.

If a motor vehicle is equipped to draw ...... "

Annex 1,

Insert a new item 10.3., to read:

"10.3.	Method used for activation of the direction indicator tell-tale (paragraph 6.5.8.)"

Item 10.3. (former), renumber as item 10.4.

B.	JUSTIFICATION

At its fifty-second and fifty-third sessions, GRE considered informal document No. GRE-52-23, submitted by the expert from the Netherlands, which addressed the problems of applying the provisions of paragraph 6.5.8. of Regulation No. 48 in the case of direction indicators equipped with multiple light sources, e.g. LED's.  At its fifty-fourth and fifty-fifth sessions, GRE invited GTB and OICA to prepare a consolidated proposal.

At the time when the provisions of paragraph 6.5.8. were drafted, filament lamps (mostly one 21W lamp in each device) were used as light sources for front and rear direction indicator lamps, and activation of the tell-tale by current control was no problem.  At the present time, more and more new vehicle types are equipped with direction indicator lamps using LED's in multiple light source arrangements.  The different layouts of such LED assemblies and the consequences regarding failure mode strategies have been carefully discussed by GTB and OICA experts.  The present proposal takes into account the aspects of safety and the available means of tell-tale activation in case of individual LED failure.  Depending on the decisions by GRE, transitional provisions will have to be added to the proposal, using the guidelines adopted by AC.1 (TRANS/WP.29/1044).

- - - - -