| UNITED NATIONS |  | E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL ECE/TRANS/WP.29/GRE/2006/52 21 July 2006 Original: ENGLISH ENGLISH AND FRENCH ONLY |

### ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations (WP.29)

Working Party on Lighting and Light-Signalling (GRE)

Fifty-seventh session

2-6 October 2006

Item 4.10. of the provisional agenda

PROPOSAL FOR DRAFT AMENDMENTS TO REGULATION No. 48

(Installation of lighting and light-signalling devices)

Submitted by the expert from the International Organization of

Motor Vehicle Manufacturers (OICA)

Note:	The text reproduced below was prepared by the expert from OICA in order to allow red colour for the rearmost side retro-reflector if it is grouped with a rear retro-reflector, non triangular.  The modifications to the current text of the Regulation (up to Supplement 13 to the 02 series of amendments) are marked in bold or strikethrough characters.

A.	PROPOSAL

Paragraph 5.15., amend to read:

"5.15.	The colours of the light emitted by the lamps are the following:

.…

| side retro-reflector, non-triangular: | amber; however the rearmost side retro-reflector can be red if it is grouped or has part of the light emitting surface in common with the rear position lamp, the rear end outline marker lamp, the rear fog lamp, the stop-lamp, or the red rearmost side-marker lamp or the rear retro-reflector, non triangular. |
| --- | --- |

.… "

B.	JUSTIFICATION

The rearmost side retro-reflector can be red if it is grouped or has a part of the light-emitting surface with another rear lamp.  This results in location of the rearmost side retro-reflector on the very end of a vehicle.  It can be regarded as similar to the situation where the rearmost side retro-reflector is grouped with the rear retro-reflector and, therefore, it may be red in this case as well.

- - - - -