| UNITED NATIONS |  | E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL ECE/TRANS/WP.29/GRE/2006/56 21 July 2006 Original: ENGLISH ENGLISH AND FRENCH ONLY |

### ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations (WP.29)

Working Party on Lighting and Light-Signalling (GRE)

Fifty-seventh session

2-6 October 2006

Item 7. of the provisional agenda

PROPOSAL FOR DRAFT AMENDMENTS TO REGULATION No. 48

(Installation of lighting and light-signalling devices)

Submitted by the experts from the Working Party "Brussels 1952" (GTB) and

the International Organization of Motor Vehicle Manufacturers (OICA)

Note:	The text reproduced below was prepared by the experts from GTB and OICA in order to insert into the Regulation provisions regarding direction indicator tell-tale activation.  The proposal consolidates and supersedes ECE/TRANS/WP.29/GRE/2006/27 as well as complements the corresponding amendments to Regulation No. 6 proposed by GTB and OICA (ECE/TRANS/WP.29/GRE/2006/55).  The modifications to the current text of the Regulation (up to Supplement 13 to the 02 series of amendments) are marked in bold characters.

The issue of transitional provisions has not yet been discussed in GTB and OICA and will be addressed, if necessary, separately at a later date.

A.	PROPOSAL

Paragraph 6.5.8., amend to read:

"6.5.8.	Tell-tale

Operating tell-tale mandatory for direction indicator lamps of categories 1, 1a, 1b, 2a and 2b.  It may be visual or auditory or both.  If it is visual it shall be a flashing light which, at least in the event of the malfunction of any of these direction indicator lamps, is either extinguished, or remains alight without flashing, or shows a marked change of frequency.  If it is entirely auditory it shall be clearly audible and shall show a marked change of frequency, at least in the event of the malfunction of any of these direction indicator lamps.

It shall be activated by the signal produced according to paragraph 6.4.3. of Regulation No. 6.

If a motor vehicle is equipped .… "

B.	JUSTIFICATION

This proposal complements the proposal by GTB and OICA for amendments to Regulation No. 6, which takes into account the comments made by several Contracting Parties at the fifty-sixth GRE session regarding front/rear direction indicator tell-tale activation in a case of light source failure.

Depending on the decision by GRE, transitional provisions will be added to the proposal, using the guidelines adopted by the Administrative Committee (AC.1) (TRANS/WP.29/1044).

- - - - -