RDW TEST CENTRUM Lelystad

TEST PLAN MCGTR	MEETBLAD nr 1

| Driver | 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Place | Lelystad |  |  |  |  |
| Vehicle make/type | Triumph |  |  |  |  |
| VIN | ND03CTI.5?? |  |  |  |  |
| Tires; tyre pressure | front: Bridgestone Battax B1020 120/70ZR17 M/C 58W; press. 2,5 bar rear: Bridgestone Battax B1020 180/55ZR17 M/C 78W; press. 2,9 bar |  |  |  |  |
| Test weights | Front 158 kg Rear 216 kg |  |  |  |  |
| Date | 10-05-2006 |  |  |  |  |
| Road-surface | Asphalt/basalt |  | Weather conditions | Dry |  |
| Wind force | 3 | m/s | Wind direction | E-NE |  |
| Barometric pressure | 1019 | mbar | Temperature | 18 | ˚C |
| Humidity | 52 | % | Remarks | - |  |

4.9.3 Stops on high friction surface (Vehicle lightly loaded )

|  | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Actual speed [km/h] | 58,0 | 60,2 | 61,2 | 59,9 | 61,6 | 60,9 |
| Deceleration [m/sec2] | 9,0 | 8,9 | 8,7 | 8,8 | 8,6 | 9,0 |
| Brake distance [m] | 17,0 | 18,3 | 19,1 | 18,3 | 19,5 | 19,0 |
| Controlforce front [daN] | 23 | 22,5 | 25 | 22 | 22 | 21 |
| Controlforce rear [daN] | 31 | 34 | 30 | 30 | 35 | 35 |
| t 40 – 20 [s] | 0.5500 | 0.6100 | 0.6600 | 0.6300 | 0.6100 | 0.5700 |
| Z | 1.0182 | 0.9180 | 0.8485 | 0.8889 | 0.9180 | 0.9825 |
| Diagram number |  |  |  |  |  |  |
| Document name: Bhoog - | 059 | 061 | 064 | 065 | 066 | 067 |

4.9.4. Stops on low friction  (Vehicle lightly loaded )

|  | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Actual speed [km/h] | 59,1 | 58,3 | 60,7 | 59,8 | 59,7 | 57,7 |
| Deceleration [m/sec2] | 2,19 | 2,18 | 2,22 | 2,10 | 2,31 | 2,11 |
| Brake distance [m] | 61,9 | 62,5 | 68,9 | 67,0 | 63,6 | 64,3 |
| Controlforce front [daN] | 20 | 19 | 18 | 20 | 19 | 11 |
| Controlforce rear [daN] | 38,5 | 33 | 33 | 34 | 40 | 39 |
| t 40 – 20 [s] | 2.5600 | 2.6500 | 2.4900 | 2.6600 | 2.3100 | 2.6400 |
| Z | 0.2188 | 0.2113 | 0.2249 | 0.2105 | 0.2424 | 0.2121 |
| Diagram number |  |  |  |  |  |  |
| Document name: b laag - | 013 | 023 | 025 | 027 | 029 | 046 |

4.9.7. Wheel lock check –low to hight surface transition  (Vehicle lightly loaded )

|  | 1 | 2 | 3 |
| --- | --- | --- | --- |
| Actual speed [km/h] | 55,3 | 57,3 | 58,0 |
| Speed low-high [km/h] | 46,3 | 52,8 | 48,9 |
| Response time transition | 0.22 | 0.23 | 0.30 |
| Controlforce front [daN] | 19 | 15 | 22 |
| Controlforce rear [daN] | 37 | 36 | 39 |
| Diagram number |  |  |  |
| Document name: Bovergang - | 016 | 017 | 018 |

RDW TEST CENTRUM Lelystad

TEST PLAN MCGTR	MEETBLAD nr 1

4. K measurement test – ECE R78 ABS Annex 4

Test results Wet Low Friction Surface

| Vstart [km/h] | Control force front [daN] | Control force rear [daN] | t40-20 [s] | Z | diagram number |
| --- | --- | --- | --- | --- | --- |
| 52,0 | 3,5 | 3,5 | 2,56 | 0.219 | BKlaag003 |
| 53,5 | 2,5 | 3,5 | 3,24 | 0.173 | BKlaag 006 |
| 51,7 | 2,0 | 4,0 | 2,71 | 0.207 | BKlaag 010 |
| 53,3 | 2,5 | 5,0 | 2,39 | 0.234 | BKlaag 012 |
| 57,8 | 2,0 | 4,0 | 2,37 | 0.236 | BKlaag 013 |
| 54,4 | 2,0 | 4,0 | 2,41 | 0.232 | BKlaag 015 |
| 52,0 | 2,5 | 4,0 | 2,07 | 0.271 | BKlaag 017 |

Test results Wet High Friction Surface

| Vstart [km/h] | Control force front [daN] | Control force rear [daN] | t40-20 [s] | Z | diagram number |
| --- | --- | --- | --- | --- | --- |
| 55,7 | 12 | 9 | 0,60 | 0.933 | BKhoog 011 |
| 60,1 | 10 | 7 | 0,64 | 0.875 | BKhoog 014 |
| 58,9 | 9,5 | 7 | 0,63 | 0.889 | BKhoog 018 |
| 55,2 | 11 | 4 | 0,63 | 0.889 | BKhoog 020 |
| 57,9 | 10 | 6 | 0,64 | 0.875 | BKhoog 021 |

Test results Dry High Friction Surface

| Vstart [km/h] | Control force front [daN] | Control force rear [daN] | t40-20 [s] | Z | diagram number |
| --- | --- | --- | --- | --- | --- |