RDW TEST CENTRUM Lelystad

TEST PLAN MCGTR	MEETBLAD nr 3

| Driver | 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Place | Lelystad |  |  |  |  |
| Vehicle make/type | Triumph |  |  |  |  |
| VIN | ND03CTI.5?? |  |  |  |  |
| Tires; tyre pressure | front: Bridgestone Battax B1020 120/70ZR17 M/C 58W; press. 2,5 bar rear: Bridgestone Battax B1020 180/55ZR17 M/C 78W; press. 2,9 bar |  |  |  |  |
| Test weights | Front 155 kg Rear 213 kg |  |  |  |  |
| Date | 15-05-2006 |  |  |  |  |
| Road-surface | Asphalt/basalt |  | Weather conditions | Dry |  |
| Wind force | 2 | m/s | Wind direction | E |  |
| Barometric pressure | 1016 | mbar | Temperature | 18 | ˚C |
| Humidity | 55 | % | Remarks | - |  |

4.9.3 Stops on high friction surface (Vehicle lightly loaded )

|  | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Actual speed [km/h] | 62,8 | 60,2 | 62,5 | 61,7 | 62,1 | 59,6 |
| Deceleration [m/sec2] | 8,91 | 9,09 | 8,96 | 8,90 | 8,91 | 8,79 |
| Brake distance [m] | 20,23 | 19,31 | 19,96 | 20,00 | 20,94 | 18,42 |
| Controlforce front [daN] | 13 | 14 | 14 | 16 | 17 | 15 |
| Controlforce rear [daN] | 20 | 20 | 23 | 22 | 25 | 30 |
| t 40 – 20 [s] | 0.6100 | 0.5900 | 0.5700 | 0.5900 | 0.5800 | 0.5900 |
| Z | 0.9180 | 0.9492 | 0.9825 | 0.9492 | 0.9655 | 0.9492 |
| Diagram number |  |  |  |  |  |  |
| Document name: F E hoog _ - | 004 | 006 | 007 | 008 | 009 | 010 |

4.9.4. Stops on low friction  (Vehicle lightly loaded )

|  | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Actual speed [km/h] | 65,4 | 59,9 | 57,0 | 66,5 | 62,6 | 63,9 |
| Deceleration [m/sec2] | 2,39 | 2,32 | 2,29 | 2,39 | 2,57 | 2,18 |
| Brake distance [m] | 73.61 | 63,14 | 58,46 | 81,62 | 66,68 | 77,76 |
| Controlforce front [daN] | 20 | 20,5 | 16 | 18 | 18 | 12,4 |
| Controlforce rear [daN] | 30 | 33 | 30 | 30 | 28 | 29 |
| t 40 – 20 [s] | 2.1700 | 2.2600 | 2.3800 | 2.4000 | 2.1600 | 2.6000 |
| Z | 0.2581 | 0.2478 | 0.2353 | 0.2333 | 0.2593 | 0.2154 |
| Diagram number |  |  |  |  |  |  |
| Document name: F E laag _ - | 010 | 012 | 014 | 016 | 018 | 020 |

4.9.7. Wheel lock check –low to hight surface transition  (Vehicle lightly loaded )

|  | 1 | 2 | 3 |
| --- | --- | --- | --- |
| Actual speed [km/h] | 62,3 | 60,8 | 61,0 |
| Speed low-high [km/h] | 50,3 | 48,7 | 50,0 |
| Response time transition | 0.15 | 0.18 | 0.30 |
| Controlforce front [daN] | 22 | 20 | 20 |
| Controlforce rear [daN] | 30 | 31 | 29 |
| Diagram number |  |  |  |
| Document name: F overgang _ - | 004 | 016 | 018 |

RDW TEST CENTRUM Lelystad

TEST PLAN MCGTR	MEETBLAD nr 3

4. K measurement test – ECE R78 ABS Annex 4

Test results Wet Low Friction Surface

| Vstart [km/h] | Control force front [daN] | Control force rear [daN] | t40-20 [s] | Z | diagram number |
| --- | --- | --- | --- | --- | --- |
| 43,7 | 2 | 2,5 | 2,67 | 0.210 | F K laag _ 008 |
| 46,5 | 1 | 3,5 | 2,47 | 0.227 | F K laag _ 021 |
| 45,0 | 2,5 | 3,5 | 2,51 | 0.223 | F K laag _ 023 |
| 49,0 | 1,5 | 4 | 2,29 | 0.245 | F K laag _ 024 |
| 47,6 | 1 | 3 | 2,39 | 0.234 | F K laag _ 028 |

Test results Wet High Friction Surface

| Vstart [km/h] | Control force front [daN] | Control force rear [daN] | t40-20 [s] | Z | diagram number |
| --- | --- | --- | --- | --- | --- |
| 45,6 | 7,5 | 11 | 0,68 | 0.824 | F K hoog _ 004 |
| 52,7 | 6,5 | 9 | 0,67 | 0.836 | F K hoog _ 005 |
| 62,5 | 7 | 6,5 | 0,64 | 0.875 | F K hoog _ 016 |
| 64,4 | 8 | 8 | 0,72 | 0.778 | F K hoog _ 023 |
| 66,5 | 8,5 | 2 | 0,70 | 0.800 | F K hoog _ 026 |
| 58,9 | 10 | 3 | 0,67 | 0.836 | F K hoog _ 027 |

Test results Dry High Friction Surface

| Vstart [km/h] | Control force front [daN] | Control force rear [daN] | t40-20 [s] | Z | diagram number |
| --- | --- | --- | --- | --- | --- |