RDW TEST CENTRUM Lelystad

TEST PLAN MCGTR	MEETBLAD nr 4

| Driver | 4 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Place | Lelystad |  |  |  |  |
| Vehicle make/type | Triumph |  |  |  |  |
| VIN | ND03CTI.5?? |  |  |  |  |
| Tires; tyre pressure | front: Bridgestone Battax B1020 120/70ZR17 M/C 58W; press. 2,5 bar rear: Bridgestone Battax B1020 180/55ZR17 M/C 78W; press. 2,9 bar |  |  |  |  |
| Test weights | Front 153 kg Rear 215 kg |  |  |  |  |
| Date | 16-05-2006 |  |  |  |  |
| Road-surface | Asphalt/basalt |  | Weather conditions | Dry |  |
| Wind force | 4 | m/s | Wind direction | W-SW |  |
| Barometric pressure | 1015 | mbar | Temperature | 16 | ˚C |
| Humidity | 72 | % | Remarks | - |  |

4.9.3 Stops on high friction surface (Vehicle lightly loaded )

|  | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Actual speed [km/h] | 59,6 | 60,3 | 61,0 | 61,8 | 62,5 | 63,4 |
| Deceleration [m/sec2] | 9,26 | 9,06 | 9,41 | 9,23 | 9,01 | 8,86 |
| Brake distance [m] | 18,40 | 18,29 | 18,68 | 19,23 | 19,97 | 19,41 |
| Controlforce front [daN] | 21 | 22,5 | 22,5 | 24 | 24 | 25 |
| Controlforce rear [daN] | 26 | 35 | 34 | 32 | 34 | 37 |
| t 40 – 20 [s] | 0.5700 | 0.5900 | 0.5700 | 0.5700 | 0.6000 | 0.6700 |
| Z | 0.9825 | 0.9492 | 0.9825 | 0.9825 | 0.9333 | 0.8358 |
| Diagram number |  |  |  |  |  |  |
| Document name: Z E hoog _ - | 004 | 011 | 012 | 013 | 021 | 022 |

4.9.4. Stops on low friction  (Vehicle lightly loaded )

|  | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Actual speed [km/h] | 55,6 | 58,8 | 59,1 | 62,7 | 61,2 | 63,0 |
| Deceleration [m/sec2] | 2,03 | 2,03 | 2,01 | 2,12 | 2,16 | 1,97 |
| Brake distance [m] | 67,7 | 68,23 | 71,80 | 78,83 | 71,69 | 79,28 |
| Controlforce front [daN] | 22,5 | 23,0 | 23,0 | 23,5 | 23 | 21 |
| Controlforce rear [daN] | 32,5 | 30,5 | 37 | 37 | 35 | 33 |
| t 40 – 20 [s] | 2.7500 | 2.8400 | 2.8800 | 2.5100 | 2.4900 | 2.7600 |
| Z | 0.2036 | 0.1972 | 0.1944 | 0.2231 | 0.2249 | 0.2029 |
| Diagram number |  |  |  |  |  |  |
| Document name: Z E laag _ - | 006 | 010 | 012 | 014 | 016 | 018 |

4.9.7. Wheel lock check –low to hight surface transition  (Vehicle lightly loaded )

|  | 1 | 2 | 3 |
| --- | --- | --- | --- |
| Actual speed [km/h] | 55,2 | 57,3 | 59,0 |
| Speed low-high [km/h] | 46,7 | 52,7 | 54,2 |
| Response time transition | 0.20 | 0.19 | 0.23 |
| Controlforce front [daN] | 23,5 | 21,5 | 22,5 |
| Controlforce rear [daN] | 37 | 47 | 43,5 |
| Diagram number |  |  |  |
| Document name: Z overgang _ - | 005 | 006 | 007 |

RDW TEST CENTRUM Lelystad

TEST PLAN MCGTR	MEETBLAD nr 4

4. K measurement test – ECE R78 ABS Annex 4

Test results Wet Low Friction Surface

| Vstart [km/h] | Control force front [daN] | Control force rear [daN] | t40-20 [s] | Z | diagram number |
| --- | --- | --- | --- | --- | --- |
| 58,4 | 2,5 | 7 | 2,21 | 0.253 | Z K laag _ 010 |
| 56,7 | 2 | 6 | 2,51 | 0.223 | Z K laag _ 015 |
| 59,6 | 2 | 6 | 2,35 | 0.238 | Z K laag _ 021 |
| 57,0 | 2 | 5,5 | 2,49 | 0.225 | Z K laag _ 024 |
| 58,2 | 2 | 6,5 | 2,26 | 0.248 | Z K laag _ 026 |
| 58,3 | 2 | 5,5 | 2,36 | 0.237 | Z K laag _ 029 |

Test results Wet High Friction Surface

| Vstart [km/h] | Control force front [daN] | Control force rear [daN] | t40-20 [s] | Z | diagram number |
| --- | --- | --- | --- | --- | --- |
| 61,8 | 9 | 11,5 | 0,69 | 0.812 | Z K hoog _ 004 |
| 62,6 | 9,5 | 11 | 0,69 | 0.812 | Z K hoog _ 005 |
| 58,2 | 10,5 | 7,5 | 0,71 | 0.789 | Z K hoog _ 010 |
| 59,8 | 11 | 6 | 0,68 | 0.824 | Z K hoog _ 011 |
| 61,5 | 11,5 | 7,5 | 0,65 | 0.862 | Z K hoog _ 013 |

Test results Dry High Friction Surface

| Vstart [km/h] | Control force front [daN] | Control force rear [daN] | t40-20 [s] | Z | diagram number |
| --- | --- | --- | --- | --- | --- |