| Transmitted by the expert from OICA | Informal Document No. GRRF-59-23 (59th GRRF, 30 January - 3 February 2006 agenda item 1.1.) |
| --- | --- |

A.	PROPOSAL

#### Insert new paragraphs 2.32. and 2.33., to read:

"2.32.	Braking signal: logic signal indicating brake activation as specified in paragraph 5.2.1.30.

2.33.	Emergency braking signal: logic signal indicating emergency braking as specified in paragraph 5.2.1.31."

Paragraph 5.2.1.30. (see TRANS/WP.29/2004/38), amend to read:

"5.2.1.30.	Generation of a braking signal to illuminate stop lamps."

Insert new paragraphs 5.2.1.31. to 5.2.1.31.2.(b), to read:

"5.2.1.31.	When a vehicle is equipped with the means to indicate emergency braking, the emergency braking signal shall be activationed and de-activationed of the emergency braking signal shall meet according to the specifications below:

5.2.1.31.1.	The signal shall be activated by the application of the service braking system as follows:

|  | Shall not be activated below |
| --- | --- |
| M1 and N1 | 6 7 m/s2 1/ |
| M2, M3, N2 and N3 | 4 5 m/s2 1/ |

The signal shall be de-activated for all vehicles at the latest when the deceleration is has fallen below 2.5 m/s2.

5.2.1.31.2.	The following conditions may also be used:

(a)	The signal may be activated by the application of the service braking system in such a manner that it would produce, in an unladen condition and engine disconnected, under the test conditions of Type-0 as described in Annex 4, a deceleration of  as follows:

|  | Shall not be activated below |
| --- | --- |
| M1 and N1 | 6 7 m/s2 1/ |
| M2, M3, N2 and N3 | 4 5 m/s2 1/ |

The signal shall be de-activated for all vehicles at the latest when the deceleration is has fallen below 2.5 m/s2.

(b)	The signal may be activated when:

-	the service braking system is applied and;

-	the antilock system is full cycling (as defined in paragraph 2. of Annex 13) and;

-	the initial speed is above 50 km/h."

Insert a new footnote 1/, to read:

"1/  When measuring the deceleration, a tolerance of ±1.25 m/s2 will be applied."

- - - - -