| Transmitted by the expert from OICA (the revised text is highlighted in yellow) | Informal Document No. GRRF-59-23-Rev.1 (59th GRRF, 30 January - 3 February 2006 agenda item 1.1.) |
| --- | --- |

A.	PROPOSAL

#### Insert new paragraphs 2.32. and 2.33., to read:

"2.32.	Braking signal: logic signal indicating brake activation as specified in paragraph 5.2.1.30.

2.33.	Emergency braking signal: logic signal indicating emergency braking as specified in paragraph 5.2.1.31."

Paragraph 5.2.1.30., amend to read:

"5.2.1.30.	Generation of a braking signal to illuminate stop lamps."

Insert new paragraphs 5.2.1.31. to 5.2.1.31.2.(b), to read:

"5.2.1.31.	When a vehicle is equipped with the means to indicate emergency braking, activation and de-activation of the emergency braking signal shall meet the specifications below:

5.2.1.31.1.	The signal shall be activated by the application of the service braking system as follows:

|  | Shall not be activated below |
| --- | --- |
| M1 and N1 | 6 m/s2 |
| M2, M3, N2 and N3 | 4 m/s2 |

The signal shall be de-activated for all vehicles at the latest when the deceleration has fallen below 2.5 m/s2.

5.2.1.31.2.	The following conditions may also be used:

(a)	The signal may be activated by the application of the service braking system in such a manner that it would produce, in an unladen condition and engine disconnected, under the test conditions of Type-0 as described in Annex 4, a deceleration as follows:

|  | Shall not be activated below |
| --- | --- |
| M1 and N1 | 6 m/s2 |
| M2, M3, N2 and N3 | 4 m/s2 |

The signal shall be de-activated for all vehicles at the latest when the deceleration has fallen below 2.5 m/s2.

or

(b)	The signal may be activated when the service braking system is applied at a speed above 50 km/h and the antilock system is fully cycling (as defined in paragraph 2. of Annex 13)

The signal shall be deactivated when the antilock system is no longer fully cycling."

- - - - -