Bus fires in city buses

- General information

  - The information were collected from Budapest Transport Company (BKV) and Budapest Fire Department (BTP)

  - The bus fleet of BKV consists of 1400 units, slowly decreasing in years.

  - Between 2002-2005 there were 19 fires in BKV buses, the observations are based on these cases. (The fires occurred in 5 different bus types)

- BKV’s rule for the driver in case of bus fire

  - Stop the bus immediately in a safe way

  - Open the doors

  - Switch off the electricity

  - Help the passengers to evacuate the bus

  - Call the BKV centre and BTP

  - Try to extinguish the fire

- Some information about 19 BKV bus fires

3.1. Who observed, recognized the fire?	3.2.  Cause of the fire

bus driver	6		electric	4

passenger	1 	fuel leaking	7

outside person	1	oil contamination	2

no information	 11	no information	6

3.3. Location of the fire			3.4. Way of extinguishing the fire

engine compartment	 12	bus driver, with extinguisher  8

out of engine compartment	5		fire brigade		8

no information	2		completely burned out	3

3.5.	Bus operation mode	3.6.  Casualties

Empty bus running to or 	No casualties in the 19 fire cases. Being in

from the garage	5 	service (11) the driver stopped the bus,

Waiting at the end station		opened the doors and the passengers could

with running engine	3	evacuate in 1-2 minutes.

Being in normal service	 11

- Further information from BTP

BTP is called not only to city bus fires, but all kind of bus fires in Budapest and its metropolitan area. Some experiences from their practice:

  - In the early stage of the fire (5-8 min.) the bus driver generally can extinguish it.

  - If the fire can develop in the passenger compartment, the driver can not extinguish it anymore with the standard fire-extinguisher.

  - In the city the fire-bridge needs – as an average – 15-20 minutes (from the alarm) to start the extinguishing. Outside the city this time period could be 30-40 minutes.

  - Essential difference between city buses and coaches: the city buses do not have tire-fire (low speed, many stops, smaller distances between the end stations, where the driver can check the tires).

- BTP proposals

On the question: “What could you propose to decrease the fire risk in buses?” the experts of BTP had the following ideas:

- more specific requirements for fixing the electric wiring and fuel lining in buses, especially where the bus parts, components can have relative displacement to each other;

- specific requirement for approved fire-wall between the passenger compartment and engine,  -heating device compartments and rear wheel arches (coaches);

- approved fire indicators (smoke or temperature sensors) in the engine, -heating device compartments. Periodical checks shall be required for these sensors, similarly to the fire-extinguishers;

- better specification is needed for the permissible flammability of materials used in buses;

- in the engine compartment a central fire-extinguisher (having more injectors) could be useful, which is driver controlled and operated from outside, without opening the door of the engine compartment.

- - - - -