WP.11/2007/63rd/INF.6

ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

Working Party on the Transport of Perishable Foodstuffs
Sixty-third session

Geneva, 12-15 November 2007

- Item 5 (c) of the provisional agenda

Correction and up-date of document 2007/10

Submitted by the Netherlands

During national preliminary discussions it was discovered that in the preparation of document 2007/10 the times allowed for classes A,D and B,E were not correctly represented.

The table with corrected times reproduced below replaces the table in document 2007/10

| Outside temperature | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | °C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class A, D | 240 | 229 | 219 | 208 | 197 | 187 | 176 | 165 | 155 | 144 | 133 | 123 | 112 | 101 | 91 | 80 | min |
| Class B, E | 300 | 291 | 281 | 272 | 263 | 253 | 244 | 235 | 225 | 216 | 207 | 197 | 188 | 179 | 169 | 160 | min |
| Class C, F | 360 | 352 | 344 | 336 | 328 | 320 | 312 | 304 | 296 | 288 | 280 | 272 | 264 | 256 | 248 | 240 | min |

Since the original drafting of document 2007/10 more data of pull-down tests have become available. The table on the next page gives results of the tests and replaces the table in Annex 1 of document 2007/10.

The conclusion of the tests is that following the test times proposed in document 2007/10, 13% would have been rejected. Based on the test times in document 2007/17, 34 % would have been rejected.

| Certificate no. | Vehicle type | Brand body | coëff K | Year of construction | Brand appl. | Refrigerant | year of Construction | T Outside | T Class | Actual test time | safety factor nw | date of test |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5102-017 | semi tr | Luchtenberg | 0,366 | 07-1998 | TK SMX II TCI | R 404a | 12-1997 | 17 | - 20 | 3h45 | 2,99 | 15-03-07 |
| 5102-020 | semi tr | Luchtenberg | 0,366 | 06-1998 | TK SMX II TCI | R 404a | 03-1998 | 16 | - 20 | 4h45 | 2,99 | 15-03-07 |
| 5102-018 | semi tr | Luchtenberg | 0,366 | 04-1998 | TK SMX II TCI | R 404a | 02-1998 | 16 | - 20 | 3h00 | 2,99 | 15-03-07 |
| 5102-021 | semi tr | Luchtenberg | 0,366 | 05-1998 | TK SMX II TCI | R 404a | 02-1998 | 16 | - 20 | 3h45 | 2,99 | 15-03-07 |
| T2710-139 | semi tr | VéDéCar | 0,322 | 1997 | TK SL-200e | R 404a | 2006 | 17 | - 20 | 3h00 | 3,05 | 26-03-07 |
| 2080-317 | semi tr | Norfrig | 0,34 | 1997 | TK SMX II | R 404a | 06-1997 | 14 | - 20 | 2h15 | 2,85 | 3-04-07 |
| 3110-093 | semi tr | Schmitz | 0,397 | 1998 | Carr Maxima | R 404a | 1998 | 28 | - 20 | 3h45 | 2,14 | 25-04-07 |
| 5130-057 | semi tr | Unitrans | 0,352 | 01-1998 | TK SB III SLE | R 404a | 01-1995 | 16 | - 20 | 2h30 | 3,68 | 8-05-07 |
| 5130-058 | semi tr | Unitrans | 0,352 | 02-1998 | TK SB I | R12/ R403b | 04-1989 | 16 | - 20 | 3h30 | 2,33 | 8-05-07 |
| 3167-001 | semi tr | Schmitz | 0,36 | 1998 | Carr Ultra | R 22 | 1998 | 21 | - 20 | 2h45 | 3,63 | 18-05-07 |
| 3143-037 | semi tr | Schmitz | 0,399 | 1999 | Carr Maxima 2 | R 404a | 1998 | 19 | - 20 | 2h30 | 2,27 | 10-07-07 |
| T3224-038 | semi tr | Chereau | 0,361 | 06-1998 | TK SMX II | R 404a | 02-1998 | 22 | - 20 | 3h18 | 2,66 | 17-07-07 |
| T3157-018 | semi tr | Bizien | 0,327 | 05-1998 | TK SMX II | R 404a | 10-1997 | 21 | - 20 | 5h10 | 2,95 | 3-08-07 |
| T3224-051 | semi tr | Chereau | 0,361 | 06-1998 | TK SMX II | R 404a | 06-1998 | 19 | - 20 | 4h35 | 2,67 | 7-08-07 |
| T3157-022 | semi tr | Bizien | 0,327 | 05-1998 | TK SMX II | R 404a | 10-1997 | 17 | - 20 | 2h40 | 2,95 | 17-08-07 |
| T3157-029 | semi tr | Bizien | 0,327 | 05-1998 | TK SMX II | R 404a | 10-1997 | 17 | - 20 | 4h10 | 2,95 | 17-08-07 |
| T3157-021 | semi tr | Bizien | 0,327 | 05-1998 | TK SMX II | R 404a | 10-1997 | 21 | - 20 | 4h18 | 2,95 | 24-08-07 |
| T3157-024 | semi tr | Bizien | 0,327 | 05-1998 | TK SMX II | R 404a | 10-1997 | 21 | - 20 | 3h53 | 2,95 | 24-08-07 |
| T3244-044 | semi tr | Chereau | 0,361 | 06-1998 | TK SMX II | R 404a | 03-1998 | 19 | - 20 | 2h45 | 2,64 | 28-08-07 |
| 3118-149 | semi tr | Norfrig | 0,374 | 09-1998 | Carr Ultra | R 404a | 1998 | 14 | - 20 | 3h32 | 3,56 | 5-09-07 |
| T3157-020 | semi tr | Bizien | 0,327 | 05-1998 | TK SMX II | R 404a | 10-1997 | 18 | - 20 | 3h30 | 2,95 | 7-09-07 |
| T3157-028 | semi tr | Bizien | 0,327 | 05-1998 | TK SMX II | R 404a | 08-1997 | 18 | - 20 | 3h07 | 2,95 | 7-09-07 |
| T3025-016 | truck | Chereau | 0,384 | 07-1998 | TK TD II SR | R 404a | 11-1997 | 15 | - 20 | 1h35 | 3,76 | 16-10-07 |