| UNITED NATIONS |  | #### E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL ECE/TRANS/WP.15/2007/17 9 August 2007 Original: ENGLISH |

ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

Working Party on the Transport of Dangerous Goods

Eighty-third session

Geneva, 5-9 November 2007

Item 5 of the provisional agenda

PROPOSALS FOR AMENDMENTS TO ANNEXES A AND B OF ADR

Chapter 8.3 - Miscellaneous requirements to be complied with by the vehicle crew

##### New section 8.3.8 - Use of cables

Transmitted by the Government of Belgium

##### SUMMARY Executive Summary : This proposal is intended to ensure that O3 and O4 trailers carrying dangerous goods can effectively use the vehicle’s anti-lock bracking system and other safety enhancing features built into the vehicle. Action to be taken : Add a paragraph 8.3.8 Related documents : None.

Introduction

1.	On vehicles where anti-lock bracking systems are mandatory, for the system to be able to function on an O3 or O4 trailer, the trailer has to be connected electrically to the towing vehicle.  From experience we have learnt that the connector and cable making this connection is far too often not used.  This means that the anti-lock bracking system on the trailer (as well as other features enhancing the braking system) cannot function. Some countries (eg. Netherlands, United Kingdom, …) mandate the use of this anti-lock bracking system cable.  Many other countries however do not mandate the use of this cable.  This proposal ensures that the ISO 7638 cable as well as other cables are used at all times when transporting dangerous goods.

2.	The Government of Belgium introduced an informal document (INF.5) mandating the use of the ISO 7638 cable at the 82nd session of the Working Party on the Transport of Dangerous Goods (WP.15).  Many contracting parties supported the proposal.  The representative of France suggested enlarging the scope of the proposal to mandate the use of all cables referred to in paragraph 9.2.2.6.3.  Given the positive reception that its informal document got, the Government of Belgium confirmed that a formal proposal would be submitted at the 83rd session. This document takes into account all recommendations made by the contracting parties during the the 82nd session as well as recommendations made by the Government of the Netherlands following the session.

Proposal

3.	Under Chapter 8.3, add a paragraph 8.3.8, Use of cables,  to read :

“In the case of a transport unit consisting of a motor vehicle and an O3 or O4 trailer with anti-lock bracking system, the connections referred to in paragraph 9.2.2.6.3 shall be connecting the towing vehicle and the trailer at all times.”

_____