| ## UNITED NATIONS |  | # E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL ECE/TRANS/WP.29/2007/3/Corr.1 28 February 2007 ENGLISH ONLY |

###### ECONOMIC COMMISSION FOR EUROPE

- INLAND TRANSPORT COMMITTEE

- World Forum for Harmonization of Vehicle Regulations

One-hundred-and-forty-first session

Geneva, 13-16 March 2007

Item 4.2.2. of the provisional agenda

PROPOSAL FOR SUPPLEMENT 5 TO REGULATION No. 13-H

(Braking)

Corrigendum

Pages 2 and 3, paragraph 5.2.11.2.1., correct to read:

"5.2.11.2.1.	It shall be possible to easily assess this wear on service brake linings from the outside or underside of the vehicle, without the removal of the wheels, by the provision of appropriate inspection holes or by some other means.  This may be achieved by utilizing simple standard workshop tools or common inspection equipment for vehicles.

Alternatively, a sensing device per wheel (twin wheels are considered as a single wheel), which will warn the driver at his driving position when lining replacement is necessary, is acceptable.  The yellow warning signal specified in paragraph 5.2.21.1.2. below may be used."

Page 4, paragraph 12.1., correct the word "may" to read "shall".

- - - - -