| ## UNITED NATIONS |  | # E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL ECE/TRANS/WP.29/2007/6 15 December 2006 ENGLISH Original: ENGLISH and FRENCH |

###### ECONOMIC COMMISSION FOR EUROPE

- INLAND TRANSPORT COMMITTEE

- World Forum for Harmonization of Vehicle Regulations

One-hundred-and-forty-first session

Geneva, 13-16 March 2007

Item 4.2.14. of the provisional agenda

PROPOSAL FOR SUPPLEMENT 5 TO REGULATION No. 106

(Pneumatic tyres for agricultural vehicles)

Submitted by Working Party on Brakes and Running Gear (GRRF)

Note:	The text reproduced below was adopted by GRRF at its sixtieth session.  It is based on ECE/TRANS/WP.29/GRRF/2006/25, not amended.  It is transmitted to World Forum for Harmonization of Vehicle Regulations (WP.29) and to the Administrative Committee (AC.1) for consideration (see ECE/TRANS/WP.29/GRRF/60, para. 33).

Annex 9

Paragraph 3.1., amend to read:

"3.1.	Mount the tyre and wheel assembly on the test axle and press it against the outer face of a smooth power-driven test drum of at least 1700 mm ± 1 per cent in diameter having a surface at least as wide as the tyre tread."

Insert a new paragraph 3.4.1., to read:

"3.4.1.	In case of a test drum diameter larger than 1700 mm ± 1 per cent, the above "percentage of test load" shall be increased as follows:

F1 = K · F2   where:

K = R1R2⋅R2 + rTR1 + rT

R1	is the diameter of test drum, in millimeter

R2	is the diameter of the reference test drum of 1700 mm

rT	is the tyre outer diameter (see paragraph 6.2. of this Regulation), in millimeter

F1	is the percentage of load to be applied for the test drum

F2	is the percentage of load, as per above table, to be applied for reference test drum of 1700 mm

Example:	K = 1 for a test drum diameter of 1700 mm;

In case of a test drum diameter of 3000 mm and a tyre diameter of 1500 mm:

K = 30001700⋅1700+15003000+1500 = 1.12"

- - - - -