| UNITED NATIONS |  | E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL ECE/TRANS/WP.29/2007/100/Add.1 4 October 2007 ENGLISH ONLY |

ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations

One-hundred-and-forty-third session

Geneva, 13 - 16 November 2007

Item 4.2.7. of the provisional agenda

1958 AGREEMENT

Consideration of draft amendments to existing Regulations

Proposal for the 11 series of amendments to Regulation No. 13 (Heavy vehicles braking)

Proposal for amendments to the proposal for the 11 series of amendments to Regulation No. 13

Adendum

Submitted by the Working Party on Brakes and Running Gear /

The text reproduced below was adopted by the Working Party on Brakes and Running Gear (GRRF) at its sixty-second session. It supersedes the transitional provisions of ECE/TRANS/WP.29/2007/106 concerning the installation of the Electronic Stability Control (ESC) on different vehicle categories.  The modifications to ECE/TRANS/WP.29/2007/100 are marked in bold characters.  It is submitted to WP.29 and AC.1. for consideration.

Paragraph 5.2.1.32., amend to read (including the insertion of a reference to a new footnote 1/ and a new footnote 1/):

"5.2.1.32.	Subject to the provisions of paragraph 12.4. to this Regulation, all vehicles of categories M2, M3, N2 and N3 / having no more than 3 axles, [may be mandated to be equipped on the basis of national regulation / shall be equipped] with a vehicle stability function.  This shall include roll-over control and directional control and meet the technical requirements of Annex 21 to this Regulation."

Paragraph 5.2.2.23., amend to read (including the insertion of a reference to a new footnote 2/ and a new footnote 2/):

"5.2.2.23.	Subject to the provisions of paragraph 12.4. to this Regulation, all vehicles of categories O3 and O4 / having no more than 3 axles and equipped with air suspension, [may be mandated to be equipped on the basis of national regulation / shall be equipped] with a vehicle stability function.  This shall include at least roll-over control and meet the technical requirements of Annex 21 to this Regulation."

Paragraph 12.1.7., amend to read (including the footnote */):

"12.1.7.	As from the official date of entry into force of the 11 series of amendments, no Contracting Party applying this Regulation shall refuse to grant type-approval under this Regulation as amended by the 11 series of amendments. */"

Paragraphs 12.4. and 12.4.1., amend to read:

"12.4.	Mandatory provisions for vehicles equipped with a vehicle stability function

12.4.1.	Requirements for the equipment of vehicles with vehicle stability functions as specified in paragraphs 5.2.1.32. and 5.2.2.23. of this Regulation, as amended by the 11 series of amendments, shall be applied as follows:

| Vehicle category | Application date (as from the date after entry into force of the 11 series of amendments) |  |
| --- | --- | --- |
|  | Contracting Parties [may/shall] grant approvals only if the vehicle type to be approved meets the requirements of this Regulation as amended by the 11 series of amendments | Contracting Parties applying this Regulation [may/shall] refuse first national or regional registration of a vehicle which does not meet the requirements of the 11 series of amendments to this Regulation |
| - M2 | 60 months | 84 months |
| M3 (Class III) M3 <16 tonnes (pneumatic transmission) M3 (Class II and B (hydraulic transmission) M3 (Class III) (hydraulic transmission) M3 (Class III) (pneumatic control transmission and hydraulic energy transmission) M3 (Class II) (pneumatic control transmission and hydraulic energy transmission) | 12 months 24 months 60 months 60 months 72 months 72 months | 36 months 48 months 84 months 84 months 96 months 96 months |
| M3 (other than above) | 24 months | 48 months |
| N2 (hydraulic transmission) N2 (pneumatic control transmission and hydraulic energy transmission) | 60 months 72 months | 84 months 96 months |
| N2 (other than above) | 48 months | 72 months |
| - N3 (2 axle tractors for semi-trailers) - N3 (2 axle tractors for semi-trailers with pneumatic control transmission (ABS)) - N3 (3 axles with electric control transmission (EBS)) - N3 (3 axles with pneumatic control transmission (ABS)) | 12 months 36 months 36 months 48 months | 36 months 60 months 60 months 72 months |
| - N3 (other than above) | 24 months | 48 months |
| O3 (combined axle load between 3.5 - 7.5 tonnes) O3 (other than above) | 48 months 36 months | 72 months 60 months |
| - O4 | 24 months | 36 months |

"

- - - - -

/  In accordance with the programme of work of the Inland Transport Committee for 2006-2010 (ECE/TRANS/166/Add.1, programme activity 02.4), the World Forum will develop, harmonize and update Regulations in order to enhance performance of vehicles with respect to brakes and running gear.  The present document is submitted in conformity with that mandate.

/	Off-road vehicles, special purpose vehicles (e.g. mobile plant using non standard vehicle chassis – e.g. cranes - , hydro-static driven vehicles in which the hydraulic drive system is also used for braking and auxiliary functions), Class I and Class A buses of categories M2 and M3, articulated buses and coaches, N2 tractors for semi-trailer with a gross vehicle mass (GVM) between 3.5 and 7.5 tonnes, shall be excluded from this requirement.

/	Trailers for exceptional load transport and trailers with areas for standing passengers shall be excluded from this requirement.

*/	This paragraph shall not prevent Denmark from continuing to mandate a vehicle stability function meeting the requirements of this Regulation.