| UNITED NATIONS |  | E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL ECE/TRANS/WP.29/2007/10 14 December 2006 Original: ENGLISH |

ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations

One-hundred-and-forty-first session

Geneva, 13-16 March 2007

Item 4.2.6. of the provisional agenda

PROPOSAL FOR SUPPLEMENT 10 TO REGULATION No. 43

(Safety glazing)

Submitted by the Working Party on General Safety Provisions (GRSG)

Note:	The text reproduced below was adopted by GRSG at its ninety-first session.  It is based on informal document GRSG-91-18, adopted as reproduced in Annex 2 to the report.  It is submitted to the World Forum for Harmonization of Vehicle Regulations (WP.29) and to the Administrative Committee (AC.1) for consideration (ECE/TRANS/WP.29/GRSG/70, para. 31).

Annex 3,

Paragraph 9.1.2.2., amend to read (including footnote 8/):

"9.1.2.2.	For windscreens of M1 vehicles and N1 8/ vehicles derived from a M1 vehicle, having the same:

(a)	driver's seat reference point

(b)	windscreen dimensional characteristics

(c)	windscreen inclination angle

the test shall be carried out in test area B defined in Annex 18, paragraph 2.3., excluding any opaque obscuration impinging on it.

For windscreens of other categories .…

__________________

8/	As defined in Annex 7 to the Consolidated Resolution on the Construction of Vehicles (R.E.3), (document TRANS/WP.29/78/Rev.1/Amend.2 as last amended by Amend.4)."

Paragraph 9.2.2.1., amend to read:

"9.2.2.1.	For windscreens of M1 vehicles and N1 vehicles restricted as described under paragraph 9.1.2.2., in test area A, extended to the median plane of the vehicle, and in the corresponding part of the windscreen symmetrical to it about the longitudinal median plane of the vehicle, and also in the reduced test area B according to paragraph 2.4. of Annex 18."

Paragraph 9.2.2.2., amend to read:

"9.2.2.2.	For windscreens of other categories of vehicles, in zone I as defined in paragraph 9.2.5.2. of this annex."

Paragraph 9.2.5.1., amend to read:

"9.2.5.1.	Zones A and B of windscreens for vehicle category M1 and N1 restricted as described under paragraph 9.1.2.2. are defined in Annex 18 to this Regulation."

Paragraph 9.2.5.2., amend to read:

"9.2.5.2.	Zones of windscreens for other categories of vehicles are defined on the basis of:"

Paragraph 9.2.6., the table, the text in the second row of the first column, amend to read:

"M1 and N1 restricted as described under paragraph 9.1.2.2."

Paragraph 9.2.6., the table, the text in the third row of the first column, amend to read:

"Other categories of vehicles"

Paragraph 9.3.5., the table, the text in the second row of the first column, amend to read:

"M1 and N1 restricted as described under paragraph 9.1.2.2."

Paragraph 9.3.5., the table, the third row of the first column, amend to read:

"Other categories of vehicles"

- - - - -