| ## UNITED NATIONS |  | # E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL ECE/TRANS/WP.29/2007/53 13 April 2007 Original: ENGLISH |

ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations

One-hundred-and-forty-second session

Geneva, 26-29 June 2007

Item 4.2.18. of the provisional agenda

1958 AGREEMENT

Consideration of draft amendments to existing Regulations

Proposal for draft Supplement 11 to Regulation No. 87 (Daytime running lamps)

Submitted by the Working Party on Lighting and Light-Signalling

The text reproduced below was adopted by the Working Party on Lighting and                           Light-Signalling (GRE) at its fifty-seventh session.  It is based on ECE/TRANS/WP.29/GRE/2007/11, as amended by GRE-57-14.  It is submitted to WP.29 and AC.1 for consideration (ECE/TRANS/WP.29/GRE/57).

Paragraph 7.4., amend to read:

"7.4.	In the case of a lamp containing more than one light source the lamp shall comply with the minimum intensity required when any one light source has failed and when all light sources are illuminated the maximum intensity shall not be exceeded.

A group of light sources, wired so that the failure of any one of them causes all of them to stop emitting light, shall be considered to be one light source."

- - - - -