| ## UNITED NATIONS |  | # E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL ECE/TRANS/WP.29/2007/61 12 July 2007 ENGLISH Original: ENGLISH AND FRENCH |

ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations

One-hundred-and-forty-third session

Geneva, 13-16 November 2007

Item 4.2.9. of the provisional agenda

1958 AGREEMENT

Consideration of draft amendments to existing Regulations

#### Proposal for Supplement 13 to the 02 series of amendments to Regulation No. 19

(Front fog lamps)

Submitted by the Working Party on Lighting and Light-signalling

The text reproduced below was adopted by the Working Party on Lighting and Light-signalling (GRE) at its fifty-seventh session.  It is based on ECE/TRANS/WP.29/GRE/2007/16, not amended.  It is submitted to WP.29 and AC.1 for consideration (ECE/TRANS/WP.29/GRE/57, para. 45).

Insert a new paragraph 1.5., to read:

"1.5.	References made in this Regulation to standard (étalon) filament lamp(s) and to Regulation No. 37 shall refer to Regulation No. 37 and its series of amendments in force at the time of application for type approval."

Paragraph 2.2.1., amend to read:

"2.2.1.	a brief technical specification including the category of filament lamp used as listed in Regulation No. 37 and its series of amendments in force at the time of application for type approval, even if the filament lamp cannot be replaced;"

Paragraph 5.3., amend to read:

"5.3.	The front fog lamp shall be equipped with one filament lamp approved according to Regulation No. 37, even if the filament lamp cannot be replaced.  Any Regulation No. 37 filament lamp may be used, provided that no restriction on the use is made in Regulation No. 37 and its series of amendments in force at the time of application for type approval."

Insert new paragraphs 5.4. to 5.4.2., to read:

"5.4.	In the case of a replaceable filament lamp(s):

5.4.1.	The design of the device shall be such that the filament lamp can be fixed in no other position but the correct one.

5.4.2.	The filament lamp holder shall conform to the characteristics given in IEC Publication 60061.  The holder data sheet relevant to the category of filament lamp used, applies."

Paragraphs 5.4. and 5.5. (former), renumber as paragraphs 5.5. and 5.6.

- - - - -