| ## UNITED NATIONS |  | # E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL ECE/TRANS/WP.29/2007/67 16 July 2007 Original: ENGLISH |

ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations

One-hundred-and-forty-third session

Geneva, 13-16 November 2007

Item 4.2.19. of the provisional agenda

1958 AGREEMENT

Consideration of draft amendments to existing Regulations

#### Proposal for the 04 series of amendments to Regulation No. 48

(Installation of lights and light-signalling devices)

Submitted by the Working Party on Lighting and Light-signalling

The text reproduced below was adopted by the Working Party on Lighting and Light-signalling (GRE) at its fifty-seventh session.  It is based on ECE/TRANS/WP.29/GRE/2007/35,  as  amended by Annex III of the report.  It is submitted to WP.29 and AC.1 for consideration (ECE/TRANS/WP.29/GRE/57, para. 33).

Paragraph 6.19.7., amend to read:

"6.19.7.	Electrical connections

The daytime running lamps shall be switched ON automatically when the device which starts and/or stops the engine is in a position which makes it possible for the engine to operate.

The daytime running lamps shall …. at short intervals.

Furthermore, the lamps referred to in paragraph 5.11. are not switched on when the daytime running lamps are switched ON.

6.19.7.1.	It shall be possible to activate and deactivate the automatic switching ON of daytime running lamps.  The means to fulfil this requirement shall not be accessible during the normal condition of use of a vehicle.  Tools may be required to access or operate these means."

Annex 1, insert a new item 10.6., to read:

"10.6.	Description of the device used to satisfy paragraph 6.19.7.1. and method to verify its status."

- - - - -