| ## UNITED NATIONS |  | # E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL ECE/TRANS/WP.29/2007/69 12 July 2007 ENGLISH Original: ENGLISH AND FRENCH |

ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations

One-hundred-and-forty-third session

Geneva, 13-16 November 2007

Item 4.2.20. of the provisional agenda

1958 AGREEMENT

Consideration of draft amendments to existing Regulations

#### Proposal for Supplement 11 to Regulation No. 50

(Front/rear position/stop-lamps, direction indicator, rear registration plate lamps (mopeds/motorcycles))

Submitted by the Working Party on Lighting and Light-signalling

The text reproduced below was adopted by the Working Party on Lighting and Light-signalling (GRE) at its fifty-seventh session.  It is based on ECE/TRANS/WP.29/GRE/2007/16, not amended.  It is submitted to WP.29 and AC.1 for consideration (ECE/TRANS/WP.29/GRE/57, para. 45).

Insert a new paragraph 2.3., to read:

"2.3.	References made in this Regulation to standard (étalon) filament lamp(s) and to Regulation No. 37 shall refer to Regulation No. 37 and its series of amendments in force at the time of application for type approval."

Paragraph 3.2.2., amend to read:

"3.2.2.	a brief technical description stating, in particular, with the exception of lamps with non-replaceable light sources:

(a)	the category or categories of filament lamp(s) prescribed; this filament lamp category shall be one of those contained in Regulation No. 37 and its series of amendments in force at the time of application for type approval; and/or

(b)	the light source module specific identification code."

Insert new paragraphs 6.4. to 6.4.3., to read:

"6.4.		In the case of replaceable filament lamp(s):

6.4.1.		Any category or categories of filament lamp(s) approved according to Regulation No. 37 may be used, provided that no restriction on the use is made in Regulation No. 37 and its series of amendments in force at the time of application for type approval.

6.4.2.		The design of the device shall be such that the filament lamp can be fixed in no other position but the correct one.

6.4.3.		The filament lamp holder shall conform to the characteristics given in IEC Publication 60061.  The holder data sheet relevant to the category of filament lamp used, applies."

- - - - -