| UNITED NATIONS |  | E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL ECE/TRANS/WP.29/2007/81 19 July 2007 Original: ENGLISH |

ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations

One-hundred-and-forty-third session

Geneva, 13 - 16 November 2007

Item 4.2.14. of the provisional agenda

1958 AGREEMENT

Consideration of draft amendments to existing Regulations

Proposal for Supplement 11 to Regulation No. 43

(Safety glazing)

Submitted by the Working Party on General Safety (GRSG)

The text reproduced below was adopted by GRSG at its one ninety-second session.  It is based on ECE/TRANS/WP.29/GRSG/2007/9, as amended in para. 21 of the report.  It is submitted to WP.29 and AC.1 for consideration (ECE/TRANS/WP.29/GRSG/71, para. 21).

Annex 3,

Paragraph 9.1.2.2., amend to read (footnote 8/ not amended):

"9.1.2.2.	For the windscreens of M1 vehicles 8/

the test shall be carried out in test area B defined in Annex 18, paragraph 2.3., excluding any opaque obscuration impinging on it.

For the windscreens of N1 vehicles, the manufacturer may request that the same test may be carried out either in test area B defined in Annex 18, paragraph 2.3., excluding any opaque obscuration impinging on it, or in the zone I defined in paragraph 9.2.5.2.3. of this annex.

For windscreens of other categories .… "

Paragraph 9.2.2.1., amend to read:

"9.2.2.1.	For vehicle category M1 in test area A, extended to the median plane of the vehicle, and in the corresponding part of the windscreen symmetrical to it about the longitudinal median plane of the vehicle, and also in the reduced test area B according to paragraph 2.4. of Annex 18."

Paragraph 9.2.2.2., amend to read:

"9.2.2.2.	For vehicles of categories M and N other than M1:

(a)	in zone I as defined in paragraph 9.2.5.2. of this annex for M2, M3, N2 and N3 vehicles;

(b)	either in zone I as defined in paragraph 9.2.5.2. of this annex or in test area A, extended to the median plane of the vehicle, and in the corresponding part of the windscreen symmetrical to it about the longitudinal median plane of the vehicle, and also in the reduced test area B according to paragraph 2.4. of Annex 18 for N1 vehicles."

Paragraph 9.2.5.1., amend to read:

"9.2.5.1.	Zones A and B of windscreens for vehicle category M1 and N1 are defined in Annex 18 to this Regulation."

Paragraph 9.2.5.2., amend to read:

"9.2.5.2.	Zones of windscreens for vehicles of categories M and N other than M1 are defined on the basis of:"

Paragraph 9.2.6., the table, the text in the second row of the first column, amend to read:

"M1 and N1."

Paragraph 9.2.6., the table, the text in the third row of the first column, amend to read:

"M and N categories other than M1"

Paragraph 9.3.5., the table, the text in the second row of the first column, amend to read:

"M1 and N1."

Paragraph 9.3.5., the table, the third row of the first column, amend to read:

"M and N categories other than M1"

Annex 18, the title, amend to read:

"PROCEDURE FOR DETERMINING TEST AREAS ON WINDSCREENS OF VEHICLES IN RELATION TO THE "V" POINTS"

- - - - -