| UNITED NATIONS |  | E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL ECE/TRANS/WP.29/2007/90 10 July 2007 Original: ENGLISH |

ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations

One-hundred-and-forty-third session

Geneva, 13 - 16 November 2007

Item 4.2.24. of the provisional agenda

1958 AGREEMENT

Consideration of draft amendments to existing Regulations

Proposal for Corrigendum 3 to the Revision 3 to Regulation No. 83

(Emission of M1 and N1 categories of vehicles)

Submitted by the Working Party on Pollution and Energy (GRPE)

The text reproduced below was adopted by GRPE at its fifty-fourth session.  It is based on ECE/TRANS/WP.29/GRPE/2007/10, as amended by para. 21 of the report.  It is submitted to WP.29 and AC.1 for consideration (ECE/TRANS/WP.29/GRPE/54, para. 21).

Annex 4 - Appendix 3

Paragraph 5.1.1.2.5., correct to read:

"5.1.1.2.5.	Take the average T of the two times t1 and t2"

Paragraph 5.1.1.2.7., correct to read:

"5.1.1.2.7.	Calculate the power by the formula

P=M.V.ΔV500 T

where:

P is expressed in kW

V =	speed of the test in m/s

ΔV =	speed deviation from speed V, in m/s as specified in paragraph 5.1.1.2.3. of this appendix

M =	reference mass in kg

T =	time in seconds (s) "

Annex 7, paragraph 4.2.1., correct "± 5 KPa" to read "± 0.5 kPa".

- - - - -