ALTERNATIVE PROPOSAL FOR SUPPLEMENT 1 TO THE 02 SERIES OF AMENDMENTS TO REGULATION No. 46

(Devices for indirect vision)

Transmitted by the representative of the European Communities

Note:	The text reproduced below is based on Document ECE/TRANS/WP.29/2006/101, which was adopted by GRSG at its ninetieth session.  It is proposed to delete certain paragraphs of text from the above-mentioned document.  These paragraphs are indicated below in strikethrough.

A. PROPOSAL

The list of contents, the Annexes, amend to read:

" ….

Annex 10	-	Calculation of the detection distance ………………………………………

Annex 11	-	Determination of the ocular points for a seat with a fixed seat back angle"

The text of the Regulation,

Paragraphs 1. to 1.2.2. and the corresponding footnote 1/, amend to read:

"1.	SCOPE

This Regulation applies:

-	to devices for indirect vision for vehicles of category M, N, and of category L 1/ 		with bodywork at least partly enclosing the driver,

-	to the installation of devices for indirect vision on vehicles of category M and N 1/.

__________________

1/	As defined in Annex 7 to the Consolidated Resolution on the Construction of Vehicles (R.E.3), (document TRANS/WP.29/78/Rev.1/Amend.2 as last amended by Amend.4)."

Paragraph 12.1., amend to read:

" …. as defined in Annex 8.  In the case of a seat with fixed seat-back angle, the location 	of the ocular points shall be adjusted in accordance with the provisions of Annex 11 to 	this Regulation.  The straight line …. "

Paragraph 15.2.1.1.1., the table, the fifth column "Wide-angle mirror Class IV", the cell for motor vehicles of category N2 ≤  7.5 t, the text in the cell, amend to read:

"Compulsory For both sides if a Class V mirror can be fitted

Optional For both sides together if not"

Paragraph 15.2.1.1.1., the table, the sixth column "Close-proximity mirror Class V", the cell for motor vehicles of category N2 ≤  7.5 t, the text in the cell, amend to read:

"Compulsory (see paragraphs 15.2.2.7. and 15.2.4.5.5.)

One on the passenger's side

Optional One on the driver's side (both must be fitted at least 2 m above the ground).  A tolerance of + 10 cm may be applied"

Insert a new Annex 11, to read:

"Annex 11

DETERMINATION OF THE OCULAR POINTS FOR A SEAT

WITH A FIXED SEAT-BACK ANGLE

1.	The position of the ocular points in relation to the R point shall be adjusted as indicated 	in the table below by X coordinates from the three-dimensional reference system. The 	table indicates the basic coordinates for a fixed seatback angle of 25 degrees.  The three-	dimensional reference system for the coordinates is as defined in Annex 8, Appendix 2 to 	this Regulation.

| Seat back angle | Horizontal coordinates |
| --- | --- |
| (in degrees) | ΔX |
| 25 | 68 mm |

2.	Further correction for fixed seat-back angles other than 25 degrees.

The table below indicates the further corrections, from the ocular position with a fixed 	seat-back angle of 25 degrees, to be made to the X and Z coordinates of the ocular points 	when the design seat-back angle is not 25 degrees.

| Seat-back angle | Horizontal coordinates | Vertical coordinates |
| --- | --- | --- |
| (in degrees) | ΔX | ΔZ |
| 5 | -186 mm | 28 mm |
| 6 | -177 mm | 27 mm |
| 7 | -167 mm | 27 mm |
| 8 | -157 mm | 27 mm |
| 9 | -147 mm | 26 mm |
| 10 | -137 mm | 25 mm |
| 11 | -128 mm | 24 mm |
| 12 | -118 mm | 23 mm |
| 13 | -109 mm | 22 mm |
| 14 | -99 mm | 21 mm |
| 15 | -90 mm | 20 mm |
| 16 | -81 mm | 18 mm |
| 17 | -72 mm | 17 mm |
| 18 | -62 mm | 15 mm |
| 19 | -53 mm | 13 mm |
| 20 | -44 mm | 11 mm |
| 21 | -35 mm | 9 mm |
| 22 | -26 mm | 7 mm |
| 23 | -18 mm | 5 mm |
| 24 | -9 mm | 3 mm |
| 25 | 0 mm | 0 mm |
| 26 | 9 mm | -3 mm |
| 27 | 17 mm | -5 mm |
| 28 | 26 mm | -8 mm |
| 29 | 34 mm | -11 mm |
| 30 | 43 mm | -14 mm |
| 31 | 51 mm | -18 mm |
| 32 | 59 mm | -21 mm |
| 33 | 67 mm | -24 mm |
| 34 | 76 mm | -28 mm |
| 35 | 84 mm | -32 mm |
| 36 | 92 mm | -35 mm |
| 37 | 100 mm | -39 mm |
| 38 | 108 mm | -43 mm |
| 39 | 115 mm | -48 mm |
| 40 | 123 mm | -52 mm |

"

B.  JUSTIFICATION

Document ECE/TRANS/WP.29/2006/101 proposes amendments to Regulation 46 in respect of three aspects:

1)	Clarification of the scope of the Regulation,

2)	The mandatory installation of additional 'blind spot' mirrors, and

3)	Adjustments to the position of the ocular points in respect of vehicles with a fixed seat back angle.

At the 91st GRSG meeting, the Netherlands raised concerns regarding 3) above.  The European Commission agrees that the issue of adjustments to the ocular point position as a result of changes to the seat back angle requires further discussion at GRSG.  However, in order to expedite the road safety benefits achievable through the installation of additional blind spot mirrors on certain classes of vehicle, the European Commission proposes that the modified installation requirements in paragraph 15. of the Regulation are adopted without further delay, together with the proposed clarification of the scope.

Therefore this document modifies ECE/TRANS/WP.29/2006/101 by removing the amendments which relate only to the adjustment of the ocular point position.

- - - - -