| UNITED NATIONS |  | E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL ECE/TRANS/WP.29/GRE/2007/19 28 December 2006 Original: ENGLISH ENGLISH AND FRENCH ONLY |

### ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations

Working Party on Lighting and Light-Signalling

Fifty-seventh session

Geneva, 26-30 March 2007

Item 10. of the provisional agenda

REGULATIONS Nos. 98, 112 AND 113

Proposal for draft amendments to Regulation No. 112

(Headlamps emitting an asymmetrical passing beam)

Submitted by the expert from the Working Party "Brussels 1952"

The text reproduced below was prepared by the expert from the Working Party "Brussels 1952" (GTB) in order to clarify the testing requirements for devices, incorporated in headlamps, as well as to remove the provision concerning user's interference in those devices.  The proposal supersedes ECE/TRANS/WP.29/GRE/2006/13.  The modifications to current text of the Regulation (up to Supplement 7 to the original version) are marked in bold characters.

A.	PROPOSAL

Paragraphs 5.9.1. and 5.9.2., amend to read:

"5.9.1.	the device is robust enough to withstand 50,000 operations under normal conditions of use.  In order to verify compliance with this requirement, the Technical Service responsible for approval tests may:

(a)	require the applicant to supply the equipment necessary to perform the test;

(b)	forego the test if the headlamp presented by the applicant is accompanied by a test report, issued by a Technical Service responsible for approval tests for headlamps of the same construction (assembly), confirming compliance with this requirement.

5.9.2.	in the case of failure, the illumination above the line H-H shall not exceed the values of a passing beam according to paragraph 6.2.5.; in addition, on headlamps designed to provide a passing and/or a driving beam to become a bend lighting, a minimum illumination of at least 3 lux shall be fulfilled in test point 25 V (VV  line, D 75 cm).

When performing the tests to verify compliance with these requirements, the Technical Service responsible for approval tests shall refer to the instructions supplied by the applicant."

Paragraph 5.9.4., should be deleted.

# B.	JUSTIFICATION

Paragraph 5.9. addresses the requirements for mechanical, electro-mechanical or other devices incorporated in the headlamp to produce alternately a driving/passing beam or bend lighting.  Based on the recommendations from the Technical Services represented in its Photometry Working Group, GTB suggests the following amendments:

(a)	to indicate the means in order to verify compliance with the provisions of paragraphs 5.9.1. and 5.9.2.

(b)	when paragraph 5.9.2. was introduced by Supplement 2 to the original version, the value of 5 lux was copied from the relevant provisions of Regulation No. 98 without taking into account the difference in measuring procedures.  The proposal intends to specify the correct value.

(c)	to delete paragraph 5.9.4., requiring that "the device must be so constructed that the user cannot, with ordinary tools, change the shape or position of the moving parts".  It was concluded that it is not possible to apply this provision in practice as there is no definite test procedure and the headlamp would have to be destroyed in order to carry out such a test.  As there have been no adverse consequences from not performing this test, the provision could be deleted without negative effects upon safety.

- - - - -