| UNITED NATIONS |  | E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL ECE/TRANS/WP.29/GRE/2007/41 19 July 2007 ENGLISH Original: FRENCH FRENCH AND ENGLISH ONLY |

ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations

Working Party on Lighting and Light-Signalling

Fifty-eighth session,

Geneva, 1-5 October 2007

Item 13 (a) of the provisional agenda

# NEW AGENDA ITEMS

## Regulation No. 65

## (Special warning lamps for motor vehicles)

## Proposed draft supplement 5 to Regulation No. 65

## Communication from the expert from Spain

The text reproduced below, submitted by the expert from Spain, contains a proposal for a supplement to Regulation No. 65. This proposal, based on the amendments to the Vienna Convention contained in document ECE/TRANS/89 of 2 September 1991, is intended to allow for the use of the colour red in special vehicles in countries that wish to include such a provision in their national legislation. Amendments made to the current text of the Regulation (including supplement 4) appear in bold.

A.	PROPOSAL

Paragraph 1.1, amend as follows:

“1.1	‘Special warning lamp’ means a lamp emitting blue, red or amber light intermittently for use on vehicles.”

Paragraph 2.1, amend as follows:

“2.1	...

It shall specify whether the special warning lamp is intended to emit amber (A), red (R) or blue (B) light, whether it falls within the directional flashing lamp (X) category, or whether it falls within the rotating or stationary flashing lamp (T) category, and whether it has one level of intensity (class 1), or two levels of intensity (class 2).

...”

Paragraph 4.4.1.3, amend as follows:

“4.4.1.3	‘T’ or ‘X’ according to the category of the unit, followed by ‘A’ or ‘B’ or ‘R’ according to the colour of the unit (see paragraph 2.1 above).”

Annex 1

Item 1, amend as follows:

“...

1.	Special warning lamp/rotating/stationary flashing lamp/directional flashing lamp/ complete bar/blue/amber/red 2/

...”

Annex 3

Add a new item, worded as follows:

“...

3	limit towards purple 		y≥0.980-x

limit towards yellow		y≤0.335”

Annex 5

Paragraph 7.2, amend the table as follows:

“...

| Category T |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Colour |  |  |
|  |  |  | blue | amber | red |
| Minimum value of the effective luminous intensity Je, within the specified vertical angles and a horizontal angle of 360° around the reference axis | 0° | by day | 120 | 230 | 120 |
|  |  | by night | 50 | 100 | 50 |
|  | ± 4° | by day | 60 | - | 60 |
|  |  | by night | 25 | - | 25 |
|  | ± 8° | by day | - | 170 | - |
|  |  | by night | - | 70 | - |
| Maximum value of the effective luminous intensity Je | Inside ± 2° | by day | 1 700 |  |  |
|  |  | by night | 700 |  |  |
|  | Inside ± 8° | by day | 1 500 |  |  |
|  |  | by night | 600 |  |  |
|  | Outside the above areas | by day | 1 000 |  |  |
|  |  | by night | 300 |  |  |

...”

Paragraph 7.3, amend the table as follows:

| Category X |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Colour |  |  |
|  |  |  | blue | amber | red |
| Minimum value of the effective luminous intensity Je on the reference axis | H = 0° V = 0° | by day | 200 | 400 | 200 |
|  |  | by night | 100 | 200 | 100 |
| Maximum value of the effective luminous intensity Je | inside H = ± 10° V = ± 4° | by day | 3 000 | 1 500 | 3 000 |
|  |  | by night | 1 500 | 600 | 1 500 |
|  | inside H = ± 20° V = ± 8° | by day | 1 500 | 1 500 | 1 500 |
|  |  | by night | 600 | 600 | 600 |
|  | outside the above areas | by day | 1 000 | 1 000 | 1 000 |
|  |  | by night | 300 | 300 | 300 |

B.	JUSTIFICATION

The amendments to the Vienna Convention contained in document ECE/TRANS/89 of 2 September 1991 allow for red light to be seen in the forward part of the vehicle in the case of stationary or rotating special fog lamps (paragraph 61 of Annex V).

Consequently, these amendments are being proposed in order to adapt Regulation No. 65 so that countries wishing to include in their national legislation the use of the colour red in special vehicles may do so.

-----