| UNITED NATIONS |  | E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL ECE/TRANS/WP.29/GRE/2007/59 18 July 2007 Original: ENGLISH ENGLISH AND FRENCH ONLY |

### ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations

Working Party on Lighting and Light-Signalling

Fifty-eighth session

Geneva, 1-5 October 2007

Item 5(d) of the provisional agenda

COLLECTIVE AMENDMENTS

Regulations Nos. 69, 70 and 104

Submitted by the expert from the European Association of Automotive Suppliers (CLEPA)

The text reproduced below was prepared by the experts from CLEPA at the request of the GRE during its fifty-seventh session in March 2007 to align the physical testing requirements and introduce additional water/solution flow requirements into Regulation No. 104, as well as a comprehensive set of tolerances across all three Regulations: Regulation No. 104, Regulation No. 69 and Regulation No. 70.  The modifications to the current text of Regulation No. 104 are marked in bold characters.

A.	PROPOSAL

## Annex 8

Paragraph 5.2.1., amend to read:

"5.2.1 	When subjected to a continuous spraying action for 60 seconds on the test component in its normal mounting conditions, a test sample shall show no damage to the retro-reflective surface or delamination from the substrate or separation from the sample mounting surface under the following set-up parameters:

(a)	Water/wash solution pressure 8 ± 0.2 MPa;

(b)	Water/wash solution temperature 60° - 5 °C;

(c)	Water/wash solution flow rate 7 ± 1 l/min;

(d)	The tip of the cleaning wand to be positioned at distance of 600 ± 20 mm away from the retro-reflective surface;

(e)	Cleaning wand to be held at no greater angle than 45 degrees from perpendicular to the retro-reflective surface;

(f)	40 degree nozzle creating wide fan pattern."

B.	JUSTIFICATION

This proposal introduces a new value for the "water/wash solution flow rate" and new tolerance parameters to ensure that uniform test results are obtained during performance testing of retro-reflective materials by different Contracting Parties.

This proposal augments the draft proposals introduced by India during the fifty-seventh session of GRE to amend Regulation No. 69 (ECE/TRANS/WP.29/GRE/2006/39) and Regulation No. 70 (ECE/TRANS/WP.29/GRE/2006/40), and may be considered by GRE in conjunction with the above documents as well as CLEPA proposals to amend them.

- - - - -