| UNITED NATIONS |  | E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL ECE/TRANS/WP.29/GRE/2007/61 18 July 2007 Original: ENGLISH ENGLISH AND FRENCH ONLY |

### ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations

Working Party on Lighting and Light-Signalling

Fifty-eighth session

Geneva, 1-5 October 2007

Item 6 of the provisional agenda

REGULATION No. 19

(Front fog lamps )

Proposal for draft corrigendum to the draft 03 series of amendments to Regulation No. 19

Submitted by the expert from the Working Party "Brussels 1952" (GTB)

The text reproduced below was prepared by the expert from the Working Party "Brussels 1952" (GTB), in order to introduce some corrections to the draft 03 series of amendments to Regulation No. 19.  It is based on the draft 03 series of amendments to Regulation No. 19, as approved and amended at the fifty-seventh GRE session (ECE/TRANS/WP.29/GRE/2006/19 and Corr.1, TRANS/WP.20/GRE/57, para. 49 and Annex VI).  The modifications to this text are marked in bold characters.

## A.	PROPOSAL

Insert a new paragraph 6.4.1.4., to read:

"6.4.1.4.	In the case of LED modules:

All measurements on front fog lamps equipped with LED module(s) shall be made at 6.3 V, 13.2 V or 28.0 V respectively, if not otherwise specified within this Regulation.  LED modules operated by an electronic light source control gear shall be measured as specified by the applicant.

The measured luminous intensity values shall be multiplied by a factor 0.7 prior to the check for compliance."

Paragraph 6.4.1.4. (former), renumber as paragraph 6.4.1.5.

Annex 12

Paragraph 3.1.3.1., amend to read:

"3.1.3.1.	all samples shall be tested under the conditions as specified in paragraph 6.4.1.4. of this Regulation."

- - - - -