| UNITED NATIONS |  | E |
| --- | --- | --- |
|  | Economic and Social Council | Distr. GENERAL ECE/TRANS/WP.29/GRE/2007/65 4 September 2007 ENGLISH ONLY |

ECONOMIC COMMISSION FOR EUROPE

INLAND TRANSPORT COMMITTEE

World Forum for Harmonization of Vehicle Regulations

Working Party on Lighting and Light-Signalling

Fifty-eighth session

Geneva, 1-5 October 2007

Item 5(g) of the provisional agenda

COLLECTIVE AMENDMENTS

Regulations Nos. 6, 7 and 48

Proposal for draft corrigendum 1 to Supplement 15 to

the 01 series of amendments Regulation No. 6

(Direction indicators)

Transmitted by the expert from France

The text reproduced below was prepared by the expert from France proposing to align on the state-of-the art realisations the visibility angles requirements of the direction indicators.  The modifications to the current text of the Regulation are marked in bold characters.

A. 	PROPOSAL

Annex 1, amend to read:

"Annex 1

CATEGORIES OF DIRECTION INDICATORS:

MINIMUM ANGLES REQUIRED FOR LIGHT DISTRIBUTION IN SPACE OF THESE CATEGORIES OF DIRECTION INDICATORS.  5/

1.	Minimum vertical visibility angles

(a) lamps of any category except category 6, with a permissible mounting height x mm above the ground: 30° above the horizontal and α° below the horizontal so that:

x > 850 mm: 	α = 15°

if	850 > x > 750 mm:	it may be reduced to α = 5°+ 0.1(x – 750)°

if	x < 750 mm:	it may be reduced to α = 5°

where x is the lower height of the lamp above the ground.

(b) lamps of category 6: 30 ° above and 5 ° below the horizontal.

2.	Minimum horizontal visibility angles

(a) Direction indicators for the front of the vehicle

Category 1: 	for use at a distance not less than 40 mm from the headlamp;

Category 1a:	for use at a distance greater than 20 mm but less than 40 mm from the headlamp;

Category 1b: 	for use at a distance less than 20 mm from the headlamp.

20°

Driving

direction

Vehicle

80°

Front direction indicator

of categories 1, 1a or 1b

20°

Reference   axis

Driving

direction

Vehicle

80°

Front direction indicator

of categories 1, 1a or 1b

(b) Direction indicators for the rear of the vehicle

Category 2a: 		rear direction indicator lamps with steady luminous intensity.

Category 2b: 		rear direction indicator lamps with variable luminous intensity.

20°

Driving

direction

Vehicle

80°

Rear direction indicator

of categories 2a or 2b

20°

Driving

direction

Vehicle

80°

Rear direction indicator

of categories 2a or 2b

20°

Reference          axis

Driving

direction

Vehicle

80°

Rear direction indicator

of categories 2a or 2b

(c) Front-side direction indicators on the side of the vehicle

"

B.	JUSTIFICATION

The current visibility requirements of the front and rear direction indicators and position lamps were written in the early 1970s.  They were easily fulfilled in those times because the shapes of the vehicles were simple and rather boxy.  Recent styling trends based on better aerodynamics and pedestrian protection lead to contoured shapes making the fulfilment of the 15 down and 45 inboard (15D-45inboard) visibility requirement impossible.

No modification is proposed for the visibility angles of the direction indicators for the side of the vehicle.

1.	Front or rear direction indicators, vertical visibility.

The current limit of 750 mm in eight is somewhat arbitrary.  The proposed formulae to Annex 1 (a) create a smooth transition between these two angles and remove the threshold effect.

2.	Front direction indicators, inboard horizontal visibility.

The interior visibility angle reduction of the front direction indicator from 45° to 20° has a minimum impact on the distance of visibility of the vehicle when observed along its longitudinal median plan.  In the case of a vehicle with 1400 mm between the reference axis of the direction indicator lamps, this angular reduction, as seen by an observer standing on the median longitudinal plan of the vehicle, leads to a minimal change of just 1.22 m:

With the current visibility angle, the lamp is visible at D1 = 1400 x 0.5 cotg 45°.

With the proposed visibility angle, the lamp is visible at D2 = 1400 x 0.5 cotg 20°.

The "danger" distance is increased by a modest D2 – D1 = 1400 x 0.5 (cotg 20° - cotg 45°) = 1220 mm.

20°

80°

1400

D

1

D

2

Current R6

Proposal

1400

45°

Observer

Driving direction

20°

80°

1400

D

1

D

2

Current R6

Proposal

1400

45°

Observer

This angular modification allows more pedestrian-friendly front ends.  It improves the aerodynamics, thus reducing the fuel consumption of the vehicle.

3.	Rear direction indicators, inboard horizontal visibility.

This proposal is solely based on styling considerations.  Pedestrian protection and aerodynamics do not apply to parts that are installed at the rear end of the vehicle.

The inboard visibility angles reduction does not jeopardize safety, as shown by the following three sketches:

- - - - -

5/	The angles shown in these arrangements are correct for devices to be mounted on the right side of the vehicle.  The arrows……